# Attachment A

Clauses from the Australasian Railway Association’s revised application for temporary exemption from the Transport Standards that the Human Rights and Equal Opportunity Commission recommends should be granted exemptions and the Accessible Public Transport Jurisdictional Committee’s decisions regarding these clauses

| # Clause | # HREOC recommendation | # APTJC’s comment |
| --- | --- | --- |
| ### 1.9 Access path | - Approve exemption permitting access only from a single nominated boundary point for existing stations – subject to the limitation that an exemption on this point does not exclude any requirement for fuller upgrading of access paths or entrances which arises under legislation giving effect to applicable provisions of the Building Code of Australia as in force during the period of the exemption. - Require report during exemption period from each operator on stations with a mix of accessible and inaccessible entrances, and constraints on upgrading entrances for access - Defer decision on nominated boarding point: further discussion required of need for and limits of exemption. | # Agree with HREOC - There may be occasions when two boundary points need to be identified depending on the design of the platform. - Suitable public information and signage will need to be provided. What will be put in place to make passengers who use the related access way less vulnerable from a personal safety point of view? |
| ### 1.11AX Assistance animals: ARA proposal | # Grant exemption on modified terms | # APTJC seeks HREOC to clarify how it intends to implement and resource this recommendation and outline the role expected of jurisdictions in this matter. |
| ### 1X2 Performance criteria | # Approve exemption regarding flange gaps, but as exemption for access paths rather than as feature of mobility aids, and subject to reporting on accompanying positive measures. - Defer decision on possible limits of exemption on staff assistance with stowage and operation: further discussion required - Refuse exemption on other points. | # Agree with HREOC. Please clarify who will do the reporting. |
| # Clause | # HREOC recommendation | # APTJC’s comment |
| #### 1X3. Orientation | - Approve exemption in relation to orientation and location of allocated spaces and priority seating on conveyances | # Approve exemption. What is to stop the operator locating priority seating in totally inappropriate places on a conveyance? |
| # 2.1 Access paths | - Approve exemption permitting access only from a single nominated boundary point for existing stations – subject to the limitation that an exemption on this point does not exclude any requirement for fuller upgrading of access paths or entrances which arises under legislation giving effect to applicable provisions of the Building Code of Australia as in force during the period of the exemption. - Require report during exemption period from each operator on stations with a mix of accessible and inaccessible entrances, and constraints on upgrading entrances for access - Defer decision on nominated boarding point: further discussion required of need for and limits of exemption. | # Agrees with intent of an exemption on this point so long as it avoids the approach of rewording the Standards. |
| - 2.4 Access paths – Minimum unobstructed width | # Grant exemption consistent with ARA revised proposal | # Grant exemption |
| - 2.4X Access paths – Minimum unobstructed width for railway platforms | - Grant exemption consistent with ARA revised proposal | # Grant exemption |
| ### 2.5AX Level crossings | - Grant exemption, subject to reporting on measures to reduce use of level crossings as part of access paths and research into other possible solutions | - Grant exemption. Who will do the research and by when? |
| ### 2.6 Access paths – Access paths – conveyances | - Grant exemption for reduction in width of access path for existing carriage doors to a minimum width of 760mm, on condition that direct assistance is available - Defer decision on other restrictions and new carriages: further discussion required to ensure sufficient specification of unavoidable constraints. - Stair access to areas of double deck carriages: grant exemption on condition that access to unique facilities is assured. | # How will direct assistance be provided? For example to get a wheelchair through a door, where the door is narrower than the wheelchair? - What information will be provided to consumers to ensure they know whether their mobility aid will fit? |
| # Clause | # HREOC recommendation | # APTJC’s comment |
| ### 3.1 Manoeuvring areas - Circulation space for wheelchairs to turn in | # Grant exemption accepting use of lower end of range of dimensions stated for existing premises and infrastructure where required by space constraints. | # Grant exemption, What information will be provided to consumers and suppliers on the allowable footprint of wheelchairs on certain services; and by whom? |
| ### 4.2 Passing areas - Two-way access paths and aerobridges | - Grant exemption for existing platforms, subject to re-opening if Premises Standards process results in different provision and with requirement for reporting during exemption period on instances where exemption has been relied on, impacts for passengers, and measures taken to minimise these. | # Grant exemption |
| ### 5.1 Resting points - When resting points must be provided | - Grant exemption for existing infrastructure to the extent required by site constraints, subject to specification that this applies where constraints completely prevent compliance rather than only adding expense or difficulty, and subject to reporting during exemption period on impacts. | # Grant exemption subject to further consultation occurring. |
| # 8.1 Boarding – Boarding points and kerbs | - Grant exemption specifying that assisted boarding is only required at a single entrance rather than all doors, subject to reporting on measures taken to ensure non-discriminatory operation including staff training and passenger information | # How does HREOC see this working in NSW, where allocated spaces between conveyances are not connected and where conveyances may have multiple access points? |
| # 8.5 Boarding – Width and surface of boarding devices | # Approve exemption | # How will an 800mm wide mobility aid get up a ramp that is narrower? (See also 2.6) Could HREOC please clarify? - Information to consumers regarding the constraints of assistance/service will be required. |
| # 8.7 Boarding – Signals requesting use of boarding device | # Approve exemption | # Approve exemption |
| # 9.1 Allocated space – Minimum size allocated space | # Approve exemption for actions consistent with the Guidelines. | # Approve exemption |
| # 9.10 Allocated space – International symbol of accessibility to be displayed | # Grant exemption | # Grant exemption |
| ### 11.2 Handrails and grabrails – Handrails to be provided on access paths | # Grant exemption for railway platforms; defer decision for other areas for further discussion | # Grant exemption for rail platforms only |
| # Clause | # HREOC recommendation | # APTJC’s comment |
| # 11.3 Handrails - Handrails on steps- | # Grant exemption | # Grant exemption |
| # 12.2 Doorways and doors – Compliance with Australian Standard – premises and infrastructure | # Grant exemption agreeing to revised proposal | # Grant exemption |
| # 12.4 Doorways and doors – Clear opening of doorways | # Grant exemption approving revised proposal | # Grant exemption |
| # 14.1 Stairs – Stairs not to be sole means of access | # Grant exemption for internal stairs on double deck carriages | # Grant exemption |
| # 14.3 Stairs – Compliance with Australian Standards - conveyances | # Approve exemption subject to conditions that direct assistance is provided for access via external stairs and report during exemption period on measures taken to ensure effectiveness of access including staff training and passenger information. | # Approve exemption subject to conditions. |
| # 15.1 Toilets – Unisex accessible toilet – premises and infrastructure | # Grant exemption approving revised proposals subject to report during exemption period on impacts and consultation. | # APTJC would like to receive more information on door pressure requirements. |
| # 15.2 Toilets – Location of accessible toilets | # Grant exemption approving revised proposal subject to reporting during exemption period on locations where co-location is not possible, impacts on passenger amenity, and management approaches including passenger information to ensure equal access. | # Grant exemption |
| # 15.3 – Unisex accessible toilet – accessible rail cars | # Compliant mobility aid: refuse separate exemption as unnecessary – issues dealt with by provisions on access path and proposal on 15.4 below. - Requirement for trains rather than for accessible carriages; grant exemption approving revised proposal subject to condition that first toilet in any accessible car by a unisex accessible toilet. | # Agree with HREOC |
| # 15.4 Toilets – Requirements for accessible toilets – ferries and accessible rail cars | # Approve exemption where compliance not possible (Narrow gauge and standard gauge stock) subject to a requirement to report during exemption period on implementation and consultation; seek further information on whether exemption should be extended to all carriages. | # Approve exemption subject to the issue of consultation. |
| # Clause | # HREOC recommendation | # APTJC’s comment |
| # 16.5 Symbols – Accessibility symbol to be visible on accessible doors | # Grant exemption | # Refuse exemption. APTJC are strongly supportive of having the symbol inside and outside the carriage. It would assist on reminding all passengers of the priority of the allocated spaces – the internal signs inform everyone using the service. While general passengers may not notice the symbol on the exterior of the carriage when they board, the internal signs may trigger other passengers to move to make the space available when a passenger boards who needs the allocated space. - HREOC has agreed to a single boarding point. What if there are multiple boarding points - should these also be marked? |
| # 17.5 Signs – Electronic notices | # Grant exemption subject to report on consultation and experience during exemption period | # Grant exemption |
| # 17.6 Signs - Raised lettering or symbols or use of Braille | # Approve exemption for conveyances; defer decision regarding infrastructure for further discussion | # Approve exemption in relation to conveyances only |
| ### 18.1 Tactile ground surface indicators -Location | # Approve exemption subject to conditions requiring report on experience and consultation during exemption period. | # Approve exemption. How does HREOC propose reporting and consultation to be undertaken? |
| ### 18.2 Tactile ground surface indicators - Style and dimensions | # Approve exemption from TGSI requirement on conveyances; further discussion required to confirm appropriateness of proposed exemption on infrastructure constraints. | # Approve exemption |
| # 20.1 Lighting – Illumination levels – premises and infrastructure | # Grant exemption | # Grant exemption |
| # 21.1 Controls – Compliance with Australian Standard – premises and infrastructure | # Grant exemption as for 15.1 | # Grant exemption |
| - 25.3 Payment of fares - Vending machines | - Approve exemption subject to consultation and reporting during exemption period on equivalent access measures. | - Approve exemption. Subject to suitable redrafting of the requirements being endorsed by people with disabilities and HREOC. |
| # Clause | # HREOC recommendation | # APTJC’s comment |
| - 26.2 Hearing augmentation - listening systems - Public address systems - conveyances | - Grant exemption subject to conditions requiring consultation and report within 12 months on equivalent access measures | - Grant exemption |
| - 27.2 Direct assistance to be provided | - Approve exemption subject to requiring consultation on equivalent access measures and report on experience during exemption period. | - How will equivalent access be measured? |
| - 27.3 Information – Size and format of printing | - Grant exemption subject to report on consultation and experience during exemption period | - Grant exemption. |
| - 28.1`Booked services – Notice of requirement for accessible travel; - 28.2 Booked services – Period of notice of requirement for accessible travel | - Grant exemption to allow requiring reasonable notice for accessible travel or assistance. - Defer decision on exemption in relation to provision of assistance transferring to seats on booked services; further discussion required | - Grant exemption. - What is reasonable notice? - Does HREOC see the current practice with airlines regarding what is reasonable, being used in rail? - Should operators publicly state what they regard reasonable to be? |
| - 31 Priority – Priority seating | - Grant exemption | - Grant exemption. What is to stop an operator from locating priority seating in totally inappropriate places on a conveyance? |