ARA EXEMPTION: AGREED CLAUSES AND GUIDELINES

Contents

1.9  Access path	4

1.11AX  Assistance Dog	4

1.11BX  Boarding Point	5

1.11CX  Booked services on trains	5

1.11DX  Unbooked services on trains	5

1.15X  Disability aid	6

1.18  Infrastructure	7

1.18X  Level crossing	7

1.19AX  Mobility aid	7

1.19BX  Nominated accessible boarding point	8

1.21  Premises	8

1.23X  Sleeping berth	9

1X.1  Design criteria	9

1X.2  Performance criteria	10

1X.3  Orientation	12

2.1  Unhindered passage	13

2.2  Continuous accessibility	14

2.4  Minimum unobstructed width	15

2.4X  Minimum unobstructed width for railway platforms	16

2.5  Poles and obstacles, etc	18

2.5AX  Level Crossings	19

2.6  Access paths — conveyances	19

2.8  Extent of path	20

3.1  Circulation space for mobility aids to turn in	21

4.1  Minimum width	22

4.2  Two-way access paths and aerobridges	22

4.3  Passing areas — conveyances	23

5.1  When resting points must be provided	24

6.1  Ramps on access paths	25

6.2  Boarding ramps	25

6.3  Minimum allowable width	26

8.1  Boarding points and kerbs	26

8.2  When boarding devices must be provided	27

8.5 Width and surface of boarding devices	28

8.6  Maximum load to be supported by boarding device	28

8.7  Signals requesting use of boarding device	29

8.8  Notification by passenger of need for boarding device	29

9.1  Minimum size for allocated space	31

9.6  Number of allocated spaces to be provided — rail, tram, light rail cars	31

9.7  Consolidation of allocated spaces	33

9.10  International symbol of accessibility to be displayed	33

10.1  Compliance with Australian Standard	34

10.1X  Compliance with Australian Standard	34

11.1  Compliance with Australian Standard — premises and infrastructure	35

11.2  Handrails to be provided on access paths	36

11.3  Handrails on steps	36

11.4  Handrails above access paths	37

11.5  Grabrail compliance requirements	37

11.6  Grabrail to be provided where fares are to be paid	38

11.7  Grabrails to be provided in allocated spaces	39

12.1  Doors on access paths	39

12.2  Compliance with Australian Standard — premises and infrastructure	40

12.3  Weight activated doors and sensors	41

12.3  Weight activated doors and sensors	41

12.4  Clear opening of doorways	41

13.1  Compliance with Australian Standard — premises and infrastructure	42

14.1 Stairs not to be sole means of access	42

14.3  Stairs on conveyances	43

15.1  Unisex accessible toilet — premises and infrastructure	44

15.2  Location of accessible toilets	44

15.3  Unisex accessible toilet — ferries and trains	45

15.4  Requirements for accessible toilets — ferries and accessible rail cars	45

16.2  Compliance with AS2899.1 (1986)	47

16.3  Accessibility symbols to incorporate directional arrows	47

16.5  Accessibility symbol to be visible on accessible doors	48

17.4  Destination signs to be visible from boarding point	48

17.5  Electronic notices	49

17.6  Raised lettering or symbols or use of Braille	49

18.1  Location	50

18.2  Style and dimensions	51

18.4  Instalment at railway stations	52

19.1  Emergency warning systems	52

20.1  Illumination levels — premises and infrastructure	53

21.1  Compliance with Australian Standard — premises and infrastructure	54

21.2  Passenger-operated devices for opening and closing doors	54

21.3  Location of passenger-operated controls for opening and locking doors	55

22.1  Tables, benches, counters, etc	56

22.5  Accessible sleeping berths — trains	57

23.1  Seats	57

24.1  Gateways and checkouts	58

25.3  Ticket vending machines	60

25.4  Circulation space in front of a ticket vending machine	60

26.2  Public address systems — conveyances	61

27.1  Access to information about transport services	61

27.2  Equivalent access	62

27.3  Size and format of printing	63

27.4  Access to information about location	63

28.1  Notice of requirement for accessible travel	64

28.2  Period of notice of requirement for accessible travel	65

28.3  Location of carers, assistants and assistance dogs	65

29.1  Equal access to food and drink services	66

30.1  Disability aids to be in addition to baggage allowance	67

31.1  Priority seating	68

| Existing Transport Standard | ARA Proposal | Agreed Standard | Agreed Guidelines |
| --- | --- | --- | --- |
| 1.9 Access path An access path is a path that permits independent travel for all passengers within public transport premises, infrastructure or conveyances. | # 1.9 Access path An access path is a path that permits independent travel for all passengers within public transport premises, infrastructure or conveyances. An access path may include a compliant walkway, ramp or lift. An access path on a rail conveyance is a path from the external passenger doorway to the allocated space, priority seats and other essential facilities for passengers with mobility aids. An access path on a railway station is a clear and unobstructed path from a nominated boundary point to a nominated accessible boarding point on each platform and to all accessible facilities. | 1.9 Access path An access path is a path that permits independent travel for all passengers within public transport premises, infrastructure or conveyances. An access path may include a compliant walkway, ramp or lift. An access path on a rail conveyance is a path from the external passenger doorway to the allocated space, priority seats and other essential facilities accessible to passengers with mobility aids. An access path on an existing railway station is a clear and unobstructed path from a nominated boundary point to a nominated accessible boarding point on each platform and to all accessible facilities. An access path on a new railway station is a path that permits independent travel for all passengers within the station.. | Public paths within new rail premises and infrastructure should comply to the maximum extent possible with the Transport Standards except where structural or technical constraints prevent compliance. On existing stations, or stations rebuilt within an existing rail corridor, at least one access path leading to all essential facilities is required to be accessible. Where possible, the nominated boundary point on existing stations should be either the principal entrance or the one most accessible to the surrounding pedestrian and transport modes. |
| No Clause | # 1.11AX Assistance Dog (1) An assistance dog, is a dog that is registered and trained to alleviate the effects of a person’s disability on public transport. (2) An assistance dog must be under the direct control of the person with the disability at all times and wear a recognised form of identification as a trained assistance dog. (3) The handler is solely responsible for the conduct, care, feeding, hygiene and toileting of their animal. | 1.11AX Assistance Animal (1) An assistance animal, is an animal that is registered and trained to alleviate the effects of a person’s disability on public transport. (2) An assistance animal must be under the direct control of the person with the disability at all times and wear a recognised form of identification as a trained assistance animal. (3) The handler is solely responsible for the conduct, care, feeding, hygiene and toileting of their animal. | An assistance animal is a 'working' animal as distinctfrom a 'companion/comfort' animal which is not trained to alleviate a disability on public transport. Dogs are the most common 'assistance animals' but other animals (trained and registered) may be needed where people are not compatible with, or have religious aversion to, dogs. The animal must be registered, and the handler must have verifiable evidence that the animal is accredited through an organisation recognised as a trainer of assistance animals for people with disabilities. |
| No Clause | # 1.11BX Boarding Point A boarding point is the place where passengers board a public transport conveyance. | 1.11BX Boarding Point A rail Boarding point is the nominated accessible boarding point from which passengers board a rail conveyance. |  |
| No Clause | # 1.11CX Booked services on trains Booked services on trains are long distance passenger train services where prior booking is required to purchase a berth, seat and/or allocated space. | Agree |  |
| No Clause | # 1.11DX Unbooked services on trains Unbooked services on trains are generally suburban passenger train services where no seat is booked or reserved for a specific person. | 1.11DX Unbooked services on trains Unbooked services on trains are passenger train services where no seat and/or allocated space is booked or reserved for a specific person. |  |
| No Clause | # 1.15X Disability aid A disability aid is a device designed to be used by a person with a disability to alleviate the effects of that disability. Disability aids include personal mobility, sensory and medical devices, but do not include devices designed for use by more than one person at the one time nor those that are primarily designed for use as transportation vehicles. Constraints that are part of providing safe and effective public transport services and operations may limit the types of aids that can be accommodated on transport services, premises and infrastructure. Passengers are responsible for the supply, assembly and operation of their disability aids at all times while using public transport services / facilities. | 1.15X Disability aid A disability aid is a device designed to be used by a person with a disability to alleviate the effects of that disability. Disability aids include mobility aids but do not include mobility aids designed for use by more than one person at a time. Constraints that are part of providing safe and effective public transport services and operations may limit the types of aids that can be accommodated on transport services, premises and infrastructure. Passengers on rail conveyances are responsible for the supply, assembly and operation of their disability aids at all times. | Disability aids include personal hygiene and medical equipment, communication and security devices. For the occupational health and safety of both passenger and crew, train crew are unable to assist with the assembly or disassembly of disability aids. |
| 1.18 Infrastructure (1) Infrastructure is any structure or facility that is used by passengers in conjunction with travelling on a public transport service. (2) Infrastructure does not include any area beyond immediate boarding points (for example, bus stops, wharves, ranks, rail stations, terminals). | # 1.18 Infrastructure (1) Infrastructure is any structure or facility that is used by passengers to gain direct access to a public transport service. Infrastructure includes boarding points, footbridges, open railway platforms and open shelters. (2) Infrastructure does not include any area beyond immediate boarding points. | 1.18 Infrastructure Infrastructure is any structure or facility that is used by passengers to gain direct access to a public transport service. Infrastructure includes boarding points, footbridges, open railway platforms and open shelters. |  |
| No Clause | # 1.18X Level crossing A level crossing is any vehicular and/or pedestrian crossing of a railway corridor at grade, as defined in AS1742.7. | 1.18X Level crossing A level crossing is any vehicular and/or pedestrian crossing of a railway corridor at grade. |  |
| No Clause | # 1.19AX Mobility aid (1) A mobility aid is a disability aid that is designed to be used by one person with a disability to alleviate the effects of their limited mobility. (2) Mobility aids include wheelchairs and motorised scooters but do not include those devices that are primarily designed for use as transport or recreational vehicles. (3) A mobility aid for use on public transport services, premises and infrastructure must meet the requirements of Part 1X Mobility Aids. | 1.19AX Mobility aid (1) A mobility aid is designed to be used by a person with a disability to alleviate the effects of their limited mobility (2) A mobility aid for use on public transport services, premises and infrastructure must meet the requirements of Part 1X Mobility Aids. | A mobility aid provides direct assistance with mobility while a disability aid is an item that assists with the disability but is not directly involved with mobility (eg medication packs, oxygen bottles, security alarms, communication devices). Mobility aids include wheelchairs and motorised scooters, white canes, assistance animals, walking frames, walking sticks and crutches. Mobility aids do not include devices primarily designed for use as recreational vehicles. |
| No Clause | # 1.19BX Nominated accessible boarding point A nominated accessible boarding point is a boarding point nominated by the provider for accessible boarding by people with disabilities to provide a coordinated and central location for providing direct assistance if necessary, and the provision of core facilities/services. | 1.19BX Nominated accessible boarding point A nominated accessible boarding point is a boarding point nominated by the provider for accessible boarding by people with disabilities. It provides a coordinated and central location for providing direct boarding/alighting assistance, and location of core facilities / services in convenient proximity. |  |
| 1.21 Premises (1) Premises are structures, buildings or attached facilities that an operator provides for passenger use as part of a public transport service. (2) Premises are a form of infrastructure. | # 1.21 Premises (1) Premises are structures, buildings or attached facilities that a provider supplies for passenger use as part of a public transport service. (2) Premises are a form of infrastructure. (3) Premises do not include footbridges, open railway platforms or open shelters, which are covered under 1.18 Infrastructure. | 1.21 Premises (1) Premises are structures, buildings or attached facilities that a provider supplies for passenger use as part of a public transport service. (2) Premises are distinct from infrastructure in that they have a building Classification in the Building Code of Australia. (3) Premises in the rail environment do not include footbridges, open railway platforms or open shelters, which are covered under 1.18 Infrastructure. |  |
| No Clause | # 1.23X Sleeping berth A sleeping berth is a sleeping space which may be in a compartment, or a seat/bed recliner chair in an open area. | Agree |  |
| No Clause | # 1X.1 Design criteria Maximum size (1) The maximum size of a mobility aid for use on public transport services, premises and infrastructure is less than 750mm wide by 1300mm long with a maximum head clearance height of 1500mm. (2) The front of the mobility aid shall be designed to meet the Figure 1X-X minimum knee and foot clearance beneath tables, counters and worktops. Manoeuvrability (3) The device must be able to turn through 180 degrees within an area of 2070 mm by 1540 mm. Mass (4) The combined mass of the passenger, the mobility aid, and any required assistant must be less than 300 kg. | Part 1X.1 Mobility aids 1X.1 Design Criteria Maximum size (1) Mobility aid footprints must be no greater than 800 mm wide and 1300mm long on rail services, premises and infrastructure. In certain limited situations in existing conveyances, infrastructure or premises, mobility aids must be able to pass through gaps of 760 mm width. (2) To access tables, counters, worktops, wash basins or the like, users of mobility aids must be able to fit the knee clearance specifications of AS1428.1. Manoeuvrability (3) The space for a turn of 180 degrees is an area with dimensions in the ranges of 2070 mm by 1540 mm. Mass (4) To access a boarding ramp, the combined mass of the passenger, the mobility aid, and any required assistant must be less than 300 kg. | In certain limited locations mobility aids must be able to pass through restricted spaces down to 760 mm wide (eg along a corridor passing an accessible toilet in a rail carriage or through a door of an existing rail carriage). Users of scooters, walking frames, crutches and assistance animals use standard seating in dining and similar areas. Wheelchair users will need tables and allocated spaces to use dining and similar. The 180 degree turn space is to accommodate a mobility aid that can comply with Part 1X. Where possible, the space allowed should be 2270 mm by 1740 mm. Scooters may reverse in and out of the space. |
| No Clause | # 1X.2 Performance criteria Braking (1) Mobility aids must have effective braking systems and passengers must apply the brakes while conveyances are in motion, and at other times as required for safe travel. Anchoring (2) Mobility aids must comply with relevant safety requirements eg criteria for anchorage points. Stability (3) Mobility aids must be stable under normal conveyance forces of acceleration, deceleration, cornering and pitching. Propulsion (4) Internal or external combustion driven aids are not permitted on transport premises, infrastructure or conveyances. Batteries (5) Electric mobility aids must comply with regulations governing the carriage of batteries on public transport. Batteries need to be adequately secured and contained. Gel or solid state options should be considered. Wheels and Gaps (6) A mobility aid should be able to : (a) cross a horizontal gap of 75 mm for pedestrian level crossings as defined in AS1742.7; and (b) mount a vertical rise (bump) up to 20 mm; and (c) cross grating gaps up to 13 mm wide and 150 mm long, and cross a horizontal gap up to 65 mm wide when combined with a vertical gap of up to 20 mm at station boarding points. Ramps (7) Mobility aids should be able to negotiate: (a) a 1 in 12 ramp unassisted; and (b) a 1 in 8 grade unassisted where the ramp is less than 1520 mm; and (c) a 1 in 4 ramp with assisted access. Operation and storage Passengers are responsible for: (8) (a) any dis/assembly, operation and manoeuvrability of their disability aids into and out of public transport conveyances, premises and infrastructure (b) storage of their disability aids prior to and after travel on public transport services. Working Environment (9) The mobility aid must be able to operate safely in internal and external environments under varying ambient conditions. | 1X.2 Performance criteria Braking (1) Mobility aids must have effective braking systems and passengers must apply the brakes while conveyances are in motion, and at other times as required for safe travel. Anchoring (2) Mobility aids must comply with relevant safety requirements eg criteria for safe anchorage. Stability (3) Mobility aids must be stable under normal conveyance forces of acceleration, deceleration, cornering and pitching. Propulsion (4) Internal or external combustion driven aids are not permitted on transport premises, infrastructure or conveyances. Batteries (5) Electric mobility aids must comply with regulations governing the carriage of batteries on public transport. Batteries need to be adequately secured and contained. Gel or solid state are preferred. Wheels and Gaps (6) A mobility aid should be able to : (a) cross a horizontal gap of 40 mm; and (b) mount a vertical rise (bump) up to 12 mm; and (c) cross grating gaps up to 13 mm wide and 150 mm long. Ramps (7) Persons using mobility aids should be able to negotiate boarding ramps: (a) a 1 in 14 grade unassisted where the ramp is longer than 1520mm length; and (b) a 1 in 8 grade unassisted where the ramp is less than 1520 mm; and (c) a 1 in 4 ramp with assisted access where the ramp is less than 1520 mm. Operation and storage (8) Rail passengers are responsible for: (a) any dis/assembly and operation of their disability aids in rail conveyances, premises and infrastructure (b) Manoeuvring their own mobility aids. (c) storage of their disability aids prior to and after travel on public transport services. Working Environment (9) The mobility aid must be able to operate safely in internal and external environments under varying ambient and operating conditions. | Owing to the technical constraints of the rail environment, flange gaps may be up to 75 mm width. Mobility aids should, where possible, be designed to cross existing flange gaps of up to 75mm. Direct assistance will only be provided at the nominated accessible boarding point. Assistance may be requested where a boarding ramp of less than 1520 mm length has a gradient steeper than 1:8. |
| No Clause | # 1X.3 Orientation Transport operators may determine the location and orientation of passengers on premises, infrastructure and conveyances. Conveyances: except dedicated school buses, and small aircraft Premises Infrastructure: except airports that do not accept regular public transport services | 1X.3 Orientation Transport operators may determine the location and orientation of allocated spaces and seating. Conveyances:  Accessible rail car |  |
| 2.1 Unhindered passage (1) An access path that allows unhindered passage must be provided along a walkway, ramp or landing. (2) An access path must comply with AS1428.2 (1992) Clause 8.1. Premises Infrastructure: except airports that do not accept regular public transport services | # 2.1 Unhindered passage (1) An access path that allows unhindered passage to a boarding point must be provided. The Provider may nominate the access path. (2) An access path must comply with AS1428.1 (2001), subject to Part 2.4X and 2.5X with the following exceptions and additions: (a) Walkways, ramps and landings shall have an unobstructed width of not less than 1200 mm. (3) An access path must have a maximum crossfall of 1 in 40 except for 1:33 if the surface is a bituminous seal. Premises Infrastructure: except airports that do not accept regular public transport services | 2.1 Unhindered passage (1) An access path that allows unhindered passage must be provided along a walkway, ramp or landing. (2) On existing premises and infrastructure, where technical constraints do not permit compliance with Part 2.1(1), an access path that allows unhindered passage to a nominated accessible boarding point must be provided. The provider may nominate the access path. (3) An access path must comply with AS1428.1 (2001), subject to Part 2.X and with the following exceptions and additions: (a) Walkways, ramps and landings shall have an unobstructed width of not less than 1200 mm. (4) An access path must have a maximum crossfall of 1 in 40 except for 1:33 if the surface is a bituminous seal. Premises: Rail premises Infrastructure: Rail infrastructure | On new infrastructure and premises, access paths can usually be accessible. Existing structures can present legitimate barriers to full compliance. Where a path of travel on an existing station can be made accessible without unjustifiable hardship, then this should occur. Where possible, the accessible entry point to platforms should give the 'best connection' with the surrounding pedestrian and transport environment. |
| 2.2 Continuous accessibility An access path must comply with AS1428.2 (1992) Clause 7, Continuous accessible path of travel. Premises Infrastructure: except airports that do not accept regular public transport services | # 2.2 Continuous accessibility An access path shall be provided as follows: (1) Accessible paths of travel within the boundary of the site shall be provided from transportation stops, accessible parking and accessible passenger loading zones, and accessible public streets or walkways to the accessible building entrance they serve. (2) Accessible paths of travel shall connect accessible buildings, facilities, and spaces that are on the same site. (3) Accessible paths of travel shall connect accessible building entrances with all accessible spaces and facilities within a building. (4) Accessible paths of travel shall connect accessible entrances of each accessible building with those exterior and interior spaces and facilities that serve it. Premises Infrastructure: except airports that do not accept regular public transport services | 2.2 Continuous accessibility An access path shall be provided as follows: (1) Accessible paths of travel within the boundary of the site shall be provided from accessible transportation stops, accessible parking and accessible passenger loading zones, and accessible public streets or walkways to the accessible building entrance they serve. (2) Accessible paths of travel shall connect accessible buildings, facilities, and spaces that are on the same site. (3) Accessible paths of travel shall connect accessible building entrances with all accessible spaces and facilities within a building. (4) Accessible paths of travel shall connect accessible entrances of each accessible building with those exterior and interior spaces and facilities that serve it. Premises: Rail premises Infrastructure: Rail infrastructure | Fatigue, caused by unnecessarily long paths of travel, is a significant barrier to the use of the built environment for many people. In the design of new facilities and the refurbishment of existing facilities, every effort must be made to minimise the length of the access paths between entrances, boarding points and other accessible facilities. |
| 2.4 Minimum unobstructed width (1) The minimum unobstructed width of an access path must be 1200 mm (AS1428.2 (1992) Clause 6.4, Width of path of travel). (2) However, the minimum unobstructed width of a moving footway may be 850 mm. Premises Infrastructure: except airports that do not accept regular public transport services | # 2.4 Minimum unobstructed width (1) The minimum unobstructed width of an access path must be 1200 mm. For doorways and gateways see relevant clauses. (2) However, the minimum unobstructed width of a moving footway may be 850 mm. Premises Infrastructure: - except airports that do not accept regular public transport services - except railway platforms | 2.4 Minimum unobstructed width (1) The minimum unobstructed width of an access path must be 1200 mm. For doorways and gateways see relevant clauses. (2) However, the minimum unobstructed width of a moving footway may be 850 mm. Premises: Rail premises Infrastructure: Rail infrastructure | Existing infrastructure and premises may not be able to meet the 1200 mm minimum width for access paths in all locations due to structural and technical constraints. In these situations, a path of >1000 mm width should be provided. If the access path stricture is of a very limited nature (eg access around a power pole or column) a minimum of 850 mm width is acceptable. Alternate paths of travel should be provided if access path strictures are of an extended nature (eg over several metres). |
| No Clause | # 2.4X Minimum unobstructed width for railway platforms Width of an access path at railway stations must comply with the following: (1) New platforms New platforms must comply with Part 2.4. (2) Existing stations and platforms Existing stations and platforms must be brought into compliance with these Standards to the maximum extent possible. Fittings and fixtures (bins, seats, vending machines) must be relocated to the extent possible to achieve maximum compliance. Requirements for warning TGSIs to be installed on platform edges may compromise clear access paths in some circumstances as indicated below. (3) Site constraints preventing fully compliant paths of travel Clear access path width may be reduced from 1200 mm to 1000 mm where site constraints or fixed structures preclude full compliance. This includes new structures constructed within existing site constraints. (4) Existing platforms - exceptionally severe site constraints Where an access path width of 1000 mm cannot be achieved adjacent to the 600 mm warning TGSI then compliance must be maximized by a range of options, which may include: (a) Equivalent access by maintaining an access path of 1000 mm (minimum) to at least one side of an obstacle. (b) Providing an access path of 1000 mm (minimum) to a nominated accessible boarding point and equivalent access to essential facilities on the platform. (5) Using platform edge warning TGSIs as part of the access path Where 1000 mm clear access path cannot be achieved by the options outlined above, warning TGSIs may form part of the access path (for example where power stanchions or other fixed structures on platforms need to be passed). The overlap of warning TGSIs and access paths must be minimised. (6) Information to passengers Where stations do not have a clear access path width of 1000 mm, operators should provide passengers with information (including at stations and through internet sites and other appropriate information methods) detailing access features provided. (7) Moving footways The minimum unobstructed width of a moving footway may be 850 mm. Infrastructure: Railway platforms | Agree | Structural and technical constraints may preclude full compliance on stations upgraded to new within an existing rail corridor. Compliance must be maximised at these locations. Warning TGSIs are an important safety feature for people who have vision impairment. However, as they pose an access barrier for many people who have mobility impairments, they do not form a legitimate part of an access path. Every attempt to keep access paths clear of TGSIs must be made, but not at the expense of the safety of blind passengers. Alternatives eg guard rails and landscaping, can be considered if TGSIs are considered inappropriate in certain circumstances. |
| 2.5 Poles and obstacles, etc (1) Poles, columns, stanchions, bollards and fixtures must not project into an access path. (2) Obstacles that abut an access path must have a luminance contrast with a background of not less than 30%. Premises Infrastructure: except airports that do not accept regular public transport services | # 2.5 Poles and obstacles, etc (1) Poles, columns, stanchions, bollards and fixtures must not project into an access path, except as provided for in Part 2.4X. (2) Obstacles that abut an access path must have a colour contrast with the background. Premises Infrastructure: except airports that do not accept regular public transport services | 2.5 Poles and obstacles, etc (1) Poles, columns, stanchions, bollards and fixtures must not project into an access path, except as provided for in Part 2.4X. (2) Obstacles that abut an access path must have a distinct colour contrast with the background. Premises: Rail premises Infrastructure: Rail infrastructure | While colour contrast is easily determined subjectively it is most important to emphasise a distinct or strong colour contrast with the background. Luminance contrast fails to consistently produce a visual contrast between surfaces (from field experience). Australian Standard (DR 00069 CP) offered guidance on inappropriate colour combinations: A combination of any two of the following colours will be difficult for people with a colour deficiency to distinguish: - green, olive green, yellow, orange, pink and red. |
| No Clause | # 2.5AX Level Crossings If a public pedestrian level crossing is used as a direct access path to a boarding point it shall comply with AS1742.7 Pedestrian Level Crossings. Infrastructure | 2.5AX Level Crossings If a public pedestrian level crossing is used as a direct access path to a boarding point it shall, so far as is technically and structurally possible, comply with the requirements for an access path. Infrastructure: Level crossings on direct access paths to boarding points | A level crossing has elements, such as 75 mm flangeway gaps, that can not at this time be made accessible. Rail providers must seek a safe solution to reduce the flangeway width, however these will remain under 'unjustifiable hardship' provisions until a technical solution is devised. |
| 2.6 Access paths — conveyances (1) Subject to subsection (3) and section 2.7, an access path that allows continuous and unhindered passage must be provided with a minimum width of at least 850 mm. (2) Subsection (1) applies to doorways and stairs, and between entrances, exits, allocated spaces and other essential facilities for passengers using wheelchairs and other mobility aids. (3) If the conveyance exists or is ordered before the commencement of this section, the minimum width may be reduced to 800 mm at any doorway restriction. Conveyances  Buses  Ferries  Trains  Trams  Light rail | # 2.6 Access paths — conveyances (1) Subject to subsection (2) and sections 2.7 and 2.8, a clear access path with a width of 850 mm, except where design constraints require local restriction to a minimum of 750 mm,, is required extending from the entrance doorway to the allocated spaces, priority seats and other essential facilities for passengers with mobility aids. (2) If the conveyance exists or is ordered before 2 April 2004, the minimum width may be reduced to 750 mm at any doorway restriction. (3) Some sections of a conveyance may not be available to all passengers. Conveyances  Buses  Ferries  Trains  Trams  Light rail | 2.6 Access paths — conveyances (1) In new conveyances, subject to subsection (2) and sections 2.7 and 2.8. a clear access path with a width of 850 mm, except where design constraints require local restriction to a minimum of 760 mm, is required extending from the entrance doorway to the allocated spaces, priority seats and other essential facilities for passengers with mobility aids. (2) If the conveyance exists or is ordered before 2 April 2004, the minimum width may be reduced to 750 mm at any doorway restriction. (3) Some sections of a conveyance may not be accessible to all passengers. Conveyances  Trains  Light rail  Trams |  |
| 2.8 Extent of path (1) An access path must extend from the entrance of a conveyance to the facilities or designated spaces provided for passengers with disabilities. (2) Up to 50 mm of an adjacent allocated space may be used as part of the access path. (3) If an access path cannot be provided, the operator must provide equivalent access by direct assistance. Conveyances  Buses  Ferries  Trains  Trams  Light rail | # 2.8 Extent of path (1) An access path must extend from the entrance of a conveyance to the allocated spaces, priority seats and other essential facilities for passengers with mobility aids and people with vision impairment using canes and assistance dogs. (2) Up to 50 mm of an adjacent allocated space may be used as part of the access path. (3) If an access path cannot be provided, the operator must provide equivalent access by direct assistance. (4) When allocated spaces adjacent to an access path are occupied, some manoeuvring by people in the allocated spaces may be required to achieve a clear access path. (5) An access path need not extend through an area with hearing augmentation. Conveyances  Buses  Ferries  Trains  Trams  Light rail | 2.8 Extent of path (1) An access path must extend from the entrance of a conveyance to the allocated spaces, priority seats and other essential facilities for passengers with mobility aids. (2) Up to 50 mm of an adjacent allocated space may be used as part of the access path. (3) If an access path cannot be provided, the operator must provide equivalent access by direct assistance. (4) When allocated spaces adjacent to an access path are occupied, some manoeuvring by people in the allocated spaces may be required to achieve a clear access path. (5) An access path need not extend to / through an area with hearing augmentation. Conveyances  Trains  Light rail  Trams | Access paths need not extend through areas of hearing augmentation. Where people who are deaf or hearing impaired need to receive essential information equivalent access solutions must be provided. On booked services, the use of a carriage chair supplied by the rail service operator may form part of a direct assistance solution where door, corridor or toilet dimensions are constrained. |
| 3.1 Circulation space for wheelchairs to turn in A manoeuvring area must comply with AS1428.2 (1992) Clause 6.2, Circulation space for a 180 degree wheelchair turn. Premises Infrastructure: except airports that do not accept regular public transport services | # 3.1 Circulation space for mobility aids to turn in A manoeuvring area must allow for a mobility aid to make an 180o turn. Where possible, the space required for a mobility aid to make an 180o turn shall be no less than 2070mm in the direction of travel and not less than 1540mm wide. Premises Infrastructure: except airports that do not accept regular public transport services | 3.1 Circulation space for mobility aids to turn in A manoeuvring area must allow for a mobility aid to make an 180o turn. Space with dimensions in the range of 2270–2070 mm in the direction of travel and 1740–1540 mm wide shall be provided. Premises: Rail premises Infrastructure: Rail infrastructure | For existing structures the largest dimension possible in the range has precedence. For new premises and infrastructure, the largest dimension only is applicable. It is accepted that existing premises and infrastructure may have space constraints that prevent the use of the enhanced requirements for a 180 degree turn. However, where possible, they should comply with the enhanced dimensions. |
| 4.1 Minimum width A passing area must have a minimum width of 1800 mm (AS1428.2 (1992) Clause 6.5 (a), Passing space for wheelchairs). | # 4.1 Minimum width Delete Clause 4.1 Minimum width | Agree |  |
| 4.2 Two-way access paths and aerobridges (1) A passing area must be provided at least every 6 metres along any two-way access path that is less than 1800 mm wide (AS1428.2 (1992) Clause 6.5 (b), Passing space for wheelchairs and Figure 3). (2) A passing area is not required on an aerobridge. Premises Infrastructure: except airports that do not accept regular public transport services | # 4.2 Two-way access paths and aerobridges (1) Where a path of travel is less than 1800mm wide, passing areas at intervals of not more than 9 metres shall be provided as follows: (a) On one side of the path of travel – the passing area shall not be less then 1600mm long and 1800mm wide. (b) With space distributed equally on both sides of the path of travel – the passing area shall be not less than 2000mm long and 1800 mm wide. (2) A passing area is not required on an aerobridge. (3) For sections of railway stations covered by Part 2.4X, no passing areas are required. Premises Infrastructure: except airports that do not accept regular public transport services | 4.2 Two-way access paths and aerobridges (1) Where an access path is less than 1800 mm wide, passing areas at intervals of not more than 9 metres shall be provided as follows: (a) On one side of the path of travel – the passing area shall not be less then 1600 mm long and 1800 mm wide. (b) With space distributed equally on both sides of the path of travel – the passing area shall be not less than 2000 mm long and 1800 mm wide. (2) For sections of existing railway stations covered by Part 2.4X, no passing areas are required. Premises: Rail premises Infrastructure: Rail infrastructure |  |
| 4.3 Passing areas — conveyances (1) A ferry designed to carry more than 1 wheelchair must include at least 1 passing area for each accessible deck. (2) A train designed to carry more than 1 wheelchair must include at least 1 passing area for each accessible rail car. (3) The passing area must enable passengers travelling in mobility aids (conforming with the assumptions indicated in Part 40.1 of the Guidelines) to pass each other. (4) The passing area may comprise part of the allocated space or circulation space or both. Conveyances  Ferries  Trains | # 4.3 Passing areas — conveyances (1) A ferry designed to carry more than 1 wheel chair must include at least 1 passing area for each accessible deck. (2) Each accessible rail car designed to carry more than 1 mobility aid must include at least 1 passing area. (3) The passing area must enable passengers travelling in mobility aids (conforming with Part 1X Mobility Aids and the assumptions indicated in Part 40.1 of the Guidelines) to pass each other. (4) The passing area may comprise part of the allocated space or circulation space or both. (5) Turn taking is an acceptable practice to allow mobility aids to pass. Conveyances  Ferries  Accessible rail car | 4.3 Passing areas — conveyances (1) Each accessible rail car designed to carry more than 1 mobility aid must include at least 1 passing area. (2) The passing area must enable passengers travelling in mobility aids (conforming with Part 1X) to pass each other. (3) The passing area may comprise part of the allocated space or circulation space or both. (4) Turn taking is an acceptable practice to allow mobility aids to pass. Conveyances  Accessible rail car | It is reasonable to expect that all passengers will be cooperative and helpful in ensuring that other passengers are able to use the transport service. |
| 5.1 When resting points must be provided (1) There must be resting points for passengers along an access path if the walking distance between facilities or services exceeds 60 metres (AS1428.2 (1992) Note to Clause 7, Continuous accessible path of travel). (2) A resting point must provide seats (AS1428.2 (1992) Clause 27.1(a), Street Furniture). Premises Infrastructure: except airports that do not accept regular public transport services | # 5.1 When resting points must be provided (1) There must be resting points for passengers along an access path if the walking distance between facilities or services exceeds 60 metres. (2) Where major site restraints do not allow for resting points as required, these shall be provided in locations where the infrastructure permits. (3) A resting point must provide seating for a minimum of two people and at least one space for a mobility aid. Seats shall be a minimum of 500 mm away from the path of travel. Premises Infrastructure: except airports that do not accept regular public transport services | 5.1 When resting points must be provided (1) There must be resting points for passengers along an access path if the walking distance between facilities or services exceeds 60 metres. (2) For existing premises or infrastructure, where major site restraints do not allow for resting points as required, these shall be provided in locations where the infrastructure permits. (3) A resting point must provide seating for a minimum of two people and at least one space for a mobility aid. Seats shall be a minimum of 500 mm away from the path of travel. Premises: Rail premises Infrastructure: Rail infrastructure | New platforms can be designed to comply fully with the existing Parts 5.1(1) and (3). For existing premises and infrastructure, full compliance shall be achieved wherever possible |
| 6.1 Ramps on access paths A ramp on an access path must comply with AS1428.2 (1992) Clause 8. Premises Infrastructure: except airports that do not accept regular public transport services | # 6.1 Ramps on access paths A ramp on an access path must comply with AS1428.1 (2001) and; (1) In outdoor conditions, ramps on access paths shall be designed so that water does not accumulate on surfaces. (For requirements for ground surfaces, see AS1428.2(1992) Clause 9.) (2) Kerb ramps shall comply with the requirements set out in the AS1428.1 (2001). Premises Infrastructure: except airports that do not accept regular public transport services | 6.1 Ramps on access paths A ramp on an access path must comply with AS1428.1 (2001) and; (1) In outdoor conditions, ramps on access paths shall be designed so that water does not accumulate on surfaces. (For requirements for ground surfaces, see AS1428.2(1992) Clause 9.) (2) Be not less than 1200 mm between handrails. (3) Kerb ramps shall comply with the requirements set out in the AS1428.1 (2001). Premises: Rail premises Infrastructure: Rail infrastructure |  |
| 6.2 Boarding ramps A boarding ramp must comply with AS3856.1 (1998) Clause 2.1.8 (b), (c), (f) and (g). Conveyances except dedicated school buses and small aircraft | # 6.2 Boarding ramps (1) When two or more ramps are deployed, they shall be attached securely to one another. (2) The surface of the ramp shall have a slip-resistant finish. The surface shall be an acceptable surface when tested in accordance with AS3698.13. Cleats may be provided to assist an attendant using the ramp. (3) A ramp shall have no protrusions extending for more than 6 mm above the surface except cleats or side edge barriers. (4) When the ramp is deployed, there shall be no opening or gap in the ramp more than 40 mm. Conveyances except dedicated school buses and small aircraft | Agree |  |
| 6.3 Minimum allowable width The minimum allowable width of a ramp is 800 mm. Conveyances except dedicated school buses and small aircraft | # 6.3 Minimum allowable width Ramps shall have a minimum trafficable width of 750 mm. Conveyances except dedicated school buses and small aircraft | 6.3 Minimum allowable width The minimum allowable width of a ramp is 800 mm. The trafficable width of the ramp width may be less than 800 mm due to kerbs on the ramp sides. Conveyances  Accessible rail car | Where an entrance to an existing conveyance is less than 800 mm the ramp width may be reduced. |
| 8.1 Boarding points and kerbs (1) Operators and providers may assume that passengers will board at a point that has a firm and level surface to which a boarding device can be deployed. (2) If a kerb is installed, it must be at least 150 mm higher than the road surface. Conveyances except dedicated school buses and small aircraft | # 8.1 Boarding points and kerbs (1) Operators and providers may assume that passengers will board at a point that has a firm and level surface to which a boarding device can be deployed. (2) If a kerb is installed, it must be at least 150 mm higher than the road surface. (3) The provider may nominate a single accessible location for boarding of a conveyance. Conveyances except dedicated school buses and small aircraft | Agree |  |
| 8.2 When boarding devices must be provided (1) A manual or power assisted boarding device must be available at any accessible entrance to a conveyance that has: (a) a vertical rise or gap exceeding 12 mm (AS3856.1 (1998) Clause 2.1.7 (f)); or (b) a horizontal gap exceeding 40 mm (AS3856.1 (1998) Clause 2.1.8 (g)). Conveyances except dedicated school buses and small aircraft | # 8.2 When boarding devices must be provided (1) A manual or power assisted boarding device must be available at any accessible entrance to a conveyance that has: (a) a vertical rise or gap exceeding 20 mm; or (b) a horizontal gap exceeding 65 mm Conveyances except dedicated school buses and small aircraft Premises Infrastructure: except airports that do not accept regular public transport services | 8.2 When boarding devices must be provided (1) A manual or power assisted boarding device must be available at any accessible entrance to a conveyance that has: (a) a vertical rise or gap exceeding 12 mm; or (b) a horizontal gap exceeding 40 mm Conveyances  Accessible rail car Infrastructure: Rail infrastructure – nominated boarding point | Realistic gap dimensions that allow the passage of conveyances on varying track conditions are unavoidable. When the gap dimensions of Part 8.2 are exceeded, then equivalent access, for example by deploying a boarding ramp, must be provided. Refer to '1X.2 Performance Criteria' for mobility aids for further guidance. |
| 8.5 Width and surface of boarding devices A boarding device must: (a) be a minimum of 800 mm wide; and (b) have a slip-resistant surface. Conveyances except dedicated school buses and small aircraft | # 8.5 Width and surface of boarding devices Delete Clause 8.5 | 8.5 Width and surface of boarding devices A boarding device must: (a) be a minimum of 800 mm wide; and (b) have a slip-resistant surface. Conveyances  Accessible rail car | Kerbs or guard rails may be counted withing the 800 mm minimum width. Boarding devices deployed at existing doors that have a width less than 800 mm, and which can not be modified, may be less than 800 mm at the point of abutment with the door opening. |
| 8.6 Maximum load to be supported by boarding device (1) A boarding device must be able to support a total passenger and mobility aid weight of up to 300 kg. (2) The device must be clearly labelled with the maximum load that it can carry, both on the boarding device and next to the accessible entrance on the outside of the conveyance. Conveyances except dedicated school buses and small aircraft | # 8.6 Maximum load to be supported by boarding device (1) A boarding device must be able to support the combined mass of the passenger, mobility aid and any assistant up to a total of 300 kg. (2) The device must be clearly labelled with the maximum load that it can carry. Conveyances except dedicated school buses and small aircraft | Agree |  |
| 8.7 Signals requesting use of boarding device (1) Any signal for requesting the deployment of a boarding device must be located in an allocated space. (2) If possible, a signal is to be placed according to the dimensions given in AS1428.2 (1992) Clause 11.4, Call buttons. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail | # 8.7 Signals requesting use of boarding device Delete | 8.7 Signals requesting use of boarding device Any signal for requesting the deployment of a boarding device must be located in or near an allocated space. Conveyances  Accessible rail car  Trams | Wherever possible the device should be placed in the allocated space in a position which is functional for a passenger using a wheelchair. |
| 8.8 Notification by passenger of need for boarding device (1) It must be possible for a passenger to notify the operator of a conveyance that he or she needs a boarding device to board or alight from a conveyance (2) If a request signal device is used, it may be located on the conveyance or at the boarding point according to the dimensions given in AS1428.2 (1992) Clause 11.4, Call buttons. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail Premises Infrastructure except airports that do not accept regular public transport services | # 8.8 Notification by passenger of need for boarding device (1) It must be possible for a passenger to notify the operator of a conveyance that he or she needs a boarding device to board or alight from a conveyance. (2) If a request signal device is used, it may be located on the conveyance or at the nominated accessible boarding point. (3) Any signal for requesting the deployment of a boarding device on a conveyance must be located in or near an allocated space or at an entrance. (4) Where possible, call buttons at entrances shall be located not less than 550 mm and not more then 1200 mm above the finished floor and not less than 500mm from an internal corner. (5) Operators may provide equivalent access by direct assistance. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail Premises Infrastructure except airports that do not accept regular public transport services | 8.8 Notification by passenger of need for boarding device (1) It must be possible for a passenger to notify the operator of a conveyance that he or she needs a boarding device to board or alight from a conveyance. (2) If a request signal device is used, it may be located on the conveyance or at the nominated accessible boarding point. (3) Any signal for requesting the deployment of a boarding device on a conveyance must be located in or near an allocated space. (4) Where possible, call buttons shall be located not less than 550 mm and not more then 1200 mm above the finished floor and not less than 500 mm from an internal corner. (5) Operators may provide equivalent access by direct assistance. Conveyances:  Trains  Light rail  Trams Premises: Rail premises Infrastructure: Rail infrastructure |  |
| 9.1 Minimum size for allocated space The minimum allocated space for a single wheelchair or similar mobility aid is 800 mm by 1300 mm (AS1428.2 (1992) Clause 6.1, Clear floor or ground space for a stationary wheelchair). Conveyances except dedicated school buses and small aircraft Premises Infrastructure except airports that do not accept regular public transport services | # 9.1 Minimum size for allocated space (1) The minimum allocated space (minimum clear floor or ground space, including knee space under objects as in Figure 1X-X) for a single mobility aid and occupant is 800 mm wide by 1300 mm long and 1500 mm high. (2) Transport operators may determine the location and orientation of allocated spaces in conveyances. (3) Mobility aid users may not be able to enter or exit an allocated space in a single manoeuvre, given the internal configurations of a conveyance. Conveyances except dedicated school buses and small aircraft Premises Infrastructure except airports that do not accept regular public transport services | 9.1 Minimum size for allocated space (1) The minimum allocated space (minimum clear floor or ground space, including knee space under objects as in Figure 1X-X) for a single mobility aid and occupant is 800 mm wide by 1300 mm long and 1500 mm high. (2) Transport operators may determine the location and orientation of allocated spaces in conveyances. (3) Mobility aid users may not be able to enter or exit an allocated space in a single manoeuvre, given the internal configurations of a conveyance. Conveyances:  Accessible rail car Premises: Rail premises Infrastructure: Rail infrastructure |  |
| 9.6 Number of allocated spaces to be provided — train cars, etc (1) At least 2 allocated spaces must be provided for each rail, tram or light rail car. (2) Up to 8 allocated spaces may be consolidated in one car of a set. (3) If different classes of travel are offered, allocated spaces must be provided in each class. Conveyances  Rail cars  Tram cars  Light rail cars | # 9.6 Number of allocated spaces to be provided — rail, tram, light rail cars Number for unbooked services (1) (a) The minimum number of allocated spaces required on an unbooked service is equal to the number of rail cars. (b) An articulated light rail vehicle or tram with a capacity of 100 or more (including standing passengers) must contain a minimum of 2 allocated spaces. Number for booked services (2) The minimum number of allocated spaces required on booked services is 2 spaces for 1 to 7 passenger seating cars, 3 spaces for 8 to 11 passenger seating cars, and 4 spaces for 12 or more passenger seating cars in a consist. (3) Up to 8 allocated spaces may be consolidated in one car of a set. (4) If seating and sleeping accommodation is provided, allocated spaces and accessible sleeping berths must be provided. Conveyances  Rail cars  Tram cars  Light rail cars | 9.6 Number of allocated spaces to be provided — rail, tram, light rail cars Number for unbooked services (1) The total number of allocated spaces provided on a rail, tram or light rail set that has more than 32 fixed seats must equal at least 2 allocated spaces per car. . (2) At least one allocated space must be provided for each rail car, and for an articulated light rail vehicle or tram that has less than 33 fixed seats per car. - An allocated space is additional to the fixed seating capacity. - Up to 8 allocated spaces may be located together in one car of a set. Number for booked services (1)For discussion. (2) Allocated spaces may be located together in one car of a set. (3) If seating and sleeping accommodation is provided, allocated spaces and accessible sleeping berths must be provided. Conveyances:  Accessible rail cars  Light rail cars | Folding seats can be placed in the allocated spaces provided they do not compromise the dimensions of the allocated space. |
| 9.7 Consolidation of allocated spaces If possible, allocated spaces are to be consolidated to accommodate larger mobility aids. Conveyances  Buses except dedicated school buses  Ferries  Trains  Trams  Light rail | # 9.7 Consolidation of allocated spaces If possible, allocated spaces are to be provided in close proximity to each other. Conveyances  Buses except dedicated school buses  Ferries  Trains  Trams  Light rail | 9.7 Consolidation of allocated spaces If possible, allocated spaces are to be provided adjacent to or near each other. Conveyances:  Trains  Light rail  Trams |  |
| 9.10 International symbol of accessibility to be displayed (1) The floor area of an allocated space must: (a) display the international symbol of accessibility; and (b) be outlined in a flush contrasting strip 25 mm wide. (2) The colours prescribed in AS1428.1 (2001) Clause 14.2 (c) are not mandatory. Conveyances  Buses  Trains  Trams  Light rail | # 9.10 International symbol of accessibility to be displayed (1) An allocated space must: (a) display the international symbol of accessibility (AS1428.2: 2001 Fig 32); and (b) if displayed on the floor, be outlined in a flush contrasting strip 25 mm wide. (2) The preferred colours of white figure on ultramarine blue (AS2700 B21) are not mandatory. Conveyances  Buses  Trains except booked services  Trams  Light rail | 9.10 International symbol of accessibility to be displayed (1) An allocated space must: (a) Clearly display the international symbol of accessibility; and (b) if displayed on the floor, be outlined in a flush contrasting strip 25 mm wide. (2) While preferred, colours of white figure on ultramarine blue are not mandatory. Conveyances:  Trains except booked services  Light rail  Trams | During times of peak crowding the symbol must remain visible from at least close proximity. |
| 10.1 Compliance with Australian Standard (1) Ground and floor surfaces must comply with AS1428.2 (1992) Clause 9, Ground and floor surfaces. (2) AS1428.1 Supplement 1 (1993) Clause C12 provides criteria for the selection of floor surfaces. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail Premises Infrastructure except airports that do not accept regular public transport services | # 10.1 Compliance with Australian Standard (1) Ground and floor surfaces must comply with AS1428.2 (1992) Clause 9, Ground and floor surfaces. (2) AS1428.1 Supplement 1 (1990) Clause C13 provides criteria for the selection of floor surfaces. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail | 10.1 Compliance with Australian Standard (1) Ground and floor surfaces must comply with AS1428.2 (1992) Clause 9, Ground and floor surfaces. (2) AS1428.1 Supplement 1 (1990) Clause C12 provides criteria for the selection of floor surfaces. Conveyances:  Trains  Light rail  Trams |  |
| No Clause | # 10.1X Compliance with Australian Standard (1) All continuous accessible paths of travel shall have a slip-resistant surface. (2) A continuous accessible path of travel shall have a texture that is traversable by a mobility aid. (3) Grates on an accessible path of travel shall have spaces not more than 13 mm wide and not more than 150 mm long. If gratings have elongated openings, they shall be placed so that the long dimension is transverse to the dominant direction of travel. Premises Infrastructure except airports that do not accept regular public transport services | Agree | Normal safety requirements are applicable to all surfaces. |
| 11.1 Compliance with Australian Standard — premises and infrastructure A handrail must comply with AS1428.2 (1992) Clause 10.1, Handrails. Premises Infrastructure except airports that do not accept regular public transport services | # 11.1 Compliance with Australian Standard — premises and infrastructure (1) A handrail must comply with AS1428.2 (1992) Clause 10.1, Handrails. Premises Infrastructure except airports that do not accept regular public transport services | 11.1 Compliance with Australian Standard — premises and infrastructure (1) A handrail must comply with AS1428.2 (1992) Clause 10.1, Handrails, except Clause 10.1.2(c). (2) Where there is a background wall, handrails shall have a distinct colour contrast with the background wall. Premises: Rail premises Infrastructure: Rail infrastructure |  |
| 11.2 Handrails to be provided on access paths (1) Handrails must be placed along an access path wherever passengers are likely to require additional support or passive guidance. (2) A handrail must not infringe an area on a roadside boarding point that may be needed to deploy a boarding device. Premises Infrastructure except airports that do not accept regular public transport services | # 11.2 Handrails to be provided on access paths (1) Handrails must be placed along an access path wherever passengers are likely to require additional support or passive guidance. (2) A handrail must not infringe an area on a roadside boarding point that may be needed to deploy a boarding device. Premises Except buildings at railway premises Infrastructure except airports that do not accept regular public transport services Except railway infrastructure | 11.2 Handrails to be provided on access paths If handrails are placed along an access path they must provide passengers additional support and passive guidance. Premises: Rail premises Infrastructure: Rail infrastructure | If handrails are to be used to direct pedestrian flow, they should also offer support as a function. |
| 11.3 Handrails on steps (1) A handrail on steps need not extend beyond the top or bottom of the steps. (2) A domed button may be placed 150 mm from any break or end of a handrail instead of an extension at a rail end (AS1428.2 (1992) Figure 5). Conveyances except dedicated school buses and small aircraft | # 11.3 Handrails on steps (1) A handrail on steps need not extend beyond the top or bottom of the steps. (2) A domed button may be placed 150 mm +/- 10 mm from any break or end of a handrail instead of an extension at a rail end. Conveyances except dedicated school buses and small aircraft | Agree |  |
| 11.4 Handrails above access paths If installed, a handrail above an access path must comply with AS1428.1 (2001) Clause 6.1 (c), Handrails and Figure 9. Conveyances except dedicated school buses and small aircraft | # 11.4 Handrails above access paths (1) If installed, a handrail above an access path must comply with AS1428.1 (2001) Clause 6.1 (c), Handrails and Figure 9, other than where local controls or equipment are in place. (2) Grabrails may be used where handrails cannot be installed. Conveyances except dedicated school buses and small aircraft | 11.4 Handrails above access paths (1) If installed, a handrail above an access path must comply with AS1428.1 (2001) Clause 6.1 (c), Handrails and Figure 9, other than where unavoidable design constraints exist or control equipment is nearby. (2) Grabrails may be used where handrails cannot be installed. Conveyances:  Trains  Light rail |  |
| 11.5 Compliance with Australian Standard A grabrail must comply with AS1428.2 (1992) Clause 10.2, Grabrails. Conveyances except dedicated school buses and small aircraft Premises Infrastructure except airports that do not accept regular public transport services | # 11.5 Grabrail compliance requirements (1) Grab rails shall be of rigid construction with circular or oval cross section of diameter 30 to 40 mm, designed for a 1100 N load in all directions, slip resistant design where required in wet areas, having hand clearances of 50 to 60 mm, and having an unobstructed hand movement on the top 270 degrees, and minimum corner and edge radii of 5 mm. (2) Horizontal grab rails require a clear area 600 mm above the rail. Where significant design constraints limit this clearance it should be maximized as much as is technically possible. Conveyances except dedicated school buses and small aircraft Premises Infrastructure except airports that do not accept regular public transport services | Agree |  |
| 11.6 Grabrail to be provided where fares are to be paid A grabrail or handrail must be provided at fixed locations where passengers are required to pay fares. Conveyances except dedicated school buses and small aircraft Premises Infrastructure except airports that do not accept regular public transport services | # 11.6 Grabrail to be provided where fares are to be paid Delete | Agree. | Delete clause. |
| 11.7 Grabrails to be provided in allocated spaces Grabrails that comply with AS1428.2 (1992) Clause 10.2, Grabrails, must be provided in all allocated spaces. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail | # 11.7 Grabrails to be provided in allocated spaces Grabrails must be provided in all allocated spaces except where anchoring of mobility aids is required as a condition of travel. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail | Agree |  |
| 12.1 Doors on access paths (1) Any doors along an access path must not present a barrier to independent passenger travel. (2) Direct assistance may be provided through security check points. Conveyances except dedicated school buses and small aircraft Premises Infrastructure except airports that do not accept regular public transport services | # 12.1 Doors on access paths (1) Any doors along an access path must not present a barrier to independent passenger travel, or equivalent access must be provided by the operator. (2) Direct assistance may be provided through security check points. (3) Doors on conveyances may be controlled or opened by the operator. Conveyances except dedicated school buses and small aircraft Premises Infrastructure except airports that do not accept regular public transport services | Agree |  |
| 12.2 Compliance with Australian Standard — premises and infrastructure Doorways and doors must comply with AS1428.2 (1992) Clause 11 (except Clause 11.5.2). Premises Infrastructure except airports that do not accept regular public transport services | # 12.2 Compliance with Australian Standard — premises and infrastructure Doorways and doors (except at toilets) must comply with AS1428.2 (1992) Clause 11 (except Clause 11.5.2). Premises Infrastructure except airports that do not accept regular public transport services | 12.2 Compliance with Australian Standard — premises and infrastructure (1) Doorways and doors must comply with AS1428.2 (1992) Clause 11 (except Clause 11.5.2) in all new premises and infrastructure. (2) In existing premises and infrastructure where legitimate constraints prevent compliance, the relevant requirements of the Building Code of Australia must be met. Premises: Rail premises Infrastructure: Rail infrastructure | Doorways are critical to the use of premises and infrastructure. Where the Transport Standard can not be met legitimately, the appropriate state Building Act applies. |
| 12.3 Weight activated doors and sensors (1) A pressure pad of a weight activated door must be sensitive enough to detect a 15 kg service animal. (2) Any other type of sensor on an access path must be able to detect movement between ground level and 500 mm above the access path. Premises Infrastructure except airports that do not accept regular public transport services | # 12.3 Weight activated doors and sensors (1) A pressure pad of a weight activated door must be sensitive enough to detect a 15 kg assistance dog. (2) Any other type of sensor on an access path must be able to detect movement between ground level and 500 mm above the access path. Premises Infrastructure except airports that do not accept regular public transport services | # 12.3 Weight activated doors and sensors (1) A pressure pad of a weight activated door must be sensitive enough to detect a 15 kg assistance animal. (2) Any other type of sensor on an access path must be able to detect movement between ground level and 500 mm above the access path. Premises Infrastructure except airports that do not accept regular public transport services |  |
| 12.4 Clear opening of doorways Doorways must comply with AS1428.2 (1992) Clause 11.5.1, Clear opening of doorways. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail | # 12.4 Clear opening of doorways The minimum clear opening of a doorway shall be 850 mm except at a toilet door which may be 750 mm. Conveyances  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail | 12.4 Clear opening of doorways (1) The minimum clear opening of a doorway shall be 850 mm. (2) In existing conveyances and in certain new situations where in either case technical constraints can be demonstrated, a toilet door may be 760 mm. Conveyances:  Trains  Light rail  Trams | Use of colour contrasting trims is recommended. Their use will assist passengers with vision impairments locate doorways. |
| 13.1 Compliance with Australian Standard — premises and infrastructure Lift facilities must comply with AS1735.12 (1999). Premises Infrastructure except airports that do not accept regular public transport services | # 13.1 Compliance with Australian Standard — premises and infrastructure Subject to the following clause (1), the lift facilities must comply with AS1735.12 (1999): The minimum lift car internal dimensions shall be 1100 mm wide by 1400 mm deep between the inside of the closed car doors to the inside back wall of the car. Premises Infrastructure except airports that do not accept regular public transport services | Agree | For a lift of 1100 mm x 1400 mm dimension, where possible a two door 'travel straight through' design is safer as there is no need to reverse out of a lift in a wheelchair. |
| 14.1 Stairs not to be sole means of access Stairs must not be the sole means of access. Conveyances Premises Infrastructure except dedicated school buses and small aircraft except airports that do not accept regular public transport services | # 14.1 Stairs not to be sole means of access Stairs must not be the sole means of access to facilities designated for passengers with mobility impairment. Conveyances Premises Infrastructure except airports that do not accept regular public transport services | Agree |  |
| 14.3 Compliance with Australian Standards — conveyances (1) If stairs are provided on a conveyance mentioned below, they must comply with: (a) The notes to AS1428.1 (2001) Clause 9.1, Stair geometry; and (b) AS1428.2 (1992) Clause 13.2, Configuration of steps, Clause 13.3, Warning strip at nosing of steps and Figures 8 and 9. However, the minimum access path width on stairs in the conveyance must be 850 mm. Conveyances  Ferries  Trains  Trams  Light rail | # 14.3 Stairs on conveyances (1) Stairs must not encroach into circulation spaces and must have opaque enclosed risers. (2) Colour contrasted warning nosing strips 50 to 75 mm width on top and 25 to 50 mm width on the vertical edge must be provided on the edge of stair treads. Stairs at the entrance (3) Stairs at the entrance to a conveyance must comply with AS 1657 (1992) Figure 4.3 measured at the centre line of the stairs. Conveyances  Ferries  Trains  Trams  Light rail | 14.3 Stairs on conveyances (1) Stairs must not encroach into circulation spaces and must have opaque enclosed risers. (2) Colour contrasted warning nosing strips 50 to 75 mm width on top and 25 to 50 mm width on the vertical edge must be provided on the edge of stair treads. Stairs at the entrance Stairs at the entrance to a conveyance must comply with AS 1657 (1992) Figure 4.3 measured at the centre line of the stairs. Conveyances:  Trains  Light rail  Trams |  |
| 15.1 Unisex accessible toilet — premises and infrastructure If toilets are provided, there must be at least one unisex accessible toilet without airlock that complies with AS1428.1 (2001) Clause 10, Sanitary facilities. Premises Infrastructure except airports that do not accept regular public transport services | # 15.1 Unisex accessible toilet — premises and infrastructure If toilets are provided, there must be at least one unisex accessible toilet or gender specific accessible toilets without airlock that complies with AS1428.1 (2001) Clause 10, Sanitary facilities (except clause 10.2.10(e)) Premises Infrastructure except airports that do not accept regular public transport services | 15.1 Unisex accessible toilet — premises and infrastructure (a) If toilets are provided, there must be at least one unisex accessible toilet without airlock that complies with AS1428.1 (2001) Clause 10, Sanitary facilities. The pressure to swing or slide the door open must not exceed 20 N. (b) Existing premises and infrastructure may retain accessible gender specific toilets if technical constraints prevent construction of unisex units. Premises: Rail premises Infrastructure: Rail infrastructure | As people with disabilities may have young children, baby change tables and the like are accepted as part of a unisex accessible toilet provided they do not compromise circulation space. |
| 15.2 Location of accessible toilets Accessible toilets must be in the same location as other toilets. Premises Infrastructure except airports that do not accept regular public transport services | # 15.2 Location of accessible toilets If possible, accessible toilets must be in the same location as other toilets or be in a location of equivalent convenience to the access path. Premises Infrastructure except airports that do not accept regular public transport services | 15.2 Location of accessible toilets (a) Accessible toilets must be in the same location as other toilets in all new premises or infrastructure. (b) On existing premises and infrastructure accessible toilets must be in the same location as other toilets or be in a location of equivalent convenience to the access path should technical or structural constraints not permit this. Premises: Rail premises Infrastructure: Rail infrastructure | Where technically and structurally possible, accessible toilets must be placed in the same location as other toilets. If located away from other toilets a location of equivalent convenience to the access path must be selected. |
| 15.3 Unisex accessible toilet — ferries and accessible rail cars If toilets are provided, there must be at least one unisex accessible toilet without airlock available to passengers using wheelchairs or mobility aids. Conveyances  Ferries  Accessible rail cars | # 15.3 Unisex accessible toilet — ferries and trains If toilets are provided, there must be at least one unisex accessible toilet without airlock available to passengers using compliant mobility aids. Conveyances  Ferries  Trains | 15.3 Unisex accessible toilet — trains If toilets are provided, there must be at least one unisex accessible toilet without airlock available to passengers with compliant mobility aids. Conveyances:  Trains | Where possible unisex accessible toilets must be accessible from the allocated spaces or equivalent access provided. |
| 15.4 Requirements for accessible toilets — ferries and accessible rail cars (1) An accessible toilet must: (a) comply with the requirements set out in this section; and (b) allow passengers in wheelchairs or mobility aids to enter, position their aids and exit. (2) The minimum dimension from the centre line of the pan to the near-side wall must be 450 mm (AS1428.1 (2001) Figure 22). (3) The minimum dimension from the centre line of the pan to the far-side wall must be 1150 mm (AS1428.1 (2001) Figure 22). (4) The minimum dimension from the back wall to the front edge of the pan must be 800 mm (AS1428.1 (2001) Figure 22). (5) The toilet seat must be between 460 mm and 480 mm above the floor (AS1428.1 (2001) Figure 18). (6) Hand washing facilities must be provided either inside or outside the toilet (AS1428.1 (2001) Clause 10.2.1 (b), Water closets). Conveyances  Ferries  Accessible rail cars | # 15.4 Requirements for accessible toilets — ferries and accessible rail cars (1) An accessible toilet must allow passengers in mobility aids to enter and exit. (2) There shall be sufficient space inside the toilet cubicle for a mobility aid to be positioned so that a disabled person can move from the mobility aid on to the toilet seat from the front or the side of the toilet. (3) All unisex toilet amenities must be accessible. (4) Hand washing facilities must be provided inside the toilet. Conveyances  Ferries  Accessible rail cars | 15.4 Requirements for accessible toilets — accessible rail cars New Accessible Rail Cars (1) An accessible toilet must allow passengers in mobility aids to enter and exit. (2) There shall be sufficient space inside the toilet cubicle for a mobility aid to be positioned so that a disabled person can move from their mobility aid on to the toilet seat from the front or the side of the toilet. (3) All unisex toilet amenities must be accessible. (4) Hand washing facilities must be provided inside the toilet. Conveyances:  Accessible rail cars | Conveyances may not be able to be brought to full compliance. Where structural and design constraints preclude full compliance, dimensions must be maximised to the fullest extent possible. Information should be available on web pages and the like giving dimensions of non-compliant units to passengers prior to departure. Where possible, an accessible toilet should comply with the following: (1) (a) comply with the requirements set out in this section; and (b) allow passengers in wheelchairs or mobility aids to enter, position their aids and exit. (2) The minimum dimension from the centre line of the pan to the near-side wall must be 450–350 mm. (3) The minimum dimension from the centre line of the pan to the far-side wall must be 1150–1100 mm. (4) The minimum dimension from the back wall to the front edge of the pan must be 800 mm. (5) The toilet seat must be between 460 mm and 480 mm above the floor. (6) Hand washing facilities must be provided inside the toilet compartment. In all instances, the largest dimensions possible should be achieved. |
| 16.2 Compliance with AS2899.1 (1986) The illustrations and symbols prescribed in AS2899.1 (1986) must be used if applicable. Conveyances Premises Infrastructure | # 16.2 Compliance with AS2899.1 (1986) The illustrations and symbols prescribed in AS2899.1 (1986), the Manual of Uniform Traffic Control Devices Part 1 AS1742.1-2003, and ISO 7001:1990 Public Information Symbols must be used if applicable. Conveyances Premises Infrastructure | Agree |  |
| 16.3 Accessibility symbols to incorporate directional arrows The symbol for accessibility must incorporate directional arrows and words or, if possible, pictograms, to show passengers the way to accessible facilities such as toilets. Conveyances Premises Infrastructure | # 16.3 Accessibility symbols to incorporate directional arrows Directional signage must incorporate accessibility symbols, pictograms (and if needed, words) and directional arrows to show passengers the way to accessible facilities, Conveyances Premises Infrastructure | Agree |  |
| 16.5 Accessibility symbol to be visible on accessible doors The international symbol of accessibility must be clearly visible both inside and outside accessible doors on these conveyances. Conveyances  Ferries  Trains  Trams  Light rail | # 16.5 Accessibility symbol to be visible on accessible doors The international symbol of accessibility must be clearly visible on the outside of accessible doors. Conveyances  Ferries  Trams  Light rail | Agree |  |
| 17.4 Destination signs to be visible from boarding point (1) Destination signs must be visible from, or available at, boarding points. (2) They may be displayed on the conveyance or within the premises or infrastructure. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail Premises Infrastructure | # 17.4 Destination signs to be visible from boarding point (1) Destination signs must be visible from, or available at, the nominated accessible boarding point. (2) They may be displayed on the conveyance or within the premises or infrastructure. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail Premises Infrastructure | Agree | Destination signs may be located at other boarding points for the benefit of the travelling public. |
| 17.5 Electronic notices (1) Presentations of words or numbers on electronic notices must be visible for at least 10 seconds, unless the electronic notice is for the purpose of ticket validation. (2) If the electronic notice is for this purpose, the words or numbers on the notice must cease to be visible before the end of 10 seconds if the ticket validation device is used by another person within that time. Premises Infrastructure | # 17.5 Electronic notices (1) Presentations of words or numbers on electronic notices must be visible for the maximum time period operationally possible, unless the electronic notice is for the purpose of ticket validation. If the electronic notice is for this purpose, the words or numbers on the notice must cease to be visible before the end of 10 seconds if the ticket validation device is used by another person within that time. Premises Infrastructure | Agree |  |
| 17.6 Raised lettering or symbols or use of Braille (1) If a sign incorporates raised lettering or symbols, they must be at least 0.8 mm above the surface of the sign. (2) If an operator or provider supplements a notice with Braille characters, they must be placed to the left of the raised characters. Conveyances Premises Infrastructure | # 17.6 Raised lettering or symbols or use of Braille (1) If a sign incorporates raised lettering or symbols, they must be at least 0.6 mm above the surface of the sign. (2) If an operator or provider supplements a notice with Braille characters, they must be left justified and placed adjacent to the text or raised characters. Conveyances Premises Infrastructure | 17.6 Raised lettering or symbols or use of Braille (a) (1) If a sign incorporates raised lettering or symbols, they must be raised 1.0 - 1.5 mm above the surface of the sign. (2) If an operator or provider supplements a notice with Braille characters, they must be left justified and placed adjacent to the text or raised characters. Premises: Rail premises Infrastructure: Rail infrastructure (b) (1) If a sign incorporates raised lettering or symbols, they must be at least 0.6 mm above the surface of the sign. (2) If an operator or provider supplements a notice with Braille characters, they must be left justified and placed adjacent to the text or raised characters. Conveyances Trains |  |
| 18.1 Location Tactile ground surface indicators must be installed on an access path to indicate stairways, ramps, changes of direction, overhead obstructions below a height of 2000 mm, and hazards within a circulation space or adjacent to a path of travel (AS1428.2 (1992) Clause 18.1, Tactile ground surface indicators). Premises Infrastructure | # 18.1 Location (1) Warning Tactile Ground Surface Indicators (TGSIs) must be installed on an access path to indicate stairways, escalators and ramps. Warning TGSIs, physical barriers or alternative wayfinding aids must be used to indicate obstructions below a height of 2000 mm. (2) Having established the circumstances that assist safe way-finding, design layouts should not over-use or over-prescribe the installation of tactile ground surface indicators, but rather should make full use of the range of environmental guidance features available so as to minimise inconvenience to other members of the community. Premises Infrastructure | Agree |  |
| 18.2 Style and dimensions (1) The style and dimensions of tactile ground surface indicators must comply with AS1428.4 (1992). (2) The stated dimensions may be reduced where a conveyance design does not provide the necessary area. Premises Infrastructure | # 18.2 Style and dimensions (1) The style and dimensions of warning tactile ground surface indicators must comply with the Figure 18AX. Where the tolerance between the truncated cones cannot be met, it shall be minimised. Premises Infrastructure | Agree | Where the tolerance between the truncated cones cannot be met, such as when TGSI tiles must be cut to allow their close abutment on a curved platform or kerb-side, it shall be minimised. |
| 18.4 Instalment at railway stations Colour contrasted tactile indicators must be installed at the edges of railway platforms as prescribed by AS1428.4 (1992) Clause 6.7. Infrastructure  Railway platforms | # 18.4 Instalment at railway stations Where alternative physical barriers, such as a fence or wall are not installed, colour contrasting warning tactile ground surface indicators must be installed as shown in Figure 18BX. Infrastructure  Railway platforms | Agree | The HREOC agreement regarding TGSIs and minimum path of travel width should be consulted in the interpretation of this Clause (See Part 2.4X). |
| 19.1 Emergency warning systems (1) If installed, emergency warning systems must comply with AS1428.2 (1992) Clause 18.2.1, Emergency warning systems, Clause 18.2.2, Audible alarms, and Clause 18.2.3, Visual alarms. (2) Provision must be made for people with vision impairment to locate the exit path in the event of an emergency. Conveyances Premises Infrastructure | # 19.1 Emergency warning systems (1) If installed, emergency warning systems must comply with AS1428.2 (1992) Clause 18.2.1, Emergency warning systems, Clause 18.2.2, Audible alarms, and Clause 18.2.3, Visual alarms. (2) Provision must be made for people with a disability to locate the exit path in the event of an emergency. Conveyances Premises Infrastructure | Agree | Deaf and vision impaired people must receive warnings equal or equivalent to others. Audio/visual warnings should be considered, as should appropriate direct assistance in crowded, confused situations. |
| 20.1 Illumination levels — premises and infrastructure Any lighting provided must comply with minimum levels of maintenance illumination for various situations shown in the notes to AS1428.2 (1992) Clause 19.1, Illumination levels. Premises Infrastructure | # 20.1 Illumination levels — premises and infrastructure (1) Any lighting provided must comply with minimum levels of maintenance illumination for various situations shown in the notes to AS1428.2 (1992) Clause 19.1, Illumination levels, except. (2) Any lighting provided on railway stations must comply with the following: Interpretation The interpretation from using both of these Australian Standards is that the ‘core zone’ of the station where tickets are sold, information is provided, vending machines, telephones and out of weather seating are placed will have 150 lux minimum maintenance illumination. Within this area spot vertical lighting of between 200-300 lux will be provided above ticket counters, timetables, information posters, vending machines and telephones. (3) Any lighting provided on pedestrian level crossings that provide direct access to a boarding point must comply with AS 1742.7 Pedestrian Level Crossings. Premises Infrastructure | Agree | For detail refer to the attached table. |
| 21.1 Compliance with Australian Standard — premises and infrastructure Controls must comply with AS1428.1 (2001) Clause 11. Premises Infrastructure except airports that do not accept regular public transport services | # 21.1 Compliance with Australian Standard — premises and infrastructure Controls must comply with AS1428.1 (2001) Clause 11, except Clause 11.1.1 (c). Premises Infrastructure except airports that do not accept regular public transport services | 21.1 Compliance with Australian Standard — premises and infrastructure Controls must comply with AS1428.1 (2001) Clause 11. Doors must not exceed 20 N of force to open. Premises: Rail premises Infrastructure: Rail infrastructure |  |
| 21.2 Passenger-operated devices for opening and closing doors Passenger-operated devices for opening and closing manual and power-assisted doors on conveyances must comply with AS1428.2 (1992) Clause 23.2, Operation, and Clause 23.3, Door handles and hardware. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail | # 21.2 Passenger-operated devices for opening and closing doors Passenger-operated devices for opening and closing manual and power-assisted doors on conveyances, other than emergency door controls, must comply with AS1428.2 (1992) Clause 23.2, Operation, and Clause 23.3, Door handles and hardware. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail | 21.2 Passenger-operated devices for opening and closing doors Passenger-operated devices for opening and closing manual and power-assisted doors on conveyances, other than emergency door controls, must be operable by people who have disabilities. Controls and operating mechanisms shall be operable with one hand and shall not require tight grasping, pinching, or twisting of the wrist. Passenger-operated devices for opening and closing manual and power-assisted doors on conveyances, other than emergency door controls, must comply with AS1428.2-1992, Clause 23.3, Door handles and hardware. Where emergency doors do not meet the above requirements , direct assistance must be provided. Conveyances:  Trains  Light rail  Trams |  |
| 21.3 Location of passenger-operated controls for opening and locking doors Passenger-operated opening and locking controls for doors on conveyances must be located according to AS1428.1 (2001) Clause 11.1.2, Location. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail | # 21.3 Location of passenger-operated controls for opening and locking doors Passenger-operated opening and locking controls for doors on conveyances must be located according to AS1428.1 (2001) Clause 11.1.2, Location except that door controls may be placed closer than 500 mm from an internal corner so long as they are accessible to a person using a compliant mobility aid. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail | 21.3 Location of passenger-operated controls for opening and locking doors Passenger-operated opening and locking controls for doors must be located according to AS1428.1 (2001) Clause 11.1.2, Location except that door controls may be placed closer than 500 mm from an internal corner, provided that there is sufficient circulation space to be accessible to a person using a mobility aid in accordance with Part 1X. Conveyances:  Trains  Light rail  Trams | For controls to be workable by people who have disabilities, it is essential that they have sufficient circulation space around them and that they be located at realistic heights above the floor or other surface. 'External controls' do however suffer variation due to platform, kerb and wharf heights not being uniform. For external controls, every effort should be made to comply on a majority of platforms. |
| 22.1 Tables, benches, counters, etc Tables, benches, counters and similar fixtures must comply with AS1428.2 (1992) Clauses 24.1, 24.1.1, 24.1.2, 24.1.3, 24.1.4 and 24.1.5. Premises except airport terminals Infrastructure | # 22.1 Tables, benches, counters, etc (1) At least one ticket counter must be suitable for passengers using mobility aids. (2) Any tables provided for designated wheelchair locations must be suitable for passengers using mobility aids as per Figure 1X-X. Conveyances  Booked services Premises except airport terminals Infrastructure | Agree |  |
| 22.5 Accessible sleeping berths — trains (1) If a train has sleeping berths, a minimum of 2 accessible berths must be provided in each set of up to 4 sleeping cars, or one accessible sleeping berth must be provided for every 32 bunks. (2) If different classes of travel are offered, accessible sleeping berths must be provided in each class. Conveyances  Trains | # 22.5 Accessible sleeping berths — trains (1) If a train has sleeping berths, a minimum of 1 accessible sleeping berth must be provided. (2) If a train has more than 2 rail cars with sleeping berths, a minimum total of 2 accessible sleeping berths must be provided. Conveyances  Trains | 22.5 Accessible sleeping berths — trains New Trains (1) If a train has sleeping berths, a minimum of 2 accessible berths must be provided in each set of up to 4 sleeping cars, or one accessible sleeping berth must be provided for every 32 bunks. (2) If different service levels for sleeping berths are offered, accessible berths must be provided in each class. Existing Trains (1) If a train has sleeping berths, a minimum of 1 accessible berth must be provided. (2) If a train has more than 2 rail cars with sleeping berths, a minimum total of 2 accessible berths must be provided. Conveyances  Trains – booked services | Accessible berths need to take 'people with mobility impairments' rather than just wheelchair users. While minimum numbers proposed for existing trains - those in service and old cars refurbished - are less than those for new, they should be exceeded if possible, and compliance with numbers for new trains met where possible. |
| 23.1 Seats Seats must comply with AS1428.2 (1992) Clause 27.2, Seating in pedestrian areas. Premises Infrastructure except airports that do not accept regular public transport services | # 23.1 Seats The design and installation of seating shall be as Figure 23X and recommended dimensions as follows: (1) The front of the seat shall have a clear space between any legs at ground level to within 150 mm of the front edge of the seat, and to within 100 mm of the seat height to allow for rearward adjustment of feet when rising. (2) Where armrests are provided, the top surface of the armrests shall be at a height of 260 ± 40 mm above the seat. (3) The front edge of the seat shall have a minimum radius of 30 mm. (4) No edge or projection shall have a radius of less than 5 mm unless protected from contact with the user. (5) The seat shall drain free of water. Premises Infrastructure except airports that do not accept regular public transport services | Agree |  |
| 24.1 Gateways and checkouts (1) Gateways and checkouts, such as ticket barriers, must comply with AS1428.2 (1992) Clause 28, Gateways and checkouts. (2) However, the width of an accessible gateway or checkout mentioned in AS1428.2 (1992) Clause 28.2 must be at least 850 mm. Premises Infrastructure except airports that do not accept regular public transport services | # 24.1 Gateways and checkouts Gateways and checkouts, such as ticket barriers, must comply with the following: (1) The international symbol for access (see Clause 16.1) shall be used to designate where access is available. (2) Where gateways and checkouts are installed, at least one barrier opening shall be not less than 850 mm wide. (3) The ticket, coin feed points shall be at a height of 800 mm to 1200 mm from the finished floor. Any controls needed to operate these machines shall have tactile applications for vision-impaired users. (4) A non-contact card reader shall be at a height range of 700 mm to 1200 mm from the finished floor. (5) Any barrier shall be not less than 1200 mm past the ticket or coin feed point in the direction of travel (see Figure 24X). Premises Infrastructure except airports that do not accept regular public transport services | 24.1 Gateways and checkouts Gateways and checkouts, such as ticket barriers, must comply with the following: (1) The international symbol for access (see Clause 16.1) shall be used to designate where access is available. (2) Where gateways and checkouts are installed, at least one barrier opening shall be not less than 850 mm wide. (3) The ticket, coin feed points shall be at a height of 800 mm to 1200 mm from the finished floor. Any controls needed to operate these machines shall have tactile applications for vision-impaired users. (4) A non-contact card reader shall be at a height range of 700 mm to 1200 mm from the finished floor. (5) Any barrier shall be not less than 1200 mm past the ticket or coin feed point in the direction of travel (see Figure 24X). Premises: Rail premises Infrastructure: Rail infrastructure | Equivalent access must be provided if for genuine technical reasons any of the specifications of '24.1 Gateways and Checkouts' can not be met. Controls requiring fine motor control to operate should be located in the range of 900 – 1100 mm above floor level. |
| 25.3 Vending machines Vending machines must comply with AS1428.2 (1992) Clause 29.1, Height, Clause 29.2, Controls, and Clause 29.3, Illumination. Conveyances Premises Infrastructure | # 25.3 Ticket vending machines Ticket vending machines must comply with the following: (1) The height of the operative components shall be between 500 mm and 1200 mm above the trafficable surface. (2) The required operating force for any control shall not exceed 20 N. (3) Illumination shall be provided in accordance with Part 20 Lighting. Conveyances Premises Infrastructure | Agree |  |
| 25.4 Circulation space in front of vending machine The circulation space in front of any vending machine must allow for a 180 degree turn as in AS1428.2 (1992) Clause 6.2, Circulation space for 180 degree wheelchair turn. Premises Infrastructure except airports that do not accept regular public transport services | # 25.4 Circulation space in front of a ticket vending machine The circulation space in front of any ticket vending machine must allow for a 180 degree turn as in Clause 3.1. Premises Infrastructure except airports that do not accept regular public transport services | Agree |  |
| 26.2 Public address systems — conveyances If a public address system is installed: (a) people who are deaf or have a hearing impairment must be able to receive a message equivalent to the message received by people without a hearing impairment; and (b) it must comply with AS1428.2 (1992) Clause 21.1, Hearing augmentation. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail | # 26.2 Public address systems — conveyances If a public address system is installed, people who are deaf or have a hearing impairment must be able to receive a message equivalent to the message received by people without a hearing impairment. Conveyances  Buses  Coaches  Ferries  Trains  Trams  Light rail | Agree | Visual information can provide a reasonably equivalent summary of audio information but is unable to exactly replicate every audio message. Other forms of equivalent access may be used to supplement visual service related messages. Essential information is: announcement of next stop, safety related announcements and any unscheduled change of service. |
| 27.1 Access to information about transport services General information about transport services must be accessible to all passengers. Conveyances Premises Infrastructure | # 27.1 Access to information about transport services (1) Information necessary to use the transport service must be accessible to all passengers. (2) Provision of accessible information assumes that all passengers have a minimum level of literacy, English language skills and an ability to communicate their destination if required. Conveyances Premises Infrastructure | Agree | Where possible, pictograms and international symbols should be incorporated in signs. |
| 27.2 Direct assistance to be provided If information cannot be supplied in a passenger’s preferred format, equivalent access must be given by direct assistance. Note See sections 33.3 to 33.6 in relation to equivalent access and direct assistance. Conveyances Premises Infrastructure | # 27.2 Equivalent access (1) If essential information cannot be supplied in a passenger’s preferred format, equivalent access must be given. (2) Certain formats may only be available from certain outlets/locations. Note See sections 33.3 to 33.6 in relation to equivalent access and direct assistance. Conveyances Premises Infrastructure | Agree | Information in alternate formats must be available in a timely manner in order to allow travel with equal amenity. |
| 27.3 Size and format of printing (1) Large print format type size must be at least 18 point sans serif characters. (2) Copy must be black on a light background. Conveyances Premises Infrastructure | # 27.3 Size and format of printing (1) If provided, large print format type size must be at least 18 point sans serif characters. Copy must optimise colour contrast between text and background. Conveyances Premises Infrastructure | Agree | People who have vision impairments demonstrate a wide range in their capacity to determine shape and colour. No one set of colours will suit all users, however bold colours with strong contrast will usually be accessible to a broad range of people. Corporate colours often fall into this category. An excellent reference to colour contrast can be found at: www.lighthouse.org/color_contrast.htm An informative note from a draft Australian Standard (DR 00069 CP) offered guidance on inappropriate colour combinations: A combination of any two of the following colours will be difficult for people with a colour deficiency to distinguish: - green, olive green, yellow, orange, pink and red. |
| 27.4 Access to information about location All passengers must be given the same level of access to information on their whereabouts during a public transport journey. Conveyances | # 27.4 Access to information about location (1) All passengers must be given the same level of access to essential information on their whereabouts during a public transport journey. (2) If this is not practicable, operators must provide equivalent access. Note See sections 33.3 to 33.6 in relation to equivalent access and direct assistance. Conveyances | Agree | Essential information is: announcement of next stop, safety related announcements and any unscheduled change of service. |
| 28.1 Notice of requirement for accessible travel Operators of booked services may request advance notice of a requirement for accessible travel. Conveyances  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains | # 28.1 Notice of requirement for accessible travel (1) Operators of booked services may request reasonable advance notice of a requirement for accessible travel. (2) Passengers with disabilities requiring assistance, or having special requirements, must notify the operator of their requirements when they book their travel. (3) Passengers requiring assistance to transfer from a mobility aid to a seat, or with feeding, personal hygiene / toileting, medication or the dis/assembly and operation of disability aids must book and travel with a carer. Conveyances  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains | Agree | Staff on booked services are able to assist by, for example, folding and storing a manual wheelchair, or storing a manual chair with the passenger in the seating / sleeping compartment, however they are unable to take aids apart for storage. Passengers must provide clear written instructions when requesting staff to assist in handling their disability aid. |
| 28.2 Period of notice of requirement for accessible travel Any advance notice required of a requirement for accessible travel must not exceed the period of notice specified for other passengers. Conveyances  Coaches  Ferries  Dial-a-ride services  Trains | # 28.2 Period of notice of requirement for accessible travel Delete Clause 28.2 (Exemption) | Reinstate | Some mobility devices may not be able to access the seating compartment on existing trains. Operators may require advanced notice to assess the feasibility of scooter access prior to confirming a booking of an allocated space. |
| 28.3 Location of carers, assistants and service animals (1) On booked services, operators must locate carers, assistants or service animals with the passenger with whom they are travelling. (2) In the case of carers or assistants, this would normally be in an adjoining seat. (3) If a passenger is travelling with a service animal, the animal must be able to accompany the passenger at all times and to travel without encroaching onto an access path. Conveyances  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains | # 28.3 Location of carers, assistants and assistance dogs (1) On booked services, operators must locate carers, assistants or assistance dogs with the passenger with whom they are travelling. (2) In the case of carers or assistants, this would normally be in an adjoining seat. (3) If a passenger is travelling with an assistance dog, the animal must be able to accompany the passenger at all times and to travel without encroaching onto an access path. Conveyances  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains | 28.3 Location of carers, assistants and assistance animals (1) On booked services, operators must locate carers, assistants or assistance animals with the passenger with whom they are travelling. (2) In the case of carers or assistants, this would normally be in an adjoining seat. (3) If a passenger is travelling with an assistance dog, the animal must be able to accompany the passenger at all times and to travel without encroaching onto an access path. Conveyances  Trains | While 'comfort animals' are not assistance animals, legitimate use may be made of trained and registered assistance animals other than dogs in directly using a public transport service. |
| 29.1 Equal access to food and drink services Operators and providers must ensure that any food or drink service that is provided as part of a public transport service is equally available to all passengers. Conveyances Premises Infrastructure | # 29.1 Equal access to food and drink services Operators and providers must ensure that any food or drink service that is provided as part of a public transport service is equally available to all passengers. Conveyances Except unbooked train services Premises Except rail premises Infrastructure Except rail infrastructure | Agree | Unbooked rail services will not normally provide food or beverages. Further, premises and infrastructure may have outlets or vending machines that are not run or owned by the transport provider / operator that are not provided as part of a transport service. If operators or providers have catering or canteens they should provide at least equivalent service to passengers with disabilities. |
| 30.1 Disability aids to be in addition to baggage allowance (1) Disability aids (for example, equipment and apparatus including mobility, technical and medical aids) are to be in addition to normal baggage allowances. (2) If possible, disability aids are to be treated in the same way as cabin or accompanied baggage. Conveyances  Aircraft  Coaches  Ferries  Trains | # 30.1 Disability aids to be in addition to baggage allowance (1) Disability aids are to be in addition to normal baggage allowances. (2) If possible, disability aids are to be treated in the same way as cabin or accompanied baggage. (3) Disability aids which will be transported in the luggage compartment must comply with the operator’s size limitations for passenger luggage. (4) Only mobility aids that comply with Part 1X may be carried on public transport conveyances. (5) Operators are not required to carry mobility aids as priority / additional baggage in the luggage compartment of booked services: (a) where allocated space for mobility aids is provided in the seating / sleeping compartment when all allocated spaces in the seating / sleeping compartment are already booked on the required service. (6) Operators are not responsible for the assembly / disassembly or operation of disability aids on public transport services, premises or infrastructure. Conveyances  Aircraft  Coaches  Ferries  Booked Trains | 30.1 Disability aids to be in addition to baggage allowance (1) Disability aids are to be in addition to normal baggage allowances. (2) If possible, disability aids are to be treated in the same way as cabin or accompanied baggage. (3) Disability aids which will be transported in the luggage compartment must comply with the operator’s size limitations for passenger luggage. (4) Only mobility aids that comply with Part 1X may be carried on public transport conveyances. (5) Operators are not required to carry mobility aids as priority / additional baggage in the luggage compartment of booked services: - where allocated space for mobility aids is provided in the seating / sleeping compartment - when all allocated spaces in the seating / sleeping compartment are already booked on the required service. (6) Operators are not responsible for the assembly / disassembly or operation of disability aids on public transport services, premises or infrastructure. Conveyances  Trains - booked services |  |
| 31.1 Priority seating Operators must designate at least 2 of the seats provided on their unbooked conveyances as priority seating for passengers with disabilities and other groups in need of special assistance (for example, the aging). Conveyances  Buses  Ferries  Rail cars  Trams  Light rail | # 31.1 Priority seating (1) Operators must designate at least 2 of the seats provided on their unbooked conveyances as priority seating for passengers with disabilities and other groups in need of special assistance (for example, the aging). (2) Operators may determine the location and orientation of priority seats. Conveyances  Buses  Ferries  Rail cars  Trams  Light rail | Agree |  |

##### TABLE 1 – RECOMMENDED LIGHTING LEVELS FOR RAILWAY STATIONS

|  | CODE REQUIREMENTS |  |  |  |  | COMMENT | RECOMMENDED ILLUMINANCE |  |  | RECOMMENDED UNIFORMITY |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CODE |  | Eav | E min | EV min |  | Eav | E min | EV min | U1 | U2 |
| ENCLOSED STATIONS |  |  |  |  |  |  |  |  |  |  |  |
| ENTRANCE, PASSAGEWAYS, WALKWAYS | AS1428.2 | SECTION 19 | 150 |  |  | AGREED | 150 |  |  | 0.5 |  |
| STAIRS | AS1428.2 | SECTION 19 | 150 |  |  | AGREED | 150 |  |  | 0.5 |  |
| RAMPS | AS1428.2 | SECTION 19 | 150 |  |  | AGREED | 150 |  |  | 0.5 |  |
| TOILETS AND LOCKER ROOMS | AS1428.2 | SECTION 19 | 200 |  |  | AGREED | 200 |  |  | 0.5 |  |
| COUNTER TOPS | AS1428.2 | SECTION 19 | 250 |  |  | AGREED | 250 |  |  | 0.5 |  |
| DISPLAYS (TIMETABLES) | AS1428.2 | SECTION 19 | 200-300 |  |  | AGREED | 200 |  |  | 0.5 |  |
| TELEPHONES (TICKET MACHINES) | AS1428.2 | SECTION 19 | 200 |  |  | AGREED | 200 |  |  | N/A |  |
| GENERAL PLATFORM | AS1680.2.1 | TABLE E1, 1.2 | 160 |  |  | AGREED | 160 |  |  | 0.5 |  |
| YELLOW LINE (PLATFORM EDGE) |  |  |  |  |  | NOTE 1 |  | 150 |  |  |  |
| OPEN STATIONS |  |  |  |  |  |  |  |  |  |  |  |
| TOILETS AND LOCKER ROOMS | AS1428.2 | SECTION 19 | 200 |  |  | AGREED | 200 |  |  | 0.5 |  |
| COUNTER TOPS | AS1428.2 | SECTION 19 | 250 |  |  | AGREED | 250 |  |  | 0.5 |  |
| DISPLAYS (TIMETABLES) | AS1428.2 | SECTION 19 | 200-300 |  |  | AGREED | 200 |  |  | 0.5 |  |
| TELEPHONES (TICKET MACHINES) | AS1428.2 | SECTION 19 | 200 |  |  | AGREED | 200 |  |  | N/A |  |
| YELLOW LINE (PLATFORM EDGE) |  |  |  |  |  | NOTE 1 |  | 30 |  |  |  |
| GENERAL PLATFORM | AS1158.3.1 | CAT P6 | 21 | 7 | 7 | NOTE 2 | 42 | 21 | 14 |  | 7 |
| COVERED AREAS | AS1680.2.1 | TABLE E1, 1.2 | 160 |  |  | AGREED | 160 |  |  | 0.5 |  |
| CORE AREAS (AWNING) | AS1680.2.1 | TABLE E1, 1.2 | 160 |  |  | AGREED | 160 |  |  | 0.5 |  |
| RAMPS AND STEPS (OPEN) | AS1158.3.1 | P8 | 7 | 2 | 2 | NOTES 4 & 5 | 42 | 21 | 14 |  | 7 |
| OPEN FOOTBRIDGE | AS1158.3.1 | CAT P8 | 7 | 2 | 2 | NOTES 4 & 5 | 42 | 21 | 14 |  | 7 |
| PRIMARY ACCESS PATHS | AS1158.3.1 | CAT P6 | 21 | 7 | 7 | NOTE 2 | 42 | 21 | 14 |  | 7 |
| ENCLOSED FOOTBRIDGE | AS 1428.2 | SECTION 19 | 150 |  |  | AGREED | 150 |  |  | 0.5 |  |
| SUBWAYS | AS1158.3.1 | CAT P10 | 35 | 17.5 | 17.5 | AGREED | 35 | 17.5 | 17.5 |  | 7 |