Disability Standards for Accessible Public Transport

Summary

| Section | Description | Comment |
| --- | --- | --- |
| 1.1 to 1.8 | Preliminary clauses | No changes |
| 1.9 | Access path | Exemption |
| 1.11AX | Assistance Dog | Exemption (New Clause) |
| 1.11BX | Boarding Point | Exemption (New Clause) |
| 1.11CX | Booked Services on Trains | Exemption (New clause) |
| 1.11DX | Unbooked Services on Trains | Exemption (New clause) |
| 1.15X | Disability Aid | Exemption (New Clause) |
| 1.18 | Infrastructure | Exemption |
| 1.18X | Level Crossing | Exemption (New Clause) |
| 1.19AX | Mobility Aid | Exemption (New Clause) |
| 1.19BX | Nominated Accessible Boarding Point | Exemption (New clause) |
| 1.21 | Premises | Exemption |
| 1.23X | Sleeping berth | Exemption |
| Part 1X | Mobility Aids | Exemption (New part) |
| 2.1 | Access paths - Unhindered passage | Exemption |
| 2.2 | Access paths - Continuous accessibility | Exemption |
| 2.3 | Access paths - Path branching into 2 or more parallel tracks | No Change |
| 2.4 | Access paths - Minimum unobstructed width | Exemption |
| 2.4X | Access paths - Minimum unobstructed width for railway platforms | Exemption (New Clause) (HREOC Outcome) |
| 2.5 | Poles and obstacles, etc | Exemption |
| 2.5AX | Level crossings | Exemption (New clause) |
| 2.6 | Access paths - Access paths — conveyances | Exemption |
| 2.7 | Minimum width between front wheel arches of bus | No Change |
| 2.8 | Access paths - Extent of path | Exemption |
| 2.9 | Access paths - When is an access path not required | No Change |
| 3.1 | Manoeuvring areas - Circulation space for wheelchairs to turn in | Exemption |
| 3.2 | Manoeuvring areas - Access for passengers in wheelchairs, etc | No Change |
| 3.3 | Manoeuvring areas - Limited on-board manoeuvring | No Change |
| 4.1 | Passing areas - Minimum width | Exemption (Deleted Clause) |
| 4.2 | Passing areas - Two-way access paths and aerobridges | Exemption |
| 4.3 | Passing areas - Passing areas — conveyances | Exemption |
| 5.1 | Resting points - When resting points must be provided | Exemption |
| 6.1 | Ramps - Ramps on access paths | Exemption |
| 6.2 | Boarding ramps | Exemption |
| 6.3 | Ramps - Minimum allowable width | Exemption |
| 6.4 | Ramps - Slope of external boarding ramps | No Change |
| 6.5 | Ramps - Slope of ramps connected to pontoon wharves | No Change |
| 7.1 | Waiting areas - Minimum number of seats to be provided | No Change |
| 7.2 | Waiting areas - Minimum number of allocated spaces to be provided | No Change |
| 8.1 | Boarding - Boarding points and kerbs | Exemption |
| 8.2 | Boarding - When boarding devices must be provided | Exemption |
| 8.3 | Boarding - Use of boarding devices | No Change |
| 8.4 | Boarding - Hail-and-ride services | No Change |
| 8.5 | Boarding - Width and surface of boarding devices | Exemption (Delete Clause) |
| 8.6 | Maximum load to be supported by boarding device | Exemption |
| 8.7 | Boarding - Signals requesting use of boarding device | Exemption (Delete Clause) |
| 8.8 | Boarding - Notification by passenger of need for boarding device | Exemption |
| 9.1 | Allocated space - Minimum size for allocated space | Exemption |
| 9.2 | Allocated space - Minimum number of allocated spaces to be provided | No Change |
| 9.3 | Minimum head room | No Change |
| 9.4 | Number of allocated spaces to be provided — buses | No Change |
| 9.5 | Number of allocated spaces to be provided — ferries | No Change |
| 9.6 | Number of allocated spaces to be provided — train cars, etc | Exemption |
| 9.7 | Allocated space - Consolidation of allocated spaces | Exemption |
| 9.8 | Allocated spaces in aircraft and coaches | No change |
| 9.9 | Use of allocated space for other purposes | No change |
| 9.10 | Allocated space - International symbol of accessibility to be displayed | Exemption |
| 9.11 | Allocated space - Movement of mobility aid in allocated space | No change |
| 10.1 | Surfaces - Compliance with Australian Standard | Exemption – Conveyances, Exemption Infrastructure/Premises see 10.X |
| 10.1X | Surfaces - Compliance with Australian Standard | Exemption (New Clause) (Infrastructure) |
| 11.1 | Handrails and grabrails - Compliance with Australian Standard — premises and infrastructure | Exemption |
| 11.2 | Handrails and grabrails - Handrails to be provided on access paths | Exemption |
| 11.3 | Handrails and grabrails - Handrails on steps | Exemption |
| 11.4 | Handrails and grabrails - Handrails above access paths | Exemption |
| 11.5 | Handrails and grabrails - Compliance with Australian Standard | Exemption |
| 11.6 | Grabrail to be provided where fares are to be paid | Exemption |
| 11.7 | Handrails and grabrails - Grabrails to be provided in allocated spaces | Exemption |
| 12.1 | Doorways and doors - Doors on access paths | Exemption |
| 12.2 | Doorways and doors - Compliance with Australian Standard — premises and infrastructure | Exemption |
| 12.3 | Doorways and doors - Weight activated doors and sensors | Exemption |
| 12.4 | Doorways and doors - Clear opening of doorways | Exemption |
| 12.5 | Doorways and doors - Vertical height of doorways | No Change |
| 12.6 | Doorways and doors - Automatic or power-assisted doors | No Change |
| 13.1 | Lifts - Compliance with Australian Standard — premises and infrastructure | Exemption |
| 14.1 | Stairs - Stairs not to be sole means of access | Exemption |
| 14.2 | Stairs - Compliance with Australian Standards — premises and infrastructure | No change |
| 14.3 | Stairs - Compliance with Australian Standards — conveyances | Exemption |
| 14.4 | Compliance with Australian Design Rule 58 — conveyances | No Change |
| 15.1 | Toilets - Unisex accessible toilet — premises and infrastructure | Exemption |
| 15.2 | Toilets - Location of accessible toilets | Exemption |
| 15.3 | Toilets - Unisex accessible toilet — ferries and accessible rail cars | Exemption |
| 15.4 | Toilets - Requirements for accessible toilets — ferries and accessible rail cars | Exemption |
| 15.5 | Accessible toilet to be provided — aircraft | No Change |
| 15.6 | Stops to be offered if accessible toilet not provided — coaches | No Change |
| 16.1 | Symbols - International symbols for accessibility and deafness | No Change |
| 16.2 | Compliance with AS2899.1 (1986) | Exemption |
| 16.3 | Symbols - Accessibility symbols to incorporate directional arrows | Exemption |
| 16.4 | Symbols - Accessibility symbol to be visible on accessible buses | No Change |
| 16.5 | Symbols - Accessibility symbol to be visible on accessible doors | Exemption |
| 17.1 | Signs - Height and illumination | No Change |
| 17.2 | Signs - Location — premises and infrastructure | No Change |
| 17.3 | Signs - Location — conveyances | No Change |
| 17.4 | Signs - Destination signs to be visible from boarding point | Exemption |
| 17.5 | Signs - Electronic notices | Exemption |
| 17.6 | Signs - Raised lettering or symbols or use of Braille | Exemption |
| 17.7 | Signs - Taxi registration numbers | No Change |
| 18.1 | Tactile ground surface indicators -Location | Exemption |
| 18.2 | Tactile ground surface indicators - Style and dimensions | Exemption |
| 18.3 | Tactile ground surface indicators - Instalment at accessible bus boarding points | No Change |
| 18.4 | Tactile ground surface indicators - Instalment at railway stations | Exemption |
| 18.5 | Tactile ground surface indicators - Instalment at wharves | No Change |
| 19.1 | Alarms - Emergency warning systems | Exemption |
| 20.1 | Lighting - Illumination levels— premises and infrastructure | Exemption |
| 20.2 | Lighting - Illumination levels — conveyances | No Change |
| 20.3 | Lighting - Dimming | No Change |
| 21.1 | Controls - Compliance with Australian Standard — premises and infrastructure | Exemption |
| 21.2 | Controls - Passenger-operated devices for opening and closing doors | Exemption |
| 21.3 | Controls Location of passenger-operated controls for opening and locking doors | Exemption |
| 21.4 | Controls Location of passenger- Signal devices for conveyances that stop on request | No Change |
| 22.1 | Furniture and fitments - Tables, benches, counters, etc | Exemption |
| 22.2 | Furniture and fitments - Information desks, check-in counters, etc — airports | No Change |
| 22.3 | Furniture and fitments - Accessible sleeping berths — ferries and trains | No Change |
| 22.4 | Furniture and fitments - Accessible sleeping berths — ferries | No Change |
| 22.5 | Furniture and fitments - Accessible sleeping berths — trains | Exemption |
| 22.6 | Furniture and fitments - Accessible berths to be connected to access path — ferries and trains | No Change |
| 23.1 | Street furniture - Seats | Exemption |
| 24.1 | Gateways - Gateways and checkouts | Exemption |
| 25.1 | Payment of fares - Passengers to pay fares | No change |
| 25.2 | Payment of fares - Fare payment and ticket validation systems | No change |
| 25.3 | Payment of fares - Vending machines | Exemption |
| 25.4 | Payment of fares - Circulation space in front of vending machine | Exemption |
| 26.1 | Hearing augmentation – Public address systems— premises and infrastructure | No change |
| 26.2 | Hearing augmentation – listening systems - Public address systems — conveyances | Exemption |
| 27.1 | Information - Access to information about transport services | Exemption |
| 27.2 | Direct assistance to be provided | Exemption |
| 27.3 | Information - Size and format of printing | Exemption |
| 27.4 | Information - Access to information about location | Exemption |
| 28.1 | Booked services - Notice of requirement for accessible travel | Exemption |
| 28.2 | Booked services - Period of notice of requirement for accessible travel | Exemption (Delete Clause) |
| 28.3 | Booked services - Location of carers, assistants and service animals | Exemption |
| 28.4 | Booked services - Accessible seats to be available for passengers with disabilities | No change |
| 29.1 | Food and drink services - Equal access to food and drink services | Exemption |
| 29.2 | Food and drink services - Distance around accessible tables | No change |
| 29.3 | Food and drink services - Space for passengers using mobility aids | No change |
| 30.1 | Belongings - Disability aids and mobility aids to be in addition to baggage allowance | Exemption |
| 31.1 | Priority - Priority seating | Exemption |
| 31.2 | Priority - Information to be provided about vacating priority seating | No change |

Contents

Part 1	Preliminary	17

Division 1.1	Purpose and application of Standards	17

1.1	Name of Standards	17

1.2	Purpose of Standards	17

1.3	Acknowledgment of rights of passengers, operators and providers	17

1.4	Application of Standards	17

1.5	Guidelines	17

1.6	Incorporation of Australian Standards and Australian Design Rules	18

1.7	Applicability of Standards	18

Division 1.2	Meaning of important terms	19

1.8	Purpose of Division 1.2	19

1.9	Access path	20

Revised Clause (Exemption)	20

1.9	Access path	20

1.10	Airport that does not accept regular public transport services	20

1.11	Allocated space	20

New Clause (Exemption) – Replaces Clause 1.23	21

1.11AX	Assistance Dog	21

Reason		21

New Clause (Exemption)	21

1.11BX	Boarding Point	21

New Clause (Exemption)	21

1.11CX	Booked Services on Trains	21

New Clause (Exemption)	22

1.11DX	Unbooked Services on Trains	22

1.12	Conveyance	22

1.13	Dedicated school bus and dedicated school bus service	22

1.14	Dial-a-ride service	22

1.15	Direct assistance	23

New Clause (Exemption)	23

1.15X	Disability aid	23

1.16	Equivalent access	24

1.17	Hail-and-ride service	24

1.18	Infrastructure	24

1.18	Infrastructure	24

New Clause (Exemption)	25

1.19	Manoeuvring areas	25

New Clause (Exemption)	25

1.19AX	Mobility aid	25

1.19BX	Nominated Accessible Boarding Point	26

1.20	Operator	26

1.21	Premises	26

Revised Clause (Exemption)	26

1.21	Premises	26

1.22	Provider	27

1.23	Public transport service	27

New Clause (Exemption)	27

1.23X	 Sleeping berth	27

1.24	Small aircraft	27

New Part (Exemption)	28

Part 1X	Mobility aids (new part)	28

1X.1	Design Criteria	28

Maximum size	28

1X.2	Performance criteria	29

1X.3	Orientation	30

(3) Manoeuvrability:  restates performance specification in existing clause 3.1.	31

Part 2	Access paths	33

2.1	Unhindered passage	33

Revised Clause (Exemption)	33

2.1	Unhindered passage	33

2.2	Continuous accessibility	34

2.2	Continuous accessibility	35

2.3	Path branching into 2 or more parallel tracks	35

2.4	Minimum unobstructed width	36

2.4	Minimum unobstructed width	36

New Clause  (HREOC Outcome)	37

2.4X	Minimum unobstructed width for railway platforms	37

2.5	Poles and obstacles, etc	38

Revised Clause (Exemption)	39

2.5	Poles and obstacles, etc	39

New Clause (Exemption)	39

2.5AX	Level Crossings	39

2.6	Access paths — conveyances	40

Revised Clause (Exemption)	40

2.6	Access paths — conveyances	40

2.7	Minimum width between front wheel arches of bus	41

2.8	Extent of path	42

Revised Clause (Exemption)	42

2.8	Extent of path	42

2.9	When is an access path not required	43

Part 3	Manoeuvring areas	44

3.1	Circulation space for wheelchairs to turn in	44

Revised Clause (Exemption)	44

3.1	Circulation space for mobility aids to turn in	44

3.2	Access for passengers in wheelchairs, etc	45

3.3	Limited on-board manoeuvring	45

Part 4	Passing areas	46

4.1	Minimum width	46

Delete Clause 4.1 Minimum width (Exemption)	46

4.2	Two-way access paths and aerobridges	46

Revised Clause  (Exemption)	46

4.2	Two-way access paths and aerobridges	46

4.3	Passing areas — conveyances	47

Revised Clause  (Exemption)	48

4.3	Passing areas — conveyances	48

Part 5	Resting points	49

5.1	When resting points must be provided	49

5.1	When resting points must be provided	49

Part 6	Ramps	51

6.1	Ramps on access paths	51

Revised Clause (Exemption)	51

6.1	Ramps on access paths	51

6.2	Boarding ramps	52

Revised Clause (Exemption)	52

6.2	Boarding ramps	52

6.3	Minimum allowable width	53

Revised Clause (Exemption)	53

6.3	Minimum allowable width	53

6.4	Slope of external boarding ramps	53

6.5	Slope of ramps connected to pontoon wharves	54

Part 7	Waiting areas	55

7.1	Minimum number of seats to be provided	55

7.2	Minimum number of allocated spaces to be provided	55

Part 8	Boarding	56

8.1	Boarding points and kerbs	56

Revised Clause (Exemption)	56

8.1	Boarding points and kerbs	56

8.2	When boarding devices must be provided	57

Revised Clause (Exemption)	57

8.2	When boarding devices must be provided	57

8.3	Use of boarding devices	58

8.4	Hail-and-ride services	58

8.5	Width and surface of boarding devices	58

Delete Clause 8.5 (Exemption)	59

8.6	Maximum load to be supported by boarding device	59

8.6	Maximum load to be supported by boarding device	59

8.7	Signals requesting use of boarding device	60

Delete Clause 8.7 (Exemption)	60

8.8	Notification by passenger of need for boarding device	61

8.8	Notification by passenger of need for boarding device	61

Part 9	Allocated space	63

9.1	Minimum size for allocated space	63

Revised Clause (Exemption)	63

9.1	Minimum size for allocated space	63

9.2	Minimum number of allocated spaces to be provided	64

9.3	Minimum head room	64

9.4	Number of allocated spaces to be provided — buses	64

9.5	Number of allocated spaces to be provided — ferries	65

9.6	Number of allocated spaces to be provided — train cars, etc	65

Revised Clause (Exemption)	65

9.6	Number of allocated spaces to be provided — train cars, etc	65

9.7	Consolidation of allocated spaces	69

Revised Clause (Exemption)	69

9.7	Consolidation of allocated spaces	69

9.8	Allocated spaces in aircraft and coaches	70

9.9	Use of allocated space for other purposes	70

9.10	International symbol of accessibility to be displayed	71

Revised Clause (Exemption)	71

9.10	International symbol of accessibility to be displayed	71

9.11	Movement of mobility aid in allocated space	72

Part 10	Surfaces	73

10.1	Compliance with Australian Standard	73

Revised Clause for Conveyances (Exemption)	73

10.1	Compliance with Australian Standard	73

10.1X	Compliance with Australian Standard	74

Part 11	Handrails and grabrails	75

11.1	Compliance with Australian Standard — premises and infrastructure	75

Revised Clause (Exemption)	75

11.1	Compliance with Australian Standard — premises and infrastructure	75

11.2	Handrails to be provided on access paths	76

Revised Clause (Exemption)	76

11.2	Handrails to be provided on access paths	76

11.3	Handrails on steps	77

Revised Clause (Exemption)	77

11.3	Handrails on steps	77

11.4	Handrails above access paths	78

Revised Clause (Exemption)	78

11.4	Handrails above access paths	78

11.5	Compliance with Australian Standard	79

Revised Clause (Exemption)	79

11.5	Grabrail compliance requirements	79

11.6	Grabrail to be provided where fares are to be paid	80

11.6	Grabrail to be provided where fares are to be paid	80

11.7	Grabrails to be provided in allocated spaces	81

Revised Clause (Exemption)	81

11.7	Grabrails to be provided in allocated spaces	81

Part 12	Doorways and doors	82

12.1	Doors on access paths	82

Revised Clause (Exemption)	82

12.1	Doors on access paths	82

12.2	Compliance with Australian Standard — premises and infrastructure	83

Revised Clause (Exemption)	83

12.2	Compliance with Australian Standard — premises and infrastructure	83

12.3	Weight activated doors and sensors	83

12.3	Weight activated doors and sensors	84

12.4	Clear opening of doorways	84

Revised Clause (Exemption)	85

12.4	Clear opening of doorways	85

12.5	Vertical height of doorways	85

12.6	Automatic or power-assisted doors	86

Part 13	Lifts	87

13.1	Compliance with Australian Standard — premises and infrastructure	87

Revised Clause (Exemption)	87

13.1	Compliance with Australian Standard — premises and infrastructure	87

Part 14	Stairs	88

14.1	Stairs not to be sole means of access	88

Revised Clause (Exemption)	88

14.1	Stairs not to be sole means of access	88

14.2	Compliance with Australian Standards — premises and infrastructure	89

14.3	Compliance with Australian Standards — conveyances	89

Revised Clause (Exemption)	89

14.3	Stairs on conveyances	89

14.4	Compliance with Australian Design Rule 58 — conveyances	91

Part 15	Toilets	92

15.1	Unisex accessible toilet — premises and infrastructure	92

Revised Clause (Exemption)	92

15.1	Unisex accessible toilet — premises and infrastructure	92

15.2	Location of accessible toilets	93

Revised Clause (Exemption)	93

15.2	Location of accessible toilets	93

15.3	Unisex accessible toilet — ferries and accessible rail cars	94

Revised Clause (Exemption)	94

15.3	Unisex accessible toilet — ferries and trains	94

15.4	Requirements for accessible toilets — ferries and accessible rail cars	94

Revised Clause (Exemption)	95

15.4	Requirements for accessible toilets — ferries and accessible rail cars	95

15.5	Accessible toilet to be provided — aircraft	96

15.6	Stops to be offered if accessible toilet not provided — coaches	97

Part 16	Symbols	98

16.1	International symbols for accessibility and deafness	98

16.2	Compliance with AS2899.1 (1986)	98

Revised Clause (Exemption)	98

16.2	Compliance with AS2899.1 (1986)	98

16.3	Accessibility symbols to incorporate directional arrows	99

Revised Clause (Exemption)	99

16.3	Accessibility symbols to incorporate directional arrows	99

16.4	Accessibility symbol to be visible on accessible buses	100

16.5	Accessibility symbol to be visible on accessible doors	100

Revised Clause (Exemption)	100

16.5	Accessibility symbol to be visible on accessible doors	100

Part 17	Signs	102

17.1	Height and illumination	102

17.2	Location — premises and infrastructure	102

17.3	Location — conveyances	102

17.4	Destination signs to be visible from boarding point	103

Revised Clause (Exemption)	104

17.4	Destination signs to be visible from boarding point	104

17.5	Electronic notices	104

Revised Clause (Exemption)	104

17.5	Electronic notices	104

17.6	Raised lettering or symbols or use of Braille	105

Revised Clause (Exemption)	105

17.6	Raised lettering or symbols or use of Braille	105

17.7	Taxi registration numbers	106

Part 18	Tactile ground surface indicators	107

18.1	Location	107

Revised Clause (Exemption)	107

18.1	Location	107

18.2	Style and dimensions	108

Revised Clause (Exemption)	108

18.2	Style and dimensions	108

18.3	Instalment at accessible bus boarding points	110

18.4	Instalment at railway stations	110

Revised Clause (Exemption)	110

18.4	Instalment at railway stations	110

18.5	Instalment at wharves	112

Part 19	Alarms	113

19.1	Emergency warning systems	113

19.1	Emergency warning systems	113

Part 20	Lighting	114

20.1	Illumination levels— premises and infrastructure	114

Revised Clause (Exemption)	114

20.1	Illumination levels— premises and infrastructure	114

20.2	Illumination levels — conveyances	115

20.3	Dimming	116

Part 21	Controls	117

21.1	Compliance with Australian Standard — premises and infrastructure	117

Revised Clause (Exemption)	117

21.1	Compliance with Australian Standard — premises and infrastructure	117

21.2	Passenger-operated devices for opening and closing doors	117

Revised Clause (Exemption)	118

21.2	Passenger-operated devices for opening and closing doors	118

21.3	Location of passenger-operated controls for opening and locking doors	118

Revised Clause (Exemption)	119

21.3	Location of passenger-operated controls for opening and locking doors	119

21.4	Signal devices for conveyances that stop on request	120

Part 22	Furniture and fitments	121

22.1	Tables, benches, counters, etc	121

Revised Clause (Exemption)	121

22.1	Tables, benches, counters, etc	121

22.2	Information desks, check-in counters, etc — airports	122

22.3	Accessible sleeping berths — ferries and trains	122

22.4	Accessible sleeping berths — ferries	122

22.5	Accessible sleeping berths — trains	123

Revised Clause (Exemption)	123

22.5	Accessible sleeping berths — trains	123

22.6	Accessible berths to be connected to access path — ferries and trains	125

Part 23	Street furniture	126

23.1	Seats	126

Revised Clause (Exemption)	126

23.1	Seats	126

Part 24	Gateways	129

24.1	Gateways and checkouts	129

Revised Clause (Exemption)	129

24.1	Gateways and checkouts	129

Part 25	Payment of fares	131

25.1	Passengers to pay fares	131

25.2	Fare payment and ticket validation systems	131

25.3	Vending machines	131

Revised Clause (Exemption)	131

25.3	Ticket vending machines	131

25.4	Circulation space in front of vending machine	132

Revised Clause (Exemption)	132

25.4	Circulation space in front of a ticket vending machine	132

Part 26	Hearing augmentation–listening systems	134

26.1	Public address systems— premises and infrastructure	134

26.2	Public address systems — conveyances	134

Revised Clause (Exemption)	134

26.2	Public address systems — conveyances	134

Part 27	Information	136

27.1	Access to information about transport services	136

Revised Clause (Exemption)	136

27.1	Access to information about transport services	136

27.2	Direct assistance to be provided	136

Revised Clause (Exemption)	137

27.2	Equivalent access	137

27.3	Size and format of printing	137

Revised Clause (Exemption)	137

27.3	Size and format of printing	137

27.4	Access to information about location	138

Revised Clause (Exemption)	138

27.4	Access to information about location	138

Part 28	Booked services	139

28.1	Notice of requirement for accessible travel	139

Revised Clause (Exemption)	139

28.1	Notice of requirement for accessible travel	139

28.2	Period of notice of requirement for accessible travel	140

Delete Clause 28.2 (Exemption)	140

28.3	Location of carers, assistants and service animals	140

Revised Clause (Exemption)	141

28.3	Location of carers, assistants and assistance dogs	141

28.4	Accessible seats to be available for passengers with disabilities	141

Part 29	Food and drink services	143

29.1	Equal access to food and drink services	143

Revised Clause (Exemption)	143

29.1	Equal access to food and drink services	143

29.2	Distance around accessible tables	144

29.3	Space for passengers using mobility aids	144

Part 30	 Belongings	145

30.1	Disability aids to be in addition to baggage allowance	145

Revised Clause (Exemption)	145

30.1	Disability aids to be in addition to baggage allowance	145

Part 31	Priority	147

31.1	Priority seating	147

31.1	Priority seating	147

31.2	Information to be provided about vacating priority seating	148

Part 32	Adoption	148

32.1	Effect and application of these Standards	148

32.2	Manufacture to be completed before target dates	148

Part 33	Compliance	149

33.1	Date for compliance with these Standards — new conveyances, premises and infrastructure	149

33.2	Date for compliance with these Standards — conveyances, premises and infrastructure in use at target dates	149

33.3	Equivalent access	149

33.4	Consultation about proposals for equivalent access	149

33.5	Equivalent access without discrimination	150

33.6	Direct assistance	150

33.7	Exceptional cases — unjustifiable hardship	150

Part 34	Review	153

34.1	Timetable for review	153

Schedule 1	Target dates for compliance	154

Part 1	Target date — 31 December 2007	154

Part 2	Target date — 31 December 2012	155

Part 3	Target date — 31 December 2017	158

Part 4	Target date — 31 December 2022	159

Part 5	Target date — 31 December 2032	160

Part 1	Preliminary

Division 1.1	Purpose and application of Standards

1.1	Name of Standards

These Standards are the Disability Standards for Accessible Public Transport .

Note   These Standards take effect as provided by subsections 31 (3) and (4) of the Disability Discrimination Act 1992.

1.2	Purpose of Standards

(1)	The Disability Discrimination Act 1992 seeks to eliminate discrimination, ‘as far as possible’, against people with disabilities. Public transport is a service covered by the Disability Discrimination Act 1992.

(2)	The purpose of these Standards is to enable public transport operators and providers to remove discrimination from public transport services.

1.3	Acknowledgment of rights of passengers, operators and providers

In prescribing how public transport is to be made accessible, these Standards acknowledge certain rights of passengers, operators and providers, as well as imposing some responsibilities.

1.4	Application of Standards

(1)	These Standards apply to the widest possible range of people with disabilities as defined by the Disability Discrimination Act 1992.

(2)	These Standards apply to all operators and the conveyances they use to provide public transport services. They also apply to providers and supporting premises and infrastructure.

1.5	Guidelines

These Standards are accompanied by Guidelines. Passengers, operators and providers need to consult the Guidelines when interpreting these Standards.

1.6	Incorporation of Australian Standards and Australian Design Rules

These Standards:

(a)	prescribe certain sections of Australian Standard 1428.1 (Design for access and mobility. Part 1 General requirements for access — Buildings, second edition, 2001) for transport premises, infrastructure and conveyances; and

(b)	prescribe some of the additional requirements of Australian Standard 1428.2 (Enhanced and additional requirements — Buildings and facilities, 1992) for transport premises, infrastructure and conveyances; and

(c)	prescribe some of the additional requirements of Australian Standard 1428.1 Supplement 1 (Design for access and mobility: General requirements for access — Buildings — Commentary, 2001) for transport premises, infrastructure and conveyances; and

(d)	prescribe some of the additional requirements of Australian Standard 1428.4 (Design for access and mobility: Tactile ground surface indicators for the orientation of people with vision impairments, 1992) for transport premises, infrastructure and conveyances; and

(e)	prescribe some of the additional requirements of Australian Standard 1735.12 (Lifts, escalators and moving walks: Facilities for persons with disabilities, 1999) for transport premises, infrastructure and conveyances; and

(f)	prescribe some of the additional requirements of Australian Standard 2220.1 and Australian Standard 2220.2 (Emergency warning and intercommunication systems in buildings, 1993) for transport premises, infrastructure and conveyances; and

(g)	prescribe some of the additional requirements of Australian Standard 2899.1 (Public information symbol signs: General information signs, 1986) for transport premises, infrastructure and conveyances; and

(h)	prescribe some of the additional requirements of Australian Standard 3856.1 (Hoists and ramps for people with disabilities — Vehicle mounted: Product requirements, 1998) for transport premises, infrastructure and conveyances; and

(i)	prescribe some of the additional requirements of Australian Design Rule 58 (Requirements for omnibuses designed for hire and reward, as amended to include Road Vehicle (National Standards) Determination No 2 of 1992) for transport premises, infrastructure and conveyances; and

(j)	apply additional requirements specific to public transport.

1.7	Applicability of Standards

These Standards apply to conveyances, premises and infrastructure as indicated in a table below each section.

Examples

Example 1   A section that includes the information set out below applies to all conveyances, premises and infrastructure.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Example 2   A section that includes the information set out below applies to all premises and infrastructure but does not apply to conveyances.

|  | Premises | Infrastructure |
| --- | --- | --- |

Example 3   A section that includes the information set out below applies only to conveyances that are buses and does not apply to premises or infrastructure.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses |  |  |

Example 4   A section that includes the information set out below applies to all infrastructure, except airports that do not accept regular public transport services, and does not apply to conveyances or premises.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Division 1.2	Meaning of important terms

1.8	Purpose of Division 1.2

(1)	This Division gives the meanings for some important terms used in these Standards.

(2)	Unless the contrary intention appears, any other term that is used in these Standards and in the Disability Discrimination Act 1992 has the same meaning in these Standards as it has in the Act.

Note   See section 4 of the Disability Discrimination Act 1992 for definitions of other relevant terms, for example disability and disability discrimination.

Original Clause

1.9	Access path

An access path is a path that permits independent travel for all passengers within public transport premises, infrastructure or conveyances.

Revised Clause (Exemption)

1.9	Access path

An access path is a path that permits independent travel for all passengers within public transport premises, infrastructure or conveyances. An access path may include a compliant walkway, ramp or lift.

An access path on a rail conveyance is a path from the external passenger doorway to the allocated space, priority seats and other essential facilities for passengers with mobility aids.

An access path on a railway station is a clear and unobstructed path from a nominated boundary point to a nominated accessible boarding point on each platform and to all accessible facilities.

Reason 1.9

Improved definition for access paths for a rail environment. The standards require various compliant features to be available at or along access paths. It is important for the effective and efficient design of rail premises, infrastructure and conveyances that it is clear where these features are to be provided.  This clarification will aid the end users to know which path is compliant e.g. stations where the topography does not allow for all paths to be compliant.

1.10	Airport that does not accept regular public transport services

An airport that does not accept regular public transport services is an airport that accepts aircraft other than aircraft that operate for the purpose mentioned in paragraph 206 (1) (c) of the Civil Aviation Regulations 1988.

1.11	Allocated space

An allocated space is a three dimensional space that can accommodate a wheelchair or similar mobility aid.

New Clause (Exemption) – Replaces Clause 1.23

1.11AX	Assistance Dog

- An assistance dog, is a dog that is registered and trained to alleviate the effects of a person’s disability on public transport.

- An assistance dog must be under the direct control of the person with the disability at all times and wear a recognised form of identification as a trained assistance dog.

- The handler is solely responsible for the conduct, care, feeding, hygiene and toileting of their animal.

Reason 1.11AX

The term “assistance dog” will need to be uniformly used throughout the document.

There is no definition in the Standards to differentiate a service / assistance animal from a therapy animal or pet.  Relevant clauses are brought across from the Guidelines and to pick up HREOC recommendations contained in its report following consultations on Section 9 (1) (f) of the Disability Discrimination Act (18 November 2003).

New Clause (Exemption)

1.11BX	Boarding Point

A boarding point is the place where passengers board a public transport conveyance.

Reason 1.11BX

To better define the boarding point for a conveyance.

New Clause (Exemption)

1.11CX	Booked services on trains

Booked services on trains are long distance passenger train services where prior booking is required to purchase a specific seat and/or allocated space.

Reason 1.11CX

To define the requirements and service.

New Clause (Exemption)

1.11DX	Unbooked services on trains

Unbooked services on trains are generally suburban passenger train services where no seat is booked or reserved for a specific person.

Reason 1.11DX

To define the requirements and differentiate booked and unbooked services.

1.12	Conveyance

(1)	A conveyance includes any of the following, to the extent that they are used to provide a public transport service:

(a)	aircraft;

(b)	buses or coaches;

(c)	ferries;

(d)	taxis;

(e)	trains, trams, light rail, monorails, rack railways;

(f)	any other rolling stock, vehicle or vessel classified as public transport within its jurisdiction by regulation or administrative action of any Government in Australia.

(2)	A conveyance does not include the following:

(a)	charter boats (including water taxis);

(b)	limousines (including chauffeured hire cars);

(c)	self-drive rental cars.

1.13	Dedicated school bus and dedicated school bus service

(1)	A bus is a dedicated school bus only during the time in which it is being used to provide a dedicated school bus service.

(2)	A dedicated school bus service is a service that operates to transport primary or secondary students to or from school or for other school purposes.

1.14	Dial-a-ride service

A dial-a-ride service is a service that:

(a)	is usually operated by a small bus; and

(b)	serves a local community; and

(c)	operates on flexible routes that allow passengers to be picked up and dropped off at their front doors.

1.15	Direct assistance

Direct assistance is help given by an operator or provider:

(a)	to make public transport accessible to a person with a disability when premises, infrastructure or conveyances do not fully comply with these Standards; or

(b)	to provide non-discriminatory access on request.

New Clause (Exemption)

1.15X	Disability aid

A disability aid is a device designed to be used by a person with a disability to alleviate the effects of that disability.

Disability aids include personal mobility, sensory and medical devices, but do not include devices designed for use by more than one person at the one time nor those that are primarily designed for use as transportation vehicles.

Constraints that are part of providing safe and effective public transport services and operations may limit the types of aids that can be accommodated on transport services, premises and infrastructure.

Passengers are responsible for the supply, assembly and operation of their disability aids at all times while using public transport services / facilities.

Reason 1.15X

Although there is reference in the Standards to disability aids, there is no definition of what this term means or includes.  Due to safety, operational and space constraints, disability and mobility aids, as opposed to transport vehicles / devices, must be defined in relation to the alleviation of a disability on a transport service.

Devices carrying more than one person, or devices that are essentially vehicles rather than a personal aid essential to alleviate a disability, are not suitable and often not safe on trains and stations.  This wording extracts relevant clauses from the Guidelines and brings them across to the Standards to provide increased certainty for passengers and operators/providers.

For the occupational health and safety of both passenger and crew, train crew are unable to assist with the assembly or disassembly of disability aids.  Staff on booked services are able to assist by folding a manual wheelchair for a passenger wanting to transfer to a seat and store the chair with the passenger in the seating / sleeping compartment, however they are unable to take aids apart for storage.

1.16	Equivalent access

(1)	Equivalent access is a process, often involving the provision of direct assistance, under which an operator or provider is permitted to vary the equipment or facilities that give access to a public transport service, so long as an equivalent standard of amenity, availability, comfort, convenience, dignity, price and safety is maintained.

(2)	Equivalent access does not include a segregated or parallel service.

1.17	Hail-and-ride service

A hail-and-ride service is a service operated by a bus that follows set routes, but may stop for passengers at any safe point on the route.

Original Clause

1.18	Infrastructure

(1)	Infrastructure is any structure or facility that is used by passengers in conjunction with travelling on a public transport service.

(2)	Infrastructure does not include any area beyond immediate boarding points (for example, bus stops, wharves, ranks, rail stations, terminals).

Revised Clause (Exemption)

1.18	Infrastructure

(1)	Infrastructure is any structure or facility that is used by passengers to gain direct access to a public transport service.  Infrastructure includes boarding points, footbridges, open railway platforms and open shelters.

(2)	Infrastructure does not include any area beyond immediate boarding points.

Reason 1.18

To make the definition of infrastructure clearer so that structures within the rail corridor that are unrelated to provision of public transport services are not subject to compliance requirements.

New Clause (Exemption)

1.18X		Level crossing

A level crossing is any vehicular and/or pedestrian crossing of a railway corridor at grade, as defined in AS1742.7.

Reason 1.18X

Level crossings were not included in the Transport Standards previously.

1.19	Manoeuvring areas

A manoeuvring area is a space in which a wheelchair or similar mobility aid is able to turn.

New Clause (Exemption)

1.19AX	Mobility aid

- A mobility aid is a disability aid that is designed to be used by one person with a disability to alleviate the effects of their limited mobility.

- Mobility aids include wheelchairs and motorised scooters but do not include those devices that are primarily designed for use as transport or recreational vehicles.

- A mobility aid for use on public transport services, premises and infrastructure must meet the requirements of Part 1X Mobility Aids.

Reason 1.19AX

Although there is reference in the Standards to mobility aids, there is no definition of what this term means.  Mobility aids, as opposed to transport vehicles / devices, must be defined in relation to the alleviation of a disability on a transport service.  Devices carrying more than one person, or devices that are essentially vehicles rather than a personal aid essential to alleviate a disability, are unsuitable and often unsafe on trains and stations.  The maximum size of a mobility aid must also be included within the Standards to apply to all clauses and not just allocated space, due to the finite dimensions of track width and gauge, track corridor width and height of overhead wiring.

Most States provide an integrated ticketing system for bus, rail and ferry.  For consistency clause 2.7 (width between wheel arches on buses) is the common constraint.

New Clause (Exemption)

1.19BX	Nominated accessible boarding point

A nominated accessible boarding point is a boarding point nominated by the Provider for accessible boarding by people with disabilities to provide a coordinated and central location for providing direct assistance if necessary, and the provision of core facilities/services.

Reason 1.19BX

This is in line with Guidelines clauses 8.2 (1) and 8.2 (2) covering independent access.

1.20	Operator

(1)	An operator is a person or organisation (including the staff of the organisation) that provides a public transport service to the public or to sections of the public.

(2)	A public transport service may have more than one operator.

Original Clause

1.21	Premises

(1)	Premises are structures, buildings or attached facilities that an operator provides for passenger use as part of a public transport service.

(2)	Premises are a form of infrastructure.

Revised Clause (Exemption)

1.21	Premises

(1)	Premises are structures, buildings or attached facilities that a provider supplies for passenger use as part of a public transport service.

(2)	Premises are a form of infrastructure.

(3)	Premises do not include footbridges, open railway platforms or open shelters, which are covered under 1.18 Infrastructure.

Reason 1.21

To better define premises with reference to clause 1.18.

The infrastructure in the rail environment is unique and often constrained in size and function by the location of rail tracks. It is sometimes not possible or relevant for the features on open rail platforms or footbridges to comply with requirements for ‘premises’.

1.22	Provider

(1)	A provider is a person or organisation that is responsible for the supply or maintenance of public transport infrastructure.

(2)	A provider need not be an operator.

1.23	Public transport service

(1)	A public transport service is an enterprise that conveys members of the public by land, water or air.

(2)	A public transport service includes:

(a)	community transport conveyances that are funded or subsidised by charity or public money and that offer services to the public; and

(b)	foreign aircraft and vessels that carry passengers to, from, or in Australia and that offer services to the public.

(3)	A public transport service does not include a service that provides adventure travel (for example, white water rafting, ballooning or amusement park rides), except to the extent that the service operates to move the public from one location to another distant location.

New Clause (Exemption)

1.23X		Sleeping berth

A sleeping berth is a sleeping space which may be in a compartment, or a  seat/bed recliner chair in an open area.

Reasons 1.23X

To provide a clear definition of sleeping berths.

1.24	Small aircraft

A small aircraft is an aircraft with less than 30 seats for the carriage of passengers.

New Part (Exemption)

Part 1X	Mobility aids (new part)

1X.1	Design Criteria

Maximum size

- The maximum size of a mobility aid for use on public transport services, premises and infrastructure is less than 750mm wide by 1300mm long with a maximum head clearance height of 1500mm.

- The front of the mobility aid shall be designed to meet the Figure 1X-X minimum knee and foot clearance beneath tables, counters and worktops.

Figure 1X-X

Manoeuvrability

- The device must be able to turn through 180 degrees within an area of 2070 mm by 1540 mm.

Mass

(4)	The combined mass of the passenger, the mobility aid, and any required assistant must be less than 300 kg.

Reasons 1X.1

The clauses in this new part extract relevant clauses from the Guidelines, particularly the criteria specified in Part 40. They are included in the Standards to provide increased certainty for passengers and operators / providers in relation to the size and characteristics required for safe travel of passenger using mobility aids on public transport.

1X.1 Size:

(1) The maximum size of a mobility aid must be defined for all transport situations, not just as it applies for the allocated space.  Without clear definition it is foreseeable that passengers will want to bring larger mobility aids on to trains and stations and they will neither fit nor be able to negotiate the constrained space available on trains and stations.  Not all devices are suitable or safe in the highly space constrained transport environment.

The maximum width of less than 750mm ensures consistency with the existing clause 2.7 which specifies the minimum access path width of 750mm that can be achieved between the wheel arches of a bus.  This specification is applied to other relevant clauses such as the trafficable width of a ramp in clause 6.3.

(2) The dimensions included in 1X.1(2) for the front of the mobility aid are taken from AS1428.2 – 1992 Clause 24.1.4 and Figure 25.

## (3) Manoeuvrability:  restates the performance specification in existing clause 3.1.

(4) Mass:  This change reflects the amendment to DDA legislation for boarding ramp load tolerance in March 2005.

1X.2	Performance criteria

Braking

- Mobility aids must have effective braking systems and passengers must apply the brakes while conveyances are in motion, and at other times as required for safe travel.

Anchoring

- Mobility aids must comply with relevant safety requirements eg criteria for anchorage points.

Stability

- Mobility aids must be stable under normal conveyance forces of acceleration, deceleration, cornering and pitching.

Propulsion

- Internal or external combustion driven aids are not permitted on transport premises, infrastructure or conveyances.

Batteries

(5)	Electric mobility aids must comply with regulations governing the carriage of batteries on public transport. Batteries need to be adequately secured and contained.  Gel or solid state options should be considered.

Wheels and Gaps

(6)	A mobility aid should be able to :

(a)	cross a horizontal gap of 75 mm for pedestrian level crossings as defined in AS1742.7; and

(b)	mount a vertical rise (bump) up to 20 mm; and

(c)	cross grating gaps up to 13 mm wide and 150 mm long,   and

(d)	cross a horizontal gap up to 65 mm wide when combined with a

vertical gap of up to 20 mm at station boarding points.

Ramps

(7)	Mobility aids should be able to negotiate:

(a)	a 1 in 12 ramp unassisted; and

(b)	a 1 in 8 grade unassisted where the ramp is less than 1520 mm; and

(c) 		a 1 in 4 ramp with assisted access.

Operation and storage

Passengers are responsible for:

(8) (a)	any dis/assembly, operation and manoeuvrability of their disability aids into and out of public transport conveyances, premises and infrastructure

(b)	 storage of their disability aids prior to and after travel on public transport services.

Working Environment

(9) 	The mobility aid must be able to operate safely in internal and external environments under varying ambient conditions.

Reasons 1X.2

1X.2  Performance criteria

(1) - (3):  incorporates existing clauses 1.22 (2) and 1.19 (2) from the Guidelines.

(4) and (5):  reflect safety requirements under OH&S legislation for safe carriage of hazardous materials.

(6)  Wheels and gaps:

(6) (a):  Many stations have level crossings forming part of the access path. The 75mm horizontal gap (when no vertical gap present) reflects the minimum maintenance width of the flange gap that can be achieved on a pedestrian level crossing which is referenced in As1742.7.  At present there are no technical solutions to reduce this gap in all situations.

(6)(b) and (6)(d):  The PTC in WA can meet the standards of 20 mm vertical gap combined with a 65 mm horizontal gap on new straight platforms, and have advised that many people with disabilities can safely cross these gaps.

This is a train - station interface issue, and infrastructure forms part of the solution. In other states, the 12 mm and 40 mm gaps are not practical to achieve or maintain, particularly on rail lines that carry both passenger and freight services and where stations have curved platforms and superelevation of the track may be required.  For a diagonal gap (ie both vertical and horizontal gaps present) exceeding 20mm vertical with 65mm horizontal, the Operator provides direct boarding assistance.

(6)(c) This clause references AS1428.1 (2001) for surfaces in an external environment.

(7) Ramps: This clause restates the performance specification in existing clause 6.4.

(8) Operation and storage:  This clause incorporates the intent of existing clause 1.21 from the Guidelines and reflects OH&S obligations to staff and passengers.

1X.2(9) Environment:  facilities are provided in an external environment, so passengers and their mobility aids must be able to be safely operated in this external environment.

1X.3	Orientation

Transport operators may determine the location and orientation of passengers on premises, infrastructure and conveyances.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses, and small aircraft |  | except airports that do not accept regular public transport services |

Reasons 1X.3

1X.3 Orientation:  This clause incorporates existing clause 1.19(1) from the Guidelines.

Part 2	Access paths

Original Clause

2.1	Unhindered passage

(1)	An access path that allows unhindered passage must be provided along a walkway, ramp or landing.

(2)	An access path must comply with AS1428.2 (1992) Clause 8.1.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

2.1	Unhindered passage

- An access path that allows unhindered passage to a boarding point must be provided.  The Provider may nominate the access path.

- An access path must comply with AS1428.1 (2001), subject to Part 2.X and with the following exceptions and additions:

(a) Walkways, ramps and landings shall have an unobstructed width of not less than 1200 mm.

.

(3) An access path must have a maximum crossfall of 1 in 40 except for 1:33 if the surface is a bituminous seal.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 2.1

This allows the Providers to identify the safest and most practicable access path at stations.  This will enable Providers to find a solution on existing infrastructure, having regards to:

- External geography

- Track configuration

Not all walkways, ramps and landings form part of the Provider’s nominated access path.

Allow landings at 9m on ramps to be consistent with the draft premises standard (Building Code of Australia (BCA)). AS1428.1-2001 Clause 5.3(b) states that landings are to be provided at 9m intervals for 1:14 ramps and 15m for 1:20 ramps.

AS1428.1-2001 Clause 5.2(a) states that landings are to be provided at 15m intervals for 1:20 walkways and 25m for 1:33 walkways.

To include reference to the HREOC Outcome (Part 2.X)

Clause 2.1(3) for cross fall references the draft AS1428.1 clause 11.1(c)

The reasoning behind the 1 in 33 and 1 in 40 cross fall is taken from AS2890.1-1993 Parking Facilities, Clause 2.4.5 Parking Spaces for People with disabilities state "A parking space shall consist of an unobstructed area having a firm, plane surface with a fall not exceeding 1:40 either parallel to, or at 90 degrees to the angle of parking (1:33 if the surface is a bituminous seal and the parking space is out-of-doors)."  This standard has been superseded by AS/NZS 2890.1-2004, which refers  to 2890.6 Off-street Parking for People with Disabilities.  2890.6 is currently in draft form and repeats this clause, as does the current draft version of AS1428.1.

Original Clause

2.2	Continuous accessibility

An access path must comply with AS1428.2 (1992) Clause 7, Continuous accessible path of travel.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

  - Continuous accessibility

An access path shall be provided as follows:

(1) Accessible paths of travel within the boundary of the site shall be provided from transportation stops, accessible parking and accessible passenger loading zones, and accessible public streets or walkways to the accessible building entrance they serve.

(2) Accessible paths of travel shall connect accessible buildings, facilities, and spaces that are on the same site.

(3) Accessible paths of travel shall connect accessible building entrances with all accessible spaces and facilities within a building.

(4) Accessible paths of travel shall connect accessible entrances of each accessible building with those exterior and interior spaces and facilities that serve it.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 2.2

This clause change reduces referencing, provides performance based criteria, and strengthens the need to connect accessible facilities.

Original Clause (No Change)

2.3	Path branching into 2 or more parallel tracks

If an access path branches into 2 or more parallel tracks:

(a)	the ends of each track must be on the main pedestrian traffic routes; and

(b)	the parallel tracks must have equal convenience and be located as close as practicable to the main pedestrian branch.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Original Clause

2.4	Minimum unobstructed width

(1)	The minimum unobstructed width of an access path must be 1200 mm (AS1428.2 (1992) Clause 6.4, Width of path of travel).

(2)	However, the minimum unobstructed width of a moving footway may be 850 mm.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption - see added clause 2.4X)

2.4	Minimum unobstructed width

(1)		The minimum unobstructed width of an access path must be 1200 mm.  For doorways and gateways see relevant clauses.

(2)		However, the minimum unobstructed width of a moving footway may be 850 mm.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services except railway platforms |

Reason 2.4

To clarify the requirements for doorway and gateway widths and reduce referencing.

New clause 2.4X has been added to cover railway station platforms to incorporate the HREOC outcome.

New Clause  (HREOC Outcome)

2.4X	Minimum unobstructed width for railway platforms

Width of an access path at railway stations must comply with the following:

(1)	New platforms

New platforms must comply with Part 2.4.

(2)	Existing stations and platforms

Existing stations and platforms must be brought into compliance with these Standards to the maximum extent possible.

Fittings and fixtures (bins, seats, vending machines) must be relocated to the extent possible to achieve maximum compliance.

Requirements for warning TGSIs to be installed on platform edges may compromise clear access paths in some circumstances as indicated below.

(3)	Site constraints preventing fully compliant paths of travel

Clear access path width may be reduced from 1200mm to 1000mm where site constraints or fixed structures preclude full compliance. This includes new structures constructed within existing site constraints.

(4)	Existing platforms - exceptionally severe site constraints

Where an access path width of 1000mm cannot be achieved adjacent to the 600mm warning TGSI then compliance must be maximized by a range of options, which may include:

(a) Equivalent access by maintaining an access path of 1000mm (minimum) to at least one side of an obstacle.

(b) Providing an access path of 1000mm (min) to a nominated accessible boarding point and equivalent access to essential facilities on the platform.

(5)	Using platform edge warning TGSIs as part of the access path

Where 1000mm clear access path cannot be achieved by the options outlined above, warning TGSIs may form part of the access path (for example where power stanchions or other fixed structures on platforms need to be passed). The overlap of warning TGSIs and access paths must be minimized.

(6)	Information to passengers

Where stations do not have a clear access path width of 1000mm, operators should provide passengers with information (including at stations and through internet sites and other appropriate information methods) detailing access features provided.

(7)  Moving footways

The minimum unobstructed width of a moving footway may be 850 mm.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  | Railway platforms |

Reason 2.4X

The new clause is added to cover railway station platforms and to incorporate the HREOC outcome in relation to access paths and TGSIs.

Clause 2.4 (2) is repeated here for railway platforms.

Original Clause

2.5	Poles and obstacles, etc

(1)	Poles, columns, stanchions, bollards and fixtures must not project into an access path.

(2)	Obstacles that abut an access path must have a luminance contrast with a background of not less than 30%.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

2.5	Poles and obstacles, etc

(1)	Poles, columns, stanchions, bollards and fixtures must not project into an access path, except as provided for in Part 2.4X.

(2)	Obstacles that abut an access path must have a colour contrast with the background.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 2.5

A standard testing procedure is required to measure luminance contrast in all the required in situ situations (eg. day, night, wet & dry). These Standards should exclude the reference to the 30% to be consistent with the draft Building Code of Australia (BCA) wording.  The current proposed testing procedures from the Australian Standards are unclear and do not produce results consistent with physical trials of people with vision impairment.  This was confirmed by Queensland University of Technology Photometric Laboratory testing.

New Clause (Exemption)

2.5AX	Level Crossings

If a public pedestrian level crossing is used as a direct access path to a boarding point it shall comply with AS1742.7 Pedestrian Level Crossings.

|  |  | Infrastructure |
| --- | --- | --- |

Reason 2.5AX

Level crossings have never been considered previously under the Transport Standards.  Draft Australian Standard AS1742.7 Pedestrian Level Crossings is out now for public comment by 23 August 2005.

Original Clause

2.6	Access paths — conveyances

(1)	Subject to subsection (3) and section 2.7, an access path that allows continuous and unhindered passage must be provided with a minimum width of at least 850 mm.

(2)	Subsection (1) applies to doorways and stairs, and between entrances, exits, allocated spaces and other essential facilities for passengers using wheelchairs and other mobility aids.

(3)	If the conveyance exists or is ordered before the commencement of this section, the minimum width may be reduced to 800 mm at any doorway restriction.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

2.6	Access paths — conveyances

(1) Subject to subsection (2) and sections 2.7 and 2.8. a clear access path with a  width of 850 mm, except where design constraints require local restriction to a minimum of 750 mm,, is required extending from the entrance doorway to the allocated spaces,  priority seats and other essential facilities for passengers with mobility aids.

(2) If the conveyance exists or is ordered before 2 April 2004, the minimum width may be reduced to 750 mm at any doorway restriction.

(3) Some sections of a conveyance may not be available to all passengers.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Ferries  Trains  Trams  Light rail |  |  |

Reason 2.6

1. Consistent with the definition in clause 1.9 of the Access Path.

2. Stairs are removed from original 2.6(2) as stairs cannot by definition form part of an access path.

3. Priority seats have been included.

4. Door widths on existing conveyances should be in line with access requirements based on Guidelines Part 40 and using allocated space dimensions and conveyance design constraints.

5. The date 2 April 2004 refers to Amendment 2004 which added the new clause for conveyances under Part 2 Access Paths.

6. Most States provide an integrated ticketing system for bus, rail and ferry.  For consistency clause 2.7 (width between wheel arches on buses) is the common constraint.

7. Clause 2.6(3) incorporates clause 14.1(2) of the guidelines to recognise the constraints of train design and access through double deck rolling stock.

Original Clause (No Change)

2.7	Minimum width between front wheel arches of bus

Between the front wheel arches of a bus, the minimum width of an access path may be reduced to 750 mm between floor level and a height of 300 mm.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses |  |  |

Original Clause

2.8	Extent of path

(1)	An access path must extend from the entrance of a conveyance to the facilities or designated spaces provided for passengers with disabilities.

(2)	Up to 50 mm of an adjacent allocated space may be used as part of the access path.

(3)	If an access path cannot be provided, the operator must provide equivalent access by direct assistance.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

2.8	Extent of path

(1) An access path must extend from the entrance of a conveyance to the allocated spaces, priority seats and other essential facilities for passengers with mobility aids and people with vision impairment using canes and assistance dogs.

(2) Up to 50 mm of an adjacent allocated space may be used as part of the access path.

(3) If an access path cannot be provided, the operator must provide equivalent access by direct assistance.

(4) When allocated spaces adjacent to an access path are occupied, some manoeuvring by people in the allocated spaces may be required to achieve a clear access path.

(5) An access path need not extend through an area with hearing augmentation.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Ferries  Trains  Trams  Light rail |  |  |

Reason 2.8

Clause (1) amended to be consistent with the definition of an access path on a conveyance. The access path is not needed for adequate access to all facilities for all disabilities as it is only people using mobility aids and people using white cane and assistance dogs who will require extra space. The current wording of Clause 2.8 captures all sections of a train with hearing augmentation for people with hearing impairment.

Clause 2.8(4) is applicable for multiple allocated spaces in one car, and cases such as two longitudinal allocated spaces and a narrow access path. Clause 2.8(4) acknowledges that some manoeuvring is required by all passengers in certain circumstances, particularly during peak operational periods, due to the limited width of conveyances and the structure gauge.

Clause 2.8.5 is inserted to be consistent with Clause 2.6(3) to avoid situations in double deck trains where the only acceptable reception area for hearing augmentation is upstairs and this area is not accessible to people with mobility aids.

Clause 2.8(1) is redundant due to 2.6 changes and could be deleted but has been left unchanged.

Original Clause (No change)

2.9	When is an access path not required

(1)	An access path need not extend inside the entrance of a conveyance.

(2)	If there is no access path inside the entrance of a conveyance, the operator must provide on-board wheelchairs or direct assistance to passengers to use on-board facilities or services.

| Conveyances |  |  |
| --- | --- | --- |
|  Accessible taxis  Coaches  Aircraft |  |  |

Part 3	Manoeuvring areas

Original Clause

3.1	Circulation space for wheelchairs to turn in

A manoeuvring area must comply with AS1428.2 (1992) Clause 6.2, Circulation space for a 180 degree wheelchair turn.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

3.1	Circulation space for mobility aids to turn in

A manoeuvring area must allow for a mobility aid to make an 180o turn.  Where possible, the space required for a mobility aid to make an 180o turn shall be no less than 2070mm in the direction of travel and not less than 1540mm wide.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 3.1

Information extracted to reduce referencing to Australian Standards.

This reinforces the criteria in the Guidelines Part 40 and Part 1X Mobility Aids of the Standards.

Original Clause (No Change)

3.2	Access for passengers in wheelchairs, etc

(1)	Passengers in wheelchairs or mobility aids must be able to enter and exit a conveyance and position their aids in the allocated space.

(2)	If this is not practicable, operators must provide equivalent access by direct assistance.

Note   See sections 33.3 to 33.6 in relation to equivalent access and direct assistance.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Ferries  Accessible rail cars  Accessible tram cars  Accessible light rail cars |  |  |

Original Clause (No Change)

3.3	Limited on-board manoeuvring

If the design restrictions of a conveyance limit on-board manoeuvring areas for wheelchairs and similar mobility aids, the operator of the conveyance must ensure equivalent access by direct assistance to passengers.

Note   See sections 33.3 to 33.6 in relation to equivalent access and direct assistance.

| Conveyances |  |  |
| --- | --- | --- |
|  Accessible taxis  Coaches  Aircraft except small aircraft |  |  |

Part 4	Passing areas

Original Clause

4.1	Minimum width

A passing area must have a minimum width of 1800 mm (AS1428.2 (1992) Clause 6.5 (a), Passing space for wheelchairs).

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Delete Clause 4.1 Minimum width (Exemption)

Reason 4.1

Clause 4.1 is redundant; information is covered in Clause 4.2.

Original Clause

4.2	Two-way access paths and aerobridges

(1)	A passing area must be provided at least every 6 metres along any two-way access path that is less than 1800 mm wide (AS1428.2 (1992) Clause 6.5 (b), Passing space for wheelchairs and Figure 3).

(2)	A passing area is not required on an aerobridge.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause  (Exemption)

4.2	Two-way access paths and aerobridges

- Where a path of travel is less than 1800mm wide, passing areas at intervals of not more than 9 metres shall be provided as follows:

- On one side of the path of travel – the passing area shall not be less then 1600mm long and 1800mm wide.

(b)	With space distributed equally on both sides of the path of travel – the passing area shall be not less than 2000mm long and 1800 mm wide.

- A passing area is not required on an aerobridge.

- For sections of railway stations covered by Part 2.4X, no passing areas are required.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 4.2

This takes into account the HREOC outcome regarding the minimum width of an access path on railway platforms.

Consistency with the various associated standards is critical for the effective and efficient design and application of these standards. It is important to remove inconsistencies and reduce the amount of cross referencing needed to obtain performance requirements. Therefore the distance between passing areas should be increased to 9m, to give consistency with the 9m for landing used by AS1428.1 and the current and proposed Building Code of Australia (BCA).

Original Clause

4.3	Passing areas — conveyances

(1)	A ferry designed to carry more than 1 wheelchair must include at least 1 passing area for each accessible deck.

(2)	A train designed to carry more than 1 wheelchair must include at least 1 passing area for each accessible rail car.

(3)	The passing area must enable passengers travelling in mobility aids (conforming with the assumptions indicated in Part 40.1 of the Guidelines) to pass each other.

(4)	The passing area may comprise part of the allocated space or circulation space or both.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trains |  |  |

Revised Clause  (Exemption)

4.3	Passing areas — conveyances

- A ferry designed to carry more than 1 wheel chair must include at least 1 passing area for each accessible deck.

- Each accessible rail car designed to carry more than 1 mobility aid must include at least 1 passing area.

- The passing area must enable passengers travelling in mobility aids (conforming with Part 1X Mobility Aids and the assumptions indicated in Part 40.1 of the Guidelines) to pass each other.

- The passing area may comprise part of the allocated space or circulation space or both.

- Turn taking is an acceptable practice to allow mobility aids to pass.

| Conveyances |  |  |
| --- | --- | --- |
| - Ferries - Accessible rail car |  |  |

Reason 4.3

The existing clause implies that each accessible rail car with even one mobility aid requires a passing area.  Hence the change from train to accessible rail car.

Turn taking should be an acceptable alternative to allow mobility aids to pass, especially in peak hours.

Part 5	Resting points

Original Clause

5.1	When resting points must be provided

(1)	There must be resting points for passengers along an access path if the walking distance between facilities or services exceeds 60 metres (AS1428.2 (1992) Note to Clause 7, Continuous accessible path of travel).

(2)	A resting point must provide seats (AS1428.2 (1992) Clause 27.1(a), Street Furniture).

| Revised Clause (Exemption) | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

5.1	When resting points must be provided

- There must be resting points for passengers along an access path if the walking distance between facilities or services exceeds 60 metres.

- Where major site restraints do not allow for resting points as required, these shall be provided in locations where the infrastructure permits.

- A resting point must provide seating for a minimum of two people and at least one space for a mobility aid. Seats shall be a minimum of 500 mm away from the path of travel.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 5.1

It is common for access paths to be more than 60 metres in length and it is also common for the dimensions of these paths to be constrained by other infrastructure (eg subways, underground stations, arcades, vertical transport requirements). These access paths may just comply with the minimum access path width and it may not be possible to widen them. A minimum width of 2.4 m is required to provide a seating zone and an access path which is not always available (see Figure 31 AS1428.2 and AS1428.2 Clause 27.1(a)). The installation of a rest point with seating may result in a dangerous and unseen constriction point for large numbers of customers using the walkway during peak periods.

The Operator must have the flexibility to assess the higher risks in restricting large numbers of customers and modify existing facilities that protrude into the access path.

Consistency with the various associated standards is critical for the effective and efficient design and application of these standards. It is important to remove inconsistencies and reduce the amount of cross referencing needed to obtain performance requirements.

Part 6	Ramps

Original Clause

6.1	Ramps on access paths

A ramp on an access path must comply with AS1428.2 (1992) Clause 8.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

6.1	Ramps on access paths

A ramp on an access path must comply with AS1428.1 (2001) and;

- In outdoor conditions, ramps on access paths shall be designed so that water does not accumulate on surfaces. (For requirements for ground surfaces, see AS1428.2(1992) Clause 9.)

- Kerb ramps shall comply with the requirements set out in the AS1428.1 (2001).

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 6.1

Consistency with the various associated standards is critical for the effective and efficient design and application of these standards. It is important to remove inconsistencies and reduce the amount of cross referencing needed to obtain performance requirements. The infrastructure installed or modified to accommodate access for customers, particularly those using mobility aids, usually has a service life of up to 40 years.  Therefore it cannot be easily modified to accommodate selective application of various standards or changes in dimensions which have a low impact on accessibility. The latest Australian Standard AS1428.1 (2001) has been adopted by the BCA.

Original Clause

6.2	Boarding ramps

A boarding ramp must comply with AS3856.1 (1998) Clause 2.1.8 (b), (c), (f) and (g).

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Revised Clause (Exemption)

6.2	Boarding ramps

(1) When two or more ramps are deployed, they shall be attached securely to one another.

(2) The surface of the ramp shall have a slip-resistant finish. The surface shall be an acceptable surface when tested in accordance with AS3698.13. Cleats may be provided to assist an attendant using the ramp.

(3) A ramp shall have no protrusions extending for more than 6 mm above the surface except cleats or side edge barriers.

(4) When the ramp is deployed, there shall be no opening or gap in the ramp more than 40 mm.

| Conveyances |  |
| --- | --- |
| except dedicated school buses and small aircraft |  |

Reason 6.2

Reduced referencing and clarification of clause.

Original Clause

6.3	Minimum allowable width

The minimum allowable width of a ramp is 800 mm.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Revised Clause (Exemption)

6.3	Minimum allowable width

Ramps shall have a minimum trafficable width of 750 mm.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Reason 6.3

This aligns with clause 2.7 related to the clear area between bus wheel arches, and Part 40 Guidelines.

Trafficable width is a more appropriate method for measuring ramps as it defines the requirements for the wheels of the mobility aid.

Original Clause (No Change)

6.4	Slope of external boarding ramps

The slope of an external boarding ramp must not exceed:

(a)	1 in 12 for unassisted access (AS3856.1 (1998) Clause 2.1.8(e)); and

(b)	1 in 8 for unassisted access where the ramp length is less than 1520 mm (AS1428.2 (1992) Clause 8.4.2 (a) and AS1428.1 (2001) Figure 8); and

(c)	1 in 4 for assisted access (AS3856.1 (1998) Clause 2.1.8 (e)).

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Original Clause (No Change)

6.5	Slope of ramps connected to pontoon wharves

The slope of a ramp connected to a pontoon wharf must comply with section 6.1 for at least 80% of the high and low tide levels listed in standard tide charts.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Pontoon wharves |

Part 7	Waiting areas

Original Clause (No Change)

7.1	Minimum number of seats to be provided

If a waiting area is provided, a minimum of 2 seats or 5% of the seats must be identified as available for passengers with disabilities if required.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Original Clause (No Change)

7.2	Minimum number of allocated spaces to be provided

If a waiting area is provided, a minimum of 2 allocated spaces or 5% of the area must be available for passengers with disabilities if required.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Part 8	Boarding

Original Clause

8.1	Boarding points and kerbs

(1)	Operators and providers may assume that passengers will board at a point that has a firm and level surface to which a boarding device can be deployed.

(2)	If a kerb is installed, it must be at least 150 mm higher than the road surface.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

8.1	Boarding points and kerbs

(1)	Operators and providers may assume that passengers will board at a point that has a firm and level surface to which a boarding device can be deployed.

(2)	If a kerb is installed, it must be at least 150 mm higher than the road surface.

(3)	The Provider may nominate a single accessible location for boarding of a conveyance.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 8.1

This allows operators and providers to concentrate their supply of information and services to one area of the platform.  This benefits both passengers and operators by improving safety and consistency and allowing for a more efficient service.  A single location may be chosen to maximise the efficient use of the platform and associated infrastructure, and services.

Original Clause

8.2	When boarding devices must be provided

(1)	A manual or power assisted boarding device must be available at any accessible entrance to a conveyance that has:

(a)	a vertical rise or gap exceeding 12 mm (AS3856.1 (1998) Clause 2.1.7 (f)); or

(b)	a horizontal gap exceeding 40 mm (AS3856.1 (1998) Clause 2.1.8 (g)).

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Revised Clause (Exemption)

8.2	When boarding devices must be provided

(1)	A manual or power assisted boarding device must be available at any accessible entrance to a conveyance that has:

(a)	a vertical rise or gap exceeding 20 mm; or

(b)	a horizontal gap exceeding 65 mm

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Reason 8.2

The PTC WA can meet the standards of 20 mm vertical gap and 65 mm horizontal gap on new straight platforms, and have advised that many people with disabilities can safely cross these gaps.

This is a train-station interface issue, and infrastructure forms part of the solution.

The 12 mm and 40 mm gaps are not practical to achieve or maintain.

In other states, platform gaps of more than 20 mm and 65mm will always apply owing to curved platforms, super elevation of the track or freight traffic.  The Operator would provide direct assistance in these cases.

Original Clause (No Change)

8.3	Use of boarding devices

(1)	If a conveyance has a manual or automatic boarding device, it must be available for use at all designated stops.

(2)	An available boarding device must be deployed if a passenger requests its use.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Original Clause (No Change)

8.4	Hail-and-ride services

(1)	If a hail-and-ride service is offered, passengers must be able to hail the service at nominated accessible boarding points where boarding devices can be deployed.

(2)	The boarding points must offer equal access to public transport services.

| Conveyances |  |  |
| --- | --- | --- |
|  Hail-and-ride services except dedicated school buses |  |  |

Original Clause

8.5	Width and surface of boarding devices

A boarding device must:

(a)	be a minimum of 800 mm wide; and

(b)	have a slip-resistant surface.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Delete Clause 8.5 (Exemption)

Reason 8.5

This aligns with clause 2.7 related to the clear area between bus wheel arches, and the Part 40 Guidelines.

Trafficable width is more appropriate method for measuring ramps.

Delete clause 8.5 which repeats information in Ramps Clauses 6.2 and 6.3.

Original Clause

8.6	Maximum load to be supported by boarding device

(1)	A boarding device must be able to support a total passenger and mobility aid weight of up to 300 kg.

(2)	The device must be clearly labelled with the maximum load that it can carry, both on the boarding device and next to the accessible entrance on the outside of the conveyance.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Revised Clause (Exemption)

8.6	Maximum load to be supported by boarding device

- A boarding device must be able to support the combined mass of the passenger, mobility aid and any assistant up to a total of 300 kg.

- The device must be clearly labelled with the maximum load that it can carry.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Reason 8.6

1. Reduce excess signage.

2.The boarding device load description is DDA legislation amended on 17 March 2005.

Original Clause

8.7	Signals requesting use of boarding device

(1)	Any signal for requesting the deployment of a boarding device must be located in an allocated space.

(2)	If possible, a signal is to be placed according to the dimensions given in AS1428.2 (1992) Clause 11.4, Call buttons.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Delete Clause 8.7 (Exemption)

Reason 8.7

This clause is deleted and covered in clause 8.8.

Original Clause

8.8	Notification by passenger of need for boarding device

(1)	It must be possible for a passenger to notify the operator of a conveyance that he or she needs a boarding device to board or alight from a conveyance

(2)	If a request signal device is used, it may be located on the conveyance or at the boarding point according to the dimensions given in AS1428.2 (1992) Clause 11.4, Call buttons.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

8.8	Notification by passenger of need for boarding device

- It must be possible for a passenger to notify the operator of a conveyance that he or she needs a boarding device to board or alight from a conveyance.

      - If a request signal device is used, it may be located on the conveyance or at the nominated accessible boarding point.

      - Any signal for requesting the deployment of a boarding device on a conveyance must be located in or near an allocated space or at an entrance.

      - Where possible, call buttons at entrances shall be located not less than 550 mm and not more then 1200 mm above the finished floor and not less than 500mm from an internal corner.

      - Operators may provide equivalent access by direct assistance.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  | except airports that do not accept regular public transport services |

Reason 8.8

To recognise the constraints of the vehicle structure and layout such as the placement of windows and the surrounding structures affecting the button position.   The passenger is most likely to use the call button when in a position to alight the train i.e. near the entrance.

AS1428.2 Figure 23 details the common reach zone of ambulant and wheelchair users for critical controls as between 700mm and 1200mm above floor level.  Design and wall space constraints on trains and platforms mean that a call button for passengers with mobility aids may need to be placed lower than the 700mm (minimum 550mm) above floor level, eg where flip up seats are installed in the allocated space under windows.  This provides greater design flexibility in constrained spaces.

The “where possible” refers to the presence of weather shields on existing rollingstock adjacent to the side door opening.

It would not be practicable for this to be implemented at all boarding points in the rail environment.  The advantage of having the request signal device at the nominated accessible boarding point is that the majority of conveyances are double ended with the operator nominating the accessible boarding point which may vary depending on the length of train and platforms and the direction of travel.

Details from Australian Standards have been extracted to reduce referencing.

The “where possible” in 8.8 (4) reflects the practical difficulties where mixing of high level and low level platforms on a line or network occurs on the exterior of the train.

Part 9	Allocated space

Original Clause

9.1	Minimum size for allocated space

The minimum allocated space for a single wheelchair or similar mobility aid is 800 mm by 1300 mm (AS1428.2 (1992) Clause 6.1, Clear floor or ground space for a stationary wheelchair).

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

9.1	Minimum size for allocated space

(1)	The minimum allocated space (minimum clear floor or ground space, including knee space under objects as in Figure 1X-X) for a single mobility aid and occupant is 800 mm wide by 1300 mm long and 1500 mm high.

(2)	Transport operators may determine the location and orientation of allocated spaces in conveyances.

(3)	Mobility aid users may not be able to enter or exit an allocated space in a single manoeuvre, given the internal configurations of a conveyance.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Reason 9.1

To reduce referencing to Australian Standards

Clause 9.1(2) and 9.1 (3) are copied from the Guidelines clauses 1.19 (1) and 9.1 (3) respectively.

Clause 9.1 (2) is inserted to allow design flexibility and to achieve an efficient usage of the space available to meet peak loading

Original Clause (No change)

9.2	Minimum number of allocated spaces to be provided

At least one allocated space must be provided in each conveyance (AS1428.2 (1992) Clause 6.1).

| Conveyances |  |  |
| --- | --- | --- |
|  Accessible taxis |  |  |

Original Clause (No Change)

9.3	Minimum head room

(1)	The minimum head room in an allocated space is 1410 mm.

Note   See section 12.5 in relation to minimum doorway opening.

(2)	For a conveyance entering service on or after 1 January 2013, the minimum headroom is 1500 mm.

| Conveyances |  |  |
| --- | --- | --- |
|  Accessible taxis |  |  |

Original Clause (No Change)

9.4	Number of allocated spaces to be provided — buses

(1)	At least 2 allocated spaces must be provided in each bus with more than 32 fixed seats.

(2)	At least one allocated space must be provided in each bus with less than 33 fixed seats.

(3)	An allocated space is additional to the fixed seating capacity.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses |  |  |

Original Clause (No Change)

9.5	Number of allocated spaces to be provided — ferries

(1)	A minimum of 2 allocated spaces must be provided for the first 32 passenger capacity on a ferry.

(2)	An additional 2 allocated spaces must be provided for each additional 100 passenger capacity.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries |  |  |

Original Clause

9.6	Number of allocated spaces to be provided — train cars, etc

(1)	At least 2 allocated spaces must be provided for each rail, tram or light rail car.

(2)	Up to 8 allocated spaces may be consolidated in one car of a set.

(3)	If different classes of travel are offered, allocated spaces must be provided in each class.

| Conveyances |  |  |
| --- | --- | --- |
|  Rail cars  Tram cars  Light rail cars |  |  |

Revised Clause (Exemption)

9.6	Number of allocated spaces to be provided — rail, tram, light rail cars

Number for unbooked services

(1)	(a) The minimum number of allocated spaces required on an unbooked service is equal to the number of rail cars.

(b) An articulated light rail vehicle or tram with a capacity of 100 or more (including standing passengers) must contain a minimum of 2 allocated spaces.

Number for booked services

(2) 	The minimum number of allocated spaces required on booked services is 2 spaces for 1 to 7 passenger seating cars, 3 spaces for 8 to 11 passenger seating cars, and 4 spaces for 12 or more passenger seating cars in a consist.

(3)	Up to 8 allocated spaces may be consolidated in one car of a set.

(4) 	If seating and sleeping accommodation is provided, allocated spaces and accessible sleeping berths must be provided.

| Conveyances |  |  |
| --- | --- | --- |
|  Rail cars  Tram cars  Light rail cars |  |  |

Reason 9.6

Changes to clause 9.6 (1) original clause– There needs to be a separation of suburban and long distance trains where the number of allocated spaces is based on the number of passengers carried so there are equal percentages for allocated space to seating ratios.  Under the existing clause, a carriage with nine passengers has to provide the same number of allocated spaces as a carriage with 150 passengers.  The short car articulated train/tram/light rail allocated space requirement is excessive.

The number of allocated spaces for Australia is much higher than overseas.  The allocated space requirement in the UK is 2 spaces for 2 to 7 vehicles, 3 spaces for 8 to 11 vehicles, and 4 spaces for 12 or more vehicles.

A comparison of the ratio of allocated spaces to seated passengers for booked services, special services, and unbooked services is given below with indicative numbers.

AS1428.1 2001 gives 156,100 as the number of people using manual and electric wheel chairs and motorised scooters  which calculates to 0.8%. of the population (ABS Survey data 1998)

| Reference Clause 9.6 | Comparison of allocated spaces to total numbers of passengers carried (typical number) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Type of Service | Unbooked Suburban single deck | Unbooked Suburban Double deck | Special Services (e.g. tourist luxury train) | Booked Long Distance Services Small Consist | Booked Long Distance Services Medium Consist | Booked Long Distance Services Large Consist |
| No of Cars | 6 | 8 | 17 | 1 | 4 | 8 |
| Total of passengers | 500 | 800 | 100 | 52 | 192 | 384 |
| 1 % of seats = | 5 | 8 | 1 | 1 | 2 | 4 |
| 2 % of seats = | 10 | 16 | 2 | 1 | 4 | 8 |
| Allocated spaces required for Transport Standards | 12 | 16 | 34 | 2 | 8 | 16 |
| Ratio of Allocated spaces to passenger seating (%) under Transport Standards. | 2% | 2% | 34% | 4% | 4% | 4% |
| ARA proposal for allocated spaces | 6 | 8 | 4 | 2 | 2 | 3 |
| ARA proposal percentage (%) Ratio of allocated spaces to total passengers | 1% | 1% | 4% | 4% | 1% | 1% |

The number of passengers booking allocated spaces on booked services is typically around 0.05% of all passengers i.e. the demand for mobility aid positions was 1 passenger in every 2,000. This is less than 1 space needed for every 5 trains, not 2 spaces per car (typically 14 spaces per train) as required under the current standard.  2 spaces, as proposed by the ARA, will significantly exceed the demand while still allowing 2 wheelchair passengers to travel together if required.

Changes to clause 9.6 (3) original clause – Providing allocated spaces in each class is inefficient on a long distance conveyance which has limited space with several classes of travel, and reduces opportunities for consolidation of allocated spaces.

Original Clause

9.7	Consolidation of allocated spaces

If possible, allocated spaces are to be consolidated to accommodate larger mobility aids.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

9.7	Consolidation of allocated spaces

If possible, allocated spaces are to be provided in close proximity to each other.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Ferries  Trains  Trams  Light rail |  |  |

Reason 9.7

It is more important for mobility aid users, their friends and/or carers to travel together than to consolidate space for larger mobility aids.

Mobility aids that comply with Part 1X should be used on Public Transport.

Original Clause- (No change)

9.8	Allocated spaces in aircraft and coaches

An operator does not have to provide allocated space in an aircraft or coach if each passenger uses a fixed seat in the aircraft or coach.

| Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches |  |  |

Original Clause – (No change)

9.9	Use of allocated space for other purposes

Allocated space may be used for other purposes if it is not required for use by a passenger in a wheelchair or similar mobility aid.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Ferries  Accessible taxis  Trains  Trams  Light rail |  |  |

Original Clause

9.10	International symbol of accessibility to be displayed

(1)	The floor area of an allocated space must:

(a)	display the international symbol of accessibility; and

(b)	be outlined in a flush contrasting strip 25 mm wide.

(2)	The colours prescribed in AS1428.1 (2001) Clause 14.2 (c) are not mandatory.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

9.10	International symbol of accessibility to be displayed

(1) An allocated space must:

(a) display the international symbol of accessibility (AS1428.2: 2001 Fig 32); and

(b) if displayed on the floor, be outlined in a flush contrasting strip 25 mm wide.

(2)	The preferred colours of white figure on ultramarine blue (AS2700 B21) are not mandatory.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Trains except booked services  Trams  Light rail |  |  |

Reason 9.10

Recommend that alternatives to floor marking be used for new vehicles to allow for flexibility and ease of changing interior layout, and to avoid permanent highlighting of areas as “special” areas.

Consultation has shown that floor markings are not always preferred by mobility aid users who feel segregated.

In (2) the details of the Australian Standard have been extracted.

On booked trains, allocated spaces are reserved and therefore not required to be identified.

Original Clause (No Change)

9.11	Movement of mobility aid in allocated space

An allocated space must contain movement of a mobility aid towards the front or sides of a conveyance.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Trams  Light rail |  |  |

Part 10	Surfaces

Original Clause

10.1	Compliance with Australian Standard

(1)	Ground and floor surfaces must comply with AS1428.2 (1992) Clause 9, Ground and floor surfaces.

(2)	AS1428.1 Supplement 1 (1993) Clause C12 provides criteria for the selection of floor surfaces.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  | except airports that do not accept regular public transport services |

Revised Clause for Conveyances (Exemption)

10.1	Compliance with Australian Standard

(1)	Ground and floor surfaces must comply with AS1428.2 (1992) Clause 9, Ground and floor surfaces.

(2)	AS1428.1 Supplement 1 (1990) Clause C13 provides criteria for the selection of floor surfaces.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Reason 10.1

Remove Infrastructure and premises from this clause and make a new clause due to the different environments.

Superseded Australian Standards such as AS1428.1-1993 are not available on current electronic standards lists.

The Supplement stated is incorrect. The correct supplement needed is AS1428.1 Supplement 1 (1990) Clause C13.

New Clause (Infrastructure) (Exemption)

10.1X	Compliance with Australian Standard

      - All continuous accessible paths of travel shall have a slip-resistant surface.

      - A continuous accessible path of travel shall have a texture that is traversable by a mobility aid.

      - Grates on an accessible path of travel shall have spaces not more than 13 mm wide and not more than 150 mm long. If gratings have elongated openings, they shall be placed so that the long dimension is transverse to the dominant direction of travel.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 10.1X

The new clause is introduced to reference AS1428.1 (2001)  Clause 12 for external environments.

Part 11	Handrails and grabrails

Original Clause

11.1	Compliance with Australian Standard — premises and infrastructure

      - A handrail must comply with AS1428.2 (1992) Clause 10.1, Handrails.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

11.1	Compliance with Australian Standard — premises and infrastructure

      - A handrail must comply with AS1428.2 (1992) Clause 10.1, Handrails, except Clause 10.1.2(c).

      - Where there is a background wall, handrails shall have a colour contrast with the background wall.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 11.1

Clause 11.1(2) extracts the Guidelines clause 11.2.

The amendments reflect the removal of the 30% Luminance contrast.

Extensive research is needed to define luminance and colour contrast requirements under different light conditions.

Original Clause

11.2	Handrails to be provided on access paths

(1)	Handrails must be placed along an access path wherever passengers are likely to require additional support or passive guidance.

(2)	A handrail must not infringe an area on a roadside boarding point that may be needed to deploy a boarding device.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

11.2	Handrails to be provided on access paths

(1)	Handrails must be placed along an access path wherever passengers are likely to require additional support or passive guidance.

(2)	A handrail must not infringe an area on a roadside boarding point that may be needed to deploy a boarding device.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  | Except buildings at railway premises | except airports that do not accept regular public transport services Except railway infrastructure |

Reason 11.2

Railway premises and infrastructure are excluded from this Standard to clarify the intent to provide assistance to those that need extra support along the accessible path. It is not possible for an Operator/Provider to anticipate the level of additional support for every single customer that uses the facility or may use it sometime in the future. The clause as it stands may require the Operator/Provider to provide handrails along the entire length of the access path from property boundary to boarding point. This could possibly be dangerous and a barrier to mobility aid access on railway infrastructure. Handrails are also provided as way-finding devices and it is noted that mobility support and way-finding features are already prescribed in other parts of these Standards. For example:

- Alternative way finding and cues are available through TGSIs.

- Ramps to 1 in 19 and stairs are currently required to have handrail.

- Seating is provided at rest areas every 60m.

- Seats are provided on platforms

- All pedestrian mazes have fences

Original Clause

11.3	Handrails on steps

(1)	A handrail on steps need not extend beyond the top or bottom of the steps.

(2)	A domed button may be placed 150 mm from any break or end of a handrail instead of an extension at a rail end (AS1428.2 (1992) Figure 5).

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Revised Clause (Exemption)

11.3	Handrails on steps

- A handrail on steps need not extend beyond the top or bottom of the steps.

- A domed button may be placed 150 mm +/- 10 mm from any break or end of a handrail instead of an extension at a rail end.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Reason 11.3

Reduced referencing to standards and uses the tolerance from Figure 5 AS1428.2.

Original Clause

11.4	Handrails above access paths

If installed, a handrail above an access path must comply with AS1428.1 (2001) Clause 6.1 (c), Handrails and Figure 9.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Revised Clause (Exemption)

11.4	Handrails above access paths

(1) If installed, a handrail above an access path must comply with AS1428.1 (2001) Clause 6.1 (c), Handrails and Figure 9, other than where local controls or equipment are in place.

(2) Grabrails may be used where handrails cannot be installed.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses, and small aircraft |  |  |

Reason 11.4

The design constraints of limited internal width and the wall space available on rollingstock do not allow equipment to be recessed above the handrail.  The above amendment reduces unnecessary breaks in the handrail and brings across relevant Guidelines clause 11.1(2) to provide design flexibility in confined spaces.

Original Clause

11.5	Compliance with Australian Standard

A grabrail must comply with AS1428.2 (1992) Clause 10.2, Grabrails.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

11.5	Grabrail compliance requirements

(1)	Grab rails shall be of rigid construction with circular or oval cross section of diameter 30 to 40 mm, designed for a 1100 N load in all directions, slip resistant design where required in wet areas, having hand clearances of 50 to 60 mm, and having an unobstructed hand movement on the top 270 degrees, and minimum corner and edge radii of 5 mm.

(2)	Horizontal grab rails require a clear area 600mm above the rail. Where significant design constraints limit this clearance it should be maximized as much as is technically possible.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses, and small aircraft |  | except airports that do not accept regular public transport services |

Reason 11.5

Reduced Australian Standard referencing.

Title changed from “Compliance with Australian Standard” to “Compliance Requirements”.

The area above the grabrail may have local controls, such as a door open button or an intercom, or the grabrail may be recessed into the wall on conveyances to reduce the risk of hip fracture (if the train was jolted or stopped suddenly).

The limited width of rollingstock does not allow equipment to be recessed above the handrail.  The above amendment reduces unnecessary breaks in the grab rails.

Original Clause

11.6	Grabrail to be provided where fares are to be paid

A grabrail or handrail must be provided at fixed locations where passengers are required to pay fares.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

11.6	Grabrail to be provided where fares are to be paid

A grabrail or handrail may be provided at fixed locations where passengers are required to pay fares by manual exchange of money.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Reason 11.6

Amended to clarify the requirements.

Operators and Providers need to provide a safe location and facilities to purchase tickets.  This allows a designer to provide a benchtop or equivalent support method.  Modern ticketing machines do not require the use of money.

Original Clause

11.7	Grabrails to be provided in allocated spaces

Grabrails that comply with AS1428.2 (1992) Clause 10.2, Grabrails, must be provided in all allocated spaces.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

11.7	Grabrails to be provided in allocated spaces

Grabrails must be provided in all allocated spaces.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Reason 11.7

Grabrails requirements are given in clause 11.5.

Removed unnecessary repeating of Australian Standard reference.

Part 12	Doorways and doors

Original Clause

12.1	Doors on access paths

(1)	Any doors along an access path must not present a barrier to independent passenger travel.

(2)	Direct assistance may be provided through security check points.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

12.1	Doors on access paths

- Any doors along an access path must not present a barrier to independent passenger travel, or equivalent access must be provided by the Operator.

- Direct assistance may be provided through security check points.

- Doors on conveyances may be controlled or opened by the operator.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Reason 12.1

Clause 12.1(3) repeats the Guidelines clause 12.2(2) which covers safe operation of doors by guard / driver.  Passengers are not allowed to open / close external doors independently for safety reasons.

Original Clause

12.2	Compliance with Australian Standard — premises and infrastructure

Doorways and doors must comply with AS1428.2 (1992) Clause 11 (except Clause 11.5.2).

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

12.2	Compliance with Australian Standard — premises and infrastructure

Doorways and doors (except at toilets) must comply with AS1428.2 (1992) Clause 11 (except Clause 11.5.2).

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 12.2

Specification for toilet doors are covered in Part 15.

Original Clause

12.3	Weight activated doors and sensors

(1)	A pressure pad of a weight activated door must be sensitive enough to detect a 15 kg service animal.

(2)	Any other type of sensor on an access path must be able to detect movement between ground level and 500 mm above the access path.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

12.3	Weight activated doors and sensors

(1)	A pressure pad of a weight activated door must be sensitive enough to detect a 15 kg assistance dog.

(2)	Any other type of sensor on an access path must be able to detect movement between ground level and 500 mm above the access path.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 12.3

The term “service animal” is changed to “assistance dog” which has been defined.

Original Clause

12.4	Clear opening of doorways

Doorways must comply with AS1428.2 (1992) Clause 11.5.1, Clear opening of doorways.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

12.4	Clear opening of doorways

The minimum clear opening of a doorway shall be 850 mm except at a toilet door which may be 750 mm.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Reason 12.4

Reduce Australian Standard referencing.  The dimension 750 mm is consistent with the trafficable width of the ramp.

The contrasting trim note in the Australian Standard is not compulsory and is not included.

The width of some existing rollingstock is too narrow with door mechanisms currently available for toilet doors.  The design feasibility and reliability of curved doors are currently being researched.

The internal width of rollingstock does not allow the current standards to be met for both the toilet door width and the access path past the toilet.

This has been confirmed by three rollingstock manufacturers who have difficulties meeting the original dimensions.

Original Clause (No Change)

12.5	Vertical height of doorways

(1)	Doorways must have an unobstructed vertical height of at least 1400 mm.

Note   See section 9.3 in relation to internal headroom.

(2)	For a conveyance entering service on or after 1 January 2013, the minimum unobstructed doorway height must be 1500 mm.

| Conveyances |  |  |
| --- | --- | --- |
|  Accessible taxis |  |  |

Original Clause (No Change)

12.6	Automatic or power-assisted doors

(1)	Doors may be fully automatic.

(2)	Power-assisted doors must not require passengers to grip or twist controls in order to operate opening devices.

(3)	Operators may provide equivalent access to conveyances by opening manual doors for people with disabilities.

Note   See sections 33.3 to 33.5 in relation to equivalent access.

| Conveyances |  |  |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  |  |

Part 13	Lifts

Original Clause

13.1	Compliance with Australian Standard — premises and infrastructure

Lift facilities must comply with AS1735.12 (1999).

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

13.1	Compliance with Australian Standard — premises and infrastructure

Subject to the following clause (1), the lift facilities must comply with AS1735.12 (1999):

- The minimum lift car internal dimensions shall be 1100 mm wide by 1400 mm deep between the inside of the closed car doors to the inside back wall of the car.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 13.1

The minimum internal lift dimension should be maintained due to existing infrastructure constraints.  Clause 13.1 of the lift code AS1735.12 (1999) is inserted to clarify the internal dimensions.

Part 14	Stairs

Original Clause

14.1	Stairs not to be sole means of access

Stairs must not be the sole means of access.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses and small aircraft |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

14.1	Stairs not to be sole means of access

Stairs must not be the sole means of access to facilities designated for passengers with mobility impairment.

.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except dedicated school buses small aircraft, internal stairs on double deck trains |  | except airports that do not accept regular public transport services |

Reason 14.1

Double deck trains are a necessity on the NSW unbooked network due to patronage demand and limited line space for trains.  Stairs are therefore also a necessity. Stairs will unavoidably be the sole means of access to the upper and lower decks, as it is not feasible to have a lift or compliant ramp on a double deck train.

Guidelines 14.1 (2)	The intent of the Disability Standards is that stairs should not be the only means of access to a conveyance nor should they prevent passengers accessing essential on-board services such as food service or toilets. However, clause 14.1(2) of the Guidelines also recognises that unavoidable differences in levels may mean that some sections of a conveyance might not be available to all passengers. On a double deck train, an access path is provided on the single deck level to allocated spaces, priority seats and other facilities for passengers with mobility impairment.

Original Clause (No change)

14.2	Compliance with Australian Standards — premises and infrastructure

Stairs must comply with:

(a)	AS1428.1 (2001) Clause 9.1 (including the notes), Stair Construction; and

(b)	AS1428.1 (2001) Clause 9.2, Stairway Handrails; and

(c)	AS1428.2 (1992) Clause 13.2, Configuration of steps, Clause 13.3, Warning strip at nosing of steps and Figures 8 and 9.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Original Clause

14.3	Compliance with Australian Standards — conveyances

(1)	If stairs are provided on a conveyance mentioned below, they must comply with:

(a)	The notes to AS1428.1 (2001) Clause 9.1, Stair geometry; and

(b)	AS1428.2 (1992) Clause 13.2, Configuration of steps, Clause 13.3, Warning strip at nosing of steps and Figures 8 and 9.

(2)	However, the minimum access path width on stairs in the conveyance must be 850 mm.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

14.3	Stairs on conveyances

- Stairs must not encroach into circulation spaces and must have opaque enclosed risers.

- Colour contrasted warning nosing strips 50 to 75 mm width on top and 25 to 50 mm width on the vertical edge must be provided on the edge of stair treads.
 
Stairs at the entrance

(3)	Stairs at the entrance to a conveyance must comply with AS 1657 (1992) Figure 4.3 measured at the centre line of the stairs.

NOTE: The combination of dimensions of going and rise must lie within the shaded area, i.e. the product of going rise must be not less than 45 000 nor more than 48 000. For example, for a 250 mm going a suitable rise would lie between 180 mm and 192 mm, and for a 170 mm rise a suitable going would be between 265 mm and 282 mm. Maximum slope for a stairway is 45 degrees and minimum slope 26.5 degrees. The diagram also shows slope lines and indicates the range of dimensions applicable to a given slope.

FIGURE 4.3 STAIRWAY RISES AND GOINGS

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trains  Trams  Light rail |  |  |

Reason 14.3

Reduce referencing to Australian Standards, change heading to match.

Due to the restricted space on trains and the need to maximise capacity both seated and standing, compliance with the dimensions specified in the current Clause 14.3 is not reasonable or required for functional accessibility.  This is a premises standard for stair geometry and configuration that has been applied to a conveyance without any analysis of the functional / technical feasibility and impact of this specification on rail operations / train design.

Internal stairs on trains do not form part of an access path for a conveyance.  On double deck trains, an access path is provided on the single deck level to allocated spaces, priority seats and other facilities for passengers with mobility impairment.  Both UK and US access standards specifically exclude internal stairs on trains from compliance due to the design and space constraints and the passenger demand in peak operational periods in a mass transport system.  Compliance requirements only apply to stairs at the entrance to a conveyance.

AS1657 is better suited to the provision of functionally accessible stairs on conveyances.  The provision of handrails on either side of a single short flight of steps provides additional assistance to passengers.

Original Clause (No Change)

14.4	Compliance with Australian Design Rule 58 — conveyances

(1)	Stairs must comply with Australian Design Rule 58 to the extent that that rule sets requirements that conflict with these Standards.

(2)	In any other case, section 14.3 applies.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses except dedicated school buses |  |  |

Part 15	Toilets

Original Clause

15.1	Unisex accessible toilet — premises and infrastructure

If toilets are provided, there must be at least one unisex accessible toilet without airlock that complies with AS1428.1 (2001) Clause 10, Sanitary facilities.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

15.1	Unisex accessible toilet — premises and infrastructure

If toilets are provided, there must be at least one unisex accessible toilet or gender specific accessible toilets without airlock that complies with AS1428.1 (2001) Clause 10, Sanitary facilities (except clause 10.2.10(e))

.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 15.1

The Clause should allow for non-unisex accessible toilets, i.e. an accessible toilet in each of the male and female toilet. The reason:

- Security reason, public perception of safety concerns with unisex toilets (as well as non-safety public concerns with shared toilets e.g. cultural sensitivity)

- At some site locations it is more suitable to have a separate accessible toilet.

The reference to the force required to open a door should be deleted (Clause 10.2.10 (e)), because door closers cannot meet this requirement.

Changes are in line with AS1428.1 (2001) Clause 10.1 Note 3.

Original Clause

15.2	Location of accessible toilets

Accessible toilets must be in the same location as other toilets.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

15.2	Location of accessible toilets

If possible, accessible toilets must be in the same location as other toilets or be in a location of equivalent convenience to the access path.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 15.2

On existing platforms and stations it is not always possible for accessible toilets to be in the same location as other toilets due to site constraints. The current location of toilets may not be a suitable or safe place to locate an accessible toilet providing equivalent convenience.

This revised clause picks up the intent of Clause 2.1.3 of the Guidelines on access paths - access paths may have to be duplicated in some locations.

Original Clause

15.3	Unisex accessible toilet — ferries and accessible rail cars

If toilets are provided, there must be at least one unisex accessible toilet without airlock available to passengers using wheelchairs or mobility aids.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Accessible rail cars |  |  |

Revised Clause (Exemption)

  - Unisex accessible toilet — ferries and trains

If toilets are provided, there must be at least one unisex accessible toilet without airlock available to passengers using compliant mobility aids.

Conveyances

- Ferries

- Trains

Reason 15.3

Changed from accessible rail cars to trains to exclude accessible service cars e.g. dining cars from being required to have an accessible toilet fitted.

Original Clause

15.4	Requirements for accessible toilets — ferries and accessible rail cars

(1)	An accessible toilet must:

(a)	comply with the requirements set out in this section; and

(b)	allow passengers in wheelchairs or mobility aids to enter, position their aids and exit.

(2)	The minimum dimension from the centre line of the pan to the near-side wall must be 450 mm (AS1428.1 (2001) Figure 22).

(3)	The minimum dimension from the centre line of the pan to the far-side wall must be 1150 mm (AS1428.1 (2001) Figure 22).

(4)	The minimum dimension from the back wall to the front edge of the pan must be 800 mm (AS1428.1 (2001) Figure 22).

(5)	The toilet seat must be between 460 mm and 480 mm above the floor (AS1428.1 (2001) Figure 18).

(6)	Hand washing facilities must be provided either inside or outside the toilet (AS1428.1 (2001) Clause 10.2.1 (b), Water closets).

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Accessible rail cars |  |  |

Revised Clause (Exemption)

15.4	Requirements for accessible toilets — ferries and accessible rail cars

(1)	An accessible toilet must allow passengers in mobility aids to enter and exit.

(2)	There shall be sufficient space inside the toilet cubicle for a mobility aid to be positioned so that a disabled person can move from the mobility aid on to the toilet seat from the front or the side of the toilet.

(3)	All unisex toilet amenities must be accessible.

(4)	Hand washing facilities must be provided inside the toilet.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Accessible rail cars |  |  |

Reason 15.4

This change provides a performance based solution which allows flexibility for the rollingstock manufacturers to achieve functional accessibility.

Internal width of rollingstock precludes compliance with this standard for the toilet door width, internal dimensions, and provision of a compliant access path past the toilet.

This has been confirmed by three rollingstock manufacturers who have difficulties meeting the original dimensions.

Note some toilet equipment manufacturers cannot modify their pedestal dimensions to suit Australia, e.g. Chemical toilets due to design and manufacturing constraints.

Dimensions can be taken from the area under the basin from AS1428.1 Figure 23.

Original Clause (No Change)

15.5	Accessible toilet to be provided — aircraft

(1)	At least one toilet must be accessible to passengers using an on-board wheelchair.

(2)	Direct assistance to the toilet door must be provided.

| Conveyances |  |  |
| --- | --- | --- |
|  Wide-body twin-aisle aircraft |  |  |

Original Clause (No Change)

15.6	Stops to be offered if accessible toilet not provided — coaches

If an on-board toilet is not accessible, the operator must offer stops, at reasonable frequency, at premises with accessible toilets.

| Conveyances |  |  |
| --- | --- | --- |
|  Coaches |  |  |

Part 16	Symbols

Original Clause (No Change)

16.1	International symbols for accessibility and deafness

(1)	The international symbols for accessibility and deafness (AS1428.1 (2001) Clause 14.2, Form of the international symbol and Clause 14.3, International symbol for deafness) must be used to identify an access path and which facilities and boarding points are accessible.

(2)	The colours prescribed in AS1428.1 (2001) Clause 14.2 (c) are not mandatory.

(3)	The size of accessibility symbols must comply with AS1428.2 (1992) Table 1.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Original Clause

16.2	Compliance with AS2899.1 (1986)

The illustrations and symbols prescribed in AS2899.1 (1986) must be used if applicable.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

16.2	Compliance with AS2899.1 (1986)

The illustrations and symbols prescribed in AS2899.1 (1986), the Manual of Uniform Traffic Control Devices Part 1 AS1742.1-2003, and ISO 7001:1990 Public Information Symbols must be used if applicable.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reason 16.2

There are only limited signs covered by AS2899.1 (1986).  Universal pictograms should be used where possible.  This places a preference on pictograms over words to help children, non-English speakers, and people with reading difficulties, and reflects operators’ understanding that pictograms are generally preferable and are most likely to be universally understood.

Original Clause

16.3	Accessibility symbols to incorporate directional arrows

The symbol for accessibility must incorporate directional arrows and words or, if possible, pictograms, to show passengers the way to accessible facilities such as toilets.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

16.3	Accessibility symbols to incorporate directional arrows

Directional signage must incorporate accessibility symbols, pictograms (and if needed, words) and directional arrows to show passengers the way to accessible facilities,

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reason 16.3

This clause places a preference on pictograms over words to help people with reading difficulties, and reflects feedback to operators that pictograms are generally preferable and are most likely to be universally understood.  The current wording applies to any sign with symbols or pictograms, but locational signage will not need arrows.

Original Clause (No Change)

16.4	Accessibility symbol to be visible on accessible buses

(1)	The international symbol of accessibility must be clearly visible on the front of accessible buses.

(2)	The symbol must also appear outside accessible doors, accompanied by a numeral indicating the number of allocated spaces provided by the bus.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses |  |  |

Original Clause

16.5	Accessibility symbol to be visible on accessible doors

The international symbol of accessibility must be clearly visible both inside and outside accessible doors on these conveyances.

| Revised Clause (Exemption) Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trains  Trams  Light rail |  |  |

16.5	Accessibility symbol to be visible on accessible doors

The international symbol of accessibility must be clearly visible on the outside of accessible doors.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trams  Light rail |  |  |

Reason 16.5

Trains have been deleted from the Conveyances list as most train operators do not fit exterior signs on rollingstock, owing to the train configuration, as not all accessible doors will be near the nominated accessible boarding point.

Duplication, covered in 16.1.  Remove the need for excessive signage.  Signs on internal doors are not necessary and detract from other safety information inside the car.  Passengers with disabilities will have already used the designated accessible door to enter the conveyance and, in most cases, will exit the same door to alight the conveyance.

Part 17	Signs

Original Clause (No Change)

17.1	Height and illumination

Signs must comply with AS1428.2 (1992) Clause 17.1, Signs, Clause 17.2, Height of letters in signs and Clause 17.3, Illumination of signs and Figure 30.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Original Clause (No Change)

17.2	Location — premises and infrastructure

Signs must be placed according to AS1428.2 (1992) Clause 17.4, Location of signs.

|  | Premises | Infrastructure |
| --- | --- | --- |

Original Clause (No Change)

17.3	Location — conveyances

(1)	If possible, signs are to be placed in accordance with AS1428.2 (1992) Clause 17.4, Location of signs and Figure 30.

(2)	If the design of the conveyance prevents strict compliance, signs must be placed above the head height of passengers, whether they are sitting or standing.

(3)	If used, destination signs must be placed above the windscreen.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Original Clause

17.4	Destination signs to be visible from boarding point

(1)	Destination signs must be visible from, or available at, boarding points.

(2)	They may be displayed on the conveyance or within the premises or infrastructure.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

17.4	Destination signs to be visible from boarding point

(1)	Destination signs must be visible from, or available at, the nominated accessible boarding point.

(2)	They may be displayed on the conveyance or within the premises or infrastructure.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Reason 17.4

Changes are made to improve the clarity of the requirement for the boarding point to be the nominated accessible boarding point.

The original clause 17.4 implies that boarding points are the whole length of the platform.

Original Clause

17.5	Electronic notices

(1)	Presentations of words or numbers on electronic notices must be visible for at least 10 seconds, unless the electronic notice is for the purpose of ticket validation.

(2)	If the electronic notice is for this purpose, the words or numbers on the notice must cease to be visible before the end of 10 seconds if the ticket validation device is used by another person within that time.

|  | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

17.5	Electronic notices

      - Presentations of words or numbers on electronic notices must be visible for the maximum time period operationally possible, unless the electronic notice is for the purpose of ticket validation.

      - If the electronic notice is for this purpose, the words or numbers on the notice must cease to be visible before the end of 10 seconds if the ticket validation device is used by another person within that time.

|  | Premises | Infrastructure |
| --- | --- | --- |

Reason 17.5

The information cannot be displayed for 10 seconds due to the amount of information to be displayed and the need for frequent updating of services within the dwell time of the train at the platform. This operational constraint is most evident during peak time, at major stations and on platforms with services to several possible destinations or on lines with a large number of stations and a variety of stopping patterns.

Original Clause

17.6	Raised lettering or symbols or use of Braille

(1)	If a sign incorporates raised lettering or symbols, they must be at least 0.8 mm above the surface of the sign.

(2)	If an operator or provider supplements a notice with Braille characters, they must be placed to the left of the raised characters.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

17.6	Raised lettering or symbols or use of Braille

- If a sign incorporates raised lettering or symbols, they must be at least 0.6 mm above the surface of the sign.

- If an operator or provider supplements a notice with Braille characters, they must be left justified and placed adjacent to the text or raised characters.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reason 17.6

The minimum raised height of lettering / symbols has been provided by the Royal Blind Society to ensure that vandal proof signage is achievable in the rail environment.  What is achievable in premises is not always so on rail, given the materials used and space constraints experienced in train design that limit the size, shape and locations of signs eg curved panels, door buttons.

Original Clause (No Change)

17.7	Taxi registration numbers

Raised taxi registration numbers must be placed on the exterior of passenger doors forward of the handle.

| Conveyances |  |  |
| --- | --- | --- |
|  Taxis |  |  |

Part 18	Tactile ground surface indicators

Original Clause

18.1	Location

Tactile ground surface indicators must be installed on an access path to indicate stairways, ramps, changes of direction, overhead obstructions below a height of 2000 mm, and hazards within a circulation space or adjacent to a path of travel (AS1428.2 (1992) Clause 18.1, Tactile ground surface indicators).

|  | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

18.1	Location

  - Warning Tactile Ground Surface Indicators (TGSIs), architectural solutions or alternative way-finding aids must be used on an access path to indicate stairways, escalators, ramps and obstructions below a height of 2000 mm.

  - Having established the circumstances that assist safe way-finding, design layouts should not over-use or over-prescribe the installation of tactile ground surface indicators, but rather should make full use of the range of environmental guidance features available so as to minimise inconvenience to other members of the community.

|  | Premises | Infrastructure |
| --- | --- | --- |

Reason 18.1

The original clause has the potential to lead to the over use of Tactile Ground Surface Indicators (TGSIs), which can create more of a hazard.

Wording from AS/NZS 1428.4:2002 Appendix B has been included.

To reduce referencing to Australian Standards.

Original Clause

18.2	Style and dimensions

(1)	The style and dimensions of tactile ground surface indicators must comply with AS1428.4 (1992).

(2)	The stated dimensions may be reduced where a conveyance design does not provide the necessary area.

|  | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

18.2	Style and dimensions

- The style and dimensions of  warning tactile ground surface indicators must comply with the Figure 18AX.

- Where the tolerance between the truncated cones cannot be met, it shall be minimised.

Figure 18AX: Dimensions of a warning tactile

|  | Premises | Infrastructure |
| --- | --- | --- |

Reason 18.2

To reduce referencing.

The clause 18.2 (2) should be deleted as TGSIs on conveyances were removed in Amendment 1 2004 and this clause was missed in the Amendment 1.

The +/- 1 mm tolerance between truncated cones is impossible when installed on a curved platform.

Original Clause (No Change)

18.3	Instalment at accessible bus boarding points

Colour-contrasted tactile indicators must be installed at accessible boarding points at bus stops or in bus zones.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Bus boarding points |

Original Clause

18.4	Instalment at railway stations

Colour contrasted tactile indicators must be installed at the edges of railway platforms as prescribed by AS1428.4 (1992) Clause 6.7.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Railway platforms |

Revised Clause (Exemption)

18.4	Instalment at railway stations

Where alternative hazard barriers, such as a fence, wall or architectural solutions are not installed, colour contrasting warning tactile indicators must be installed as shown in Figure 18BX.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Railway platforms |

Figure 18BX: Examples of warning tactiles at railway platforms.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Railway platforms |

Reason 18.4

Reduced referencing to Australian Standards and places the emphasis on reducing hazards.

Original Clause (No Change)

18.5	Instalment at wharves

Colour contrasted tactile indicators must be installed at wharf edges as prescribed by AS1428.4 (1992) Clause 6.8.

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Passenger wharves |

Part 19	Alarms

Original Clause

19.1	Emergency warning systems

(1)	If installed, emergency warning systems must comply with AS1428.2 (1992) Clause 18.2.1, Emergency warning systems, Clause 18.2.2, Audible alarms, and Clause 18.2.3, Visual alarms.

(2)	Provision must be made for people with vision impairment to locate the exit path in the event of an emergency.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

19.1	Emergency warning systems

(1)	If installed, emergency warning systems must comply with AS1428.2 (1992) Clause 18.2.1, Emergency warning systems, Clause 18.2.2, Audible alarms, and Clause 18.2.3, Visual alarms.

(2)	Provision must be made for people with a disability to locate the exit path in the event of an emergency.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reason 19.1

All people with a disability must be able to locate the emergency exit, not just people with vision impairment.

Part 20	Lighting

Original Clause

20.1	Illumination levels— premises and infrastructure

Any lighting provided must comply with minimum levels of maintenance illumination for various situations shown in the notes to AS1428.2 (1992) Clause 19.1, Illumination levels.

|  | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

20.1	Illumination levels— premises and infrastructure

(1)	Any lighting provided must comply with minimum levels of maintenance illumination for various situations shown in the notes to AS1428.2 (1992) Clause 19.1, Illumination levels, except.

(2) Any lighting provided on railway stations must comply with the following:

|  | Recommended minimum luminance |  |  |  |
| --- | --- | --- | --- | --- |
|  | Code | Eav | E min | EV min |
| Enclosed stations |  |  |  |  |
| Entrance, passageways, walkways | AS1428.2 | 150 |  |  |
| Stairs | AS1428.2 | 150 |  |  |
| Ramps | AS1428.2 | 150 |  |  |
| Toilets and locker rooms | AS1428.2 | 200 |  |  |
| Counter tops | AS1428.2 | 250 |  |  |
| Displays (timetables) | AS1428.2 | 200 |  |  |
| Telephones (ticket machines) | AS1428.2 | 200 |  |  |
| General platform | AS1680.2.1 | 160 |  |  |
| Yellow line (platform edge) |  |  | 150 |  |
| Open stations |  |  |  |  |
| Toilets and locker rooms | AS1428.2 | 200 |  |  |
| Counter tops | AS1428.2 | 250 |  |  |
| Displays (timetables) | AS1428.2 | 200 |  |  |
| Telephones (ticket machines) | AS1428.2 | 200 |  |  |
| Yellow line (platform edge) |  |  | 30 |  |
| General platform | AS1158.3.1 | 42 | 21 | 14 |
| Covered areas | AS1680.2.1 | 160 |  |  |
| Core areas (awning) | AS1680.2.1 | 160 |  |  |
| Ramps and steps (open) | AS1158.3.1 | 42 | 21 | 14 |
| Open footbridge | AS1158.3.1 | 42 | 21 | 14 |
| Primary access paths | AS1158.3.1 | 42 | 21 | 14 |
| Enclosed footbridge | AS 1428.2 | 150 |  |  |
| Subways | AS1158.3.1 | 35 | 17.5 | 17.5 |

Interpretation

The interpretation from using both of these Australian Standards is that the ‘core zone’ of the station where tickets are sold, information is provided, vending machines, telephones and out of weather seating are placed will have 150 lux minimum maintenance illumination. Within this area spot vertical lighting of between 200-300 lux will be provided above ticket counters, timetables, information posters, vending machines and telephones.

(3) Any lighting provided on pedestrian level crossings that provide direct access to a boarding point must comply with AS 1742.7 Pedestrian Level Crossings.

|  | Premises | Infrastructure |
| --- | --- | --- |

Reason 20.1

The clause referenced in the original clause only deals with internal lighting and not outdoor situations or platform lighting. The Australasian Railway Association (ARA) has submitted a suggested lighting specification to Accessible Public Transport National Advisory Committee (APTNAC) which it believes would meet the intent of the standard and be more appropriate to the external environment. The lighting levels specified in the original clause would be difficult to achieve in the external environment without impacting on the requirements of the obtrusive lighting code.

Original Clause (No Change)

20.2	Illumination levels — conveyances

(1)	Any lighting provided must comply with minimum levels of maintenance illumination for various situations shown in the notes to AS1428.2 (1992) Clause 19.1, Illumination levels.

(2)	Lighting should be at least 150 lux at the entrance and at the point where a passenger pays his or her fare.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Original Clause (No Change)

20.3	Dimming

Internal lighting may be dimmed as required to avoid reflection interfering with an operator’s vision.

| Conveyances |  |  |
| --- | --- | --- |

Part 21	Controls

Original Clause

21.1	Compliance with Australian Standard — premises and infrastructure

Controls must comply with AS1428.1 (2001) Clause 11.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

21.1	Compliance with Australian Standard — premises and infrastructure

Controls must comply with AS1428.1 (2001) Clause 11, except Clause 11.1.1 (c).

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 21.1

No door closers can satisfy this force criteria in clause 11.1.1(c).

Original Clause

21.2	Passenger-operated devices for opening and closing doors

Passenger-operated devices for opening and closing manual and power-assisted doors on conveyances must comply with AS1428.2 (1992) Clause 23.2, Operation, and Clause 23.3, Door handles and hardware.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

21.2	Passenger-operated devices for opening and closing doors

Passenger-operated devices for opening and closing manual and power-assisted doors on conveyances, other than emergency door controls, must comply with AS1428.2 (1992) Clause 23.2, Operation, and Clause 23.3, Door handles and hardware.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Reason 21.2

The emergency door controls are designed to be of higher force to avoid inadvertent use and vandalism.

Original Clause

21.3	Location of passenger-operated controls for opening and locking doors

Passenger-operated opening and locking controls for doors on conveyances must be located according to AS1428.1 (2001) Clause 11.1.2, Location.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

21.3	Location of passenger-operated controls for opening and locking doors

Passenger-operated opening and locking controls for doors on conveyances must be located according to AS1428.1 (2001) Clause 11.1.2, Location except that door controls may be placed closer than 500 mm from an internal corner so long as they are accessible to a person using a compliant mobility aid.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Reason 21.3

Measurement of door control to floor height varies with different platform heights.  The original clause tends to limit door controls to being fitted on the doors only, when weather shields are fitted internally adjacent to the doors.  Weather shields provide protection from rain and other weather conditions.

Original Clause (No Change)

21.4	Signal devices for conveyances that stop on request

(1)	Conveyances that stop on request must be equipped with signal devices that comply with AS1428.2 (1992) Clauses 23.2 and 23.3.

(2)	If a signal is operated by a button or pad, one surface dimension must be at least 25 mm.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Trams  Light rail |  |  |

Part 22	Furniture and fitments

Original Clause

22.1	Tables, benches, counters, etc

Tables, benches, counters and similar fixtures must comply with AS1428.2 (1992) Clauses 24.1, 24.1.1, 24.1.2, 24.1.3, 24.1.4 and 24.1.5.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  | except airport terminals |  |

Revised Clause (Exemption)

22.1	Tables, benches, counters, etc

- At least one ticket counter must be suitable for passengers using mobility aids.

- Any tables provided for designated wheelchair locations must be suitable for passengers using mobility aids as per Figure 1X-X.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  Booked services | except airport terminals |  |

Reason 22.1

The referenced clauses in AS1428.2 (1992) do not cover the ticket counter.  These are the only counters or tables used by passengers on premises / infrastructure.  Tables on booked services must be functional for passengers using mobility aids within the space constraints of a train.  Design flexibility must be maintained by a performance specification not a prescribed specification developed for premises.

Original Clause (No Change)

22.2	Information desks, check-in counters, etc — airports

At least 5% of information desks, check-in counters and similar airport fixtures used by passengers must be suitable for use by passengers in wheelchairs or similar mobility aids.

|  | Premises |  |
| --- | --- | --- |
|  |  Airport terminals |  |

Original Clause (No Change)

22.3	Accessible sleeping berths — ferries and trains

(1)	Accessible sleeping berths must comply with AS1428.2 (1992) Clause 24.3 (a), (c) and (d), Beds.

(2)	Passengers in wheelchairs or mobility aids must be able to enter and exit the berth, and position their aids, so that they can get into the bed or bunk.

(3)	If this is not possible, or where recliner chairs are used, operators must provide equivalent access by direct assistance.

Note   See sections 33.3 to 33.6 in relation to equivalent access and direct assistance.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trains |  |  |

Original Clause (No Change)

22.4	Accessible sleeping berths — ferries

If a ferry has sleeping berths, at least one accessible sleeping berth must be provided for every 32 beds or bunks on the ferry.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries |  |  |

Original Clause

22.5	Accessible sleeping berths — trains

(1)	If a train has sleeping berths, a minimum of 2 accessible berths must be provided in each set of up to 4 sleeping cars, or one accessible sleeping berth must be provided for every 32 bunks.

(2)	If different classes of travel are offered, accessible sleeping berths must be provided in each class.

| Conveyances |  |  |
| --- | --- | --- |
|  Trains |  |  |

Revised Clause (Exemption)

22.5	Accessible sleeping berths — trains

(1) If a train has sleeping berths, a minimum of 1 accessible sleeping berth must be provided.

(2) If a train has more than 2 rail cars with sleeping berths, a minimum total of 2 accessible sleeping berths must be provided.

| Conveyances |  |  |
| --- | --- | --- |
|  Trains |  |  |

Reason 22.5

A seat/bed recliner chair type accommodation is common in Europe equivalent to the airline “Sky bed” and is replacing traditional compartments in modern trains.

The construction of PWD berths is unique and independent of class.

Current usage by passengers with mobility aids is less than one percent, and should not exceed the percentage allowances for allocated spaces.

As a comparison, parking spaces provide 1% of parking spaces for people with disabilities in off street parking (AS2890.1)

Waiting areas allocate 5% of seats for all types of disabilities. Using ABS data below, 34% of 5% (156,100/455,100 * 5%) of all disabilities is 1.7% as the mobility aids below.

AS1428.1 2001 gives 156,100 as the number of people using manual and electric wheel chairs, and motorised scooters  which calculates to 0.8%. of the population (ABS Survey data 1998)

| Reference Clause 9.6 | Comparison of accessible sleeping berths to total numbers of sleeping berths |  |  |  |
| --- | --- | --- | --- | --- |
| Type of Service | Sleeper Booked Long Distance Services- Typical Small consist | Sleeper Booked Long Distance Services- Typical Medium consist | Sleeper Booked Long Distance Services- Typical Large Consist | Special Services e.g tourist luxury train |
| No of Sleeping Cars | 2 | 4 | 6 | 11 |
| Sleeping passengers | 30 | 56 | 94 | 100 |
| Transport Standards 22.5 (1) 1 accessible berth per 32 = | 1 | 2 | 3 | 4 |
| Transport Standards 22.5 (1) 2 accessible berths per 4 sleeping cars = | 1 | 2 | 3 | 6 |
| ARA proposal | 1 | 2 | 2 | 2 |
| Transport Standards percentage - Accessible sleeping berths to passenger berths ratio (%) . | 3% | 4% | 3% | 4% to 6% |
| ARA Percentage – Accessible sleeping berths to passenger berths ratio (%) | 3% | 4% | 2% | 2% |

Original Clause (No Change)

22.6	Accessible berths to be connected to access path — ferries and trains

(1)	Accessible berths must be connected by an access path to essential facilities such as toilets, bathrooms and food service areas that are not part of the accessible cabin.

(2)	If this is not practicable, operators must provide equivalent access by direct assistance.

Note   See sections 33.3 to 33.6 in relation to equivalent access and direct assistance.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trains |  |  |

Part 23	Street furniture

Original Clause

23.1	Seats

Seats must comply with AS1428.2 (1992) Clause 27.2, Seating in pedestrian areas.

| Revised Clause (Exemption) | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

  - Seats

The design and installation of seating shall be as Figure 23X and recommended dimensions as follows:

(1) The front of the seat shall have a clear space between any legs at ground level to within 150 mm of the front edge of the seat, and to within 100 mm of the seat height to allow for rearward adjustment of feet when rising.

(2) Where armrests are provided, the top surface of the armrests shall be at a height of 260 ±40 mm above the seat.

(3) The front edge of the seat shall have a minimum radius of 30 mm.

(4) No edge or projection shall have a radius of less than 5 mm unless protected from contact with the user.

(5) The seat shall drain free of water.

FIGURE 23X TYPICAL PARK BENCH SEATING – Recommended dimensions

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 23.1

Reduce referencing and provide design flexibility to achieve functional accessibility in the public domain.

Part 24	Gateways

Original Clause

24.1	Gateways and checkouts

(1)	Gateways and checkouts, such as ticket barriers, must comply with AS1428.2 (1992) Clause 28, Gateways and checkouts.

(2)	However, the width of an accessible gateway or checkout mentioned in AS1428.2 (1992) Clause 28.2 must be at least 850 mm.

| Revised Clause (Exemption) | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

  - Gateways and checkouts

Gateways and checkouts, such as ticket barriers, must comply with the following:

- The international symbol for access (see Clause 16.1) shall be used to designate where access is available.

- Where gateways and checkouts are installed, at least one barrier opening shall be not less than 850 mm wide.

- The ticket, coin feed points shall be at a height of 800 mm to 1200 mm from the finished floor. Any controls needed to operate these machines shall have tactile applications for vision-impaired users.

- A non-contact card reader shall be at a height range of 700 mm to 1200 mm from the finished floor.

- Any barrier shall be not less than 1200 mm past the ticket or coin feed point in the direction of travel (see Figure 24X).

FIGURE 24X BARRIERS WITH TICKET OR COIN FEED POINTS

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reason 24.1

Changes are made to reduce referencing.

Card readers can be higher than 900mm above finished floor level as they only need to be swiped.  AS1428.2 Figure 23 details the common reach zone of ambulant and wheelchair users for critical controls as between 700mm and 1200mm above floor level.  The height of coin operation is brought into line with the requirements for vending machines and phone which allow coin operation at up to 1100-1200mm above floor level.  This provides greater design flexibility to meet the needs of both ambulant and wheelchair passengers.

Part 25	Payment of fares

Original Clause (No change)

25.1	Passengers to pay fares

All passengers must be prepared to pay fares.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Original Clause (No change)

25.2	Fare payment and ticket validation systems

(1)	Fare payment and ticket validation systems must not require actions from passengers with disabilities that exceed the requirements for other passengers.

(2)	For passengers who have difficulties with standard fare payment systems, operators and providers must offer a form of payment that meets equivalent access principles.

Note   See sections 33.3 to 33.5 in relation to equivalent access.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Original Clause

25.3	Vending machines

Vending machines must comply with AS1428.2 (1992) Clause 29.1, Height, Clause 29.2, Controls, and Clause 29.3, Illumination.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

25.3	Ticket vending machines

Ticket vending machines must comply with the following:

- The height of the operative components shall be between 500 mm and 1200 mm above the trafficable surface.

- The required operating force for any control shall not exceed 19.5 N.

- Illumination shall be provided in accordance with Part 20 Lighting.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reason 25.3

To include “Ticket” in the heading to this Clause for clarification.  Transport operators / providers have no control over the design of other types of vending machines captured in the current wording.

AS1428.2 (1992) Clause 29.2 (b) and (c) cannot be achieved on existing ticket machines due to the  large number of destinations in a mass transit system that all need to be on the machine - eg 306 destinations in NSW unbooked system. They would not all fit if they had to include tactile and Braille lettering.

Reduce referencing.

Original Clause

25.4	Circulation space in front of vending machine

The circulation space in front of any vending machine must allow for a 180 degree turn as in AS1428.2 (1992) Clause 6.2, Circulation space for 180 degree wheelchair turn.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Revised Clause (Exemption)

25.4	Circulation space in front of a ticket vending machine

The circulation space in front of any ticket vending machine must allow for a 180 degree turn as in Clause 3.1.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Reasons 25.4

Reduce referencing and clarify wording.

Part 26	Hearing augmentation–listening systems

Original Clause (No change)

26.1	Public address systems— premises and infrastructure

If a public address system is installed, it must comply with AS1428.2 (1992) Clause 21.1, Hearing augmentation.

|  | Premises | Infrastructure |
| --- | --- | --- |

Original Clause

26.2	Public address systems — conveyances

If a public address system is installed:

(a)	people who are deaf or have a hearing impairment must be able to receive a message equivalent to the message received by people without a hearing impairment; and

(b)	it must comply with AS1428.2 (1992) Clause 21.1, Hearing augmentation.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Revised Clause (Exemption)

26.2	Public address systems — conveyances

If a public address system is installed, people who are deaf or have a hearing impairment must be able to receive a message equivalent to the message received by people without a hearing impairment.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Coaches  Ferries  Trains  Trams  Light rail |  |  |

Reason 26.2

Tests on WA and QR conveyances (reports submitted to APTNAC) show current hearing augmentation installations cannot meet Australian Standard requirements.  Visual information can provide a reasonably equivalent summary of audio information but is unable to exactly replicate every audio message. Other forms of direct assistance may be used to supplement visual messaging.

Part 27	Information

Original Clause

27.1	Access to information about transport services

General information about transport services must be accessible to all passengers.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

27.1	Access to information about transport services

(1)	Information necessary to use the transport service must be accessible to all passengers.

(2)	Provision of accessible information assumes that all passengers have a minimum level of literacy, English language skills and an ability to communicate their destination if required.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reasons 27.1

To clarify the requirements, clause 27.1 (2) is added.  Relevant clauses extracted from clauses  27.1 (2) and 1.18(3) of the Guidelines.

Original Clause

27.2	Direct assistance to be provided

If information cannot be supplied in a passenger’s preferred format, equivalent access must be given by direct assistance.

Note   See sections 33.3 to 33.6 in relation to equivalent access and direct assistance.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

27.2	Equivalent access

(1) If essential information cannot be supplied in a passenger’s preferred format, equivalent access must be given.

(2) Certain formats may only be available from certain outlets/locations.

Note   See sections 33.3 to 33.6 in relation to equivalent access and direct assistance.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reason 27.2

By definition, direct assistance is a form of equivalent access.

By having direct assistance written in, it limits the options of providing equivalent access.

Rewording and new clause (2) - clauses 27.2 (2) and (4) of the Guidelines brought across to Standards to reflect intention of Standards and constraints of mass transport system.

Original Clause

27.3	Size and format of printing

(1)	Large print format type size must be at least 18 point sans serif characters.

(2)	Copy must be black on a light background.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

  - Size and format of printing

- If provided, large print format type size must be at least 18 point sans serif characters.

- Copy must optimise colour contrast between text and background.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Reason 27.3

The original clause limits marketing and printing options.

Printed material needs to have good contrast when photocopied.

Route maps are usually shown as coloured lines for easier reading of station sequence.  Additionally the colour is integrated with the colour of electronic displays and paper timetables.

Original Clause

27.4	Access to information about location

All passengers must be given the same level of access to information on their whereabouts during a public transport journey.

| Revised Clause (Exemption) Conveyances |  |  |
| --- | --- | --- |

27.4	Access to information about location

(1) All passengers must be given the same level of access to essential information on their whereabouts during a public transport journey.

(2) If this is not practicable, operators must provide equivalent access.

Note   See sections 33.3 to 33.6 in relation to equivalent access and direct assistance.

| Reason 27.4 Conveyances |  |  |
| --- | --- | --- |

Equivalent access may be provided in ways other than by direct assistance eg visual displays.

Part 28	Booked services

Original Clause

28.1	Notice of requirement for accessible travel

Operators of booked services may request advance notice of a requirement for accessible travel.

| Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains |  |  |

Revised Clause (Exemption)

28.1	Notice of requirement for accessible travel

      - Operators of booked services may request reasonable advance notice of a requirement for accessible travel.

      - Passengers with disabilities requiring assistance, or having special requirements, must notify the operator of their requirements when they book their travel.

      - Passengers requiring assistance to transfer from a mobility aid to a seat, or with feeding, personal hygiene / toileting, medication or the dis/assembly and operation of disability aids must book and travel with a carer.

| Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains |  |  |

Reason 28.1

To cover the requirement of Part 28.2.  Notification of special needs is required by the operator to ensure that where independent access is not possible / available, direct assistance will be feasible and available.

The 28.1(2) clause is extracted from the Guidelines and brought across to the Standards to provide certainty that where independent access cannot be provided, the requirement for direct assistance is known and may be met by the operator and on-board attendants.

Train crew on booked services can assist passengers with disabilities by folding down a manual wheelchair and stowing it with the passenger in the seating / sleeping compartment; providing buffet food to the seat if required; walking someone to the toilet door or retrieving an aid and passing it to the passenger for their use. However, to ensure the occupational health and safety of both passenger and crew, crew are unable to dis/assemble or operate disability aids or provide feeding, medication or toileting assistance.

Original Clause

28.2	Period of notice of requirement for accessible travel

Any advance notice required of a requirement for accessible travel must not exceed the period of notice specified for other passengers.

| Conveyances |  |  |
| --- | --- | --- |
|  Coaches  Ferries  Dial-a-ride services  Trains |  |  |

Delete Clause 28.2 (Exemption)

Reason 28.2

Contradictory with the guidelines 28.1 (2) and previous clauses, and is covered in the amendments to 28.1

The operator on booked services may need additional notice to allow time to remove seating.

Booked services use an on-board manifest to ensure that on-board staff are informed of and prepared for passengers’ requirements.

Original Clause

28.3	Location of carers, assistants and service animals

(1)	On booked services, operators must locate carers, assistants or service animals with the passenger with whom they are travelling.

(2)	In the case of carers or assistants, this would normally be in an adjoining seat.

(3)	If a passenger is travelling with a service animal, the animal must be able to accompany the passenger at all times and to travel without encroaching onto an access path.

| Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains |  |  |

Revised Clause (Exemption)

28.3	Location of carers, assistants and assistance dogs

(1)	On booked services, operators must locate carers, assistants or assistance dogs with the passenger with whom they are travelling.

(2)	In the case of carers or assistants, this would normally be in an adjoining seat.

(3)	If a passenger is travelling with an assistance dog, the animal must be able to accompany the passenger at all times and to travel without encroaching onto an access path.

| Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains |  |  |

Reason 28.3

Terminology used reflects revised definition of service animal to assistance dog.

Original Clause (No change)

28.4	Accessible seats to be available for passengers with disabilities

(1)	Accessible seats must be kept for passengers with disabilities.

(2)	Operators must allocate unbooked accessible seats to other passengers only after all other standard seats are filled.

| Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches  Ferries  Dial-a-ride services  Trains |  |  |

Part 29	Food and drink services

Original Clause

29.1	Equal access to food and drink services

Operators and providers must ensure that any food or drink service that is provided as part of a public transport service is equally available to all passengers.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Revised Clause (Exemption)

29.1	Equal access to food and drink services

Operators and providers must ensure that any food or drink service that is provided as part of a public transport service is equally available to all passengers.

| Conveyances Except unbooked train services | Premises Except rail premises | Infrastructure Except rail infrastructure |  |  |
| --- | --- | --- | --- | --- |

Reason 29.1

Transport operators / providers are not and should not be responsible for the accessibility of all retail outlets selling food and drink within the transport precinct.  It could be argued that they serve transport customers, but the services are not provided by the transport operators / providers, nor are they integral to the provision of essential transport services / facilities. With the current wording operators / providers are responsible for the provision of direct assistance to any food / drink outlet or vending machine that may be on or near the station.

Food / drink services are often provided as an integral part of booked train services.

Original Clause (No change)

29.2	Distance around accessible tables

The distance around accessible tables in food and drink service areas must comply with AS1428.2 (1992) Clause 24.1.7.

|  | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except airports that do not accept regular public transport services |

Original Clause (No change)

29.3	Space for passengers using mobility aids

Operators may provide space for passengers using mobility aids in food and drink service areas or give the same convenience by equivalent access.

Note   See sections 33.3 to 33.5 in relation to equivalent access.

| Conveyances |  |  |
| --- | --- | --- |
|  Ferries  Trains |  |  |

Part 30		Belongings

Original Clause

30.1	Disability aids to be in addition to baggage allowance

(1)	Disability aids (for example, equipment and apparatus including mobility, technical and medical aids) are to be in addition to normal baggage allowances.

(2)	If possible, disability aids are to be treated in the same way as cabin or accompanied baggage.

| Revised Clause (Exemption) Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches  Ferries  Trains |  |  |

30.1	Disability aids to be in addition to baggage allowance

(1)	Disability aids are to be in addition to normal baggage allowances.

(2)	If possible, disability aids are to be treated in the same way as cabin or accompanied baggage.

(3) Disability aids which will be transported in the luggage compartment must comply with the operator’s size limitations for passenger luggage.

(4) Only mobility aids that comply with Part 1X may be carried on public transport conveyances.

(5) Operators are not required to carry mobility aids as priority / additional baggage in the luggage compartment of booked services:

        - where allocated space for mobility aids is provided in the seating / sleeping compartment

when all allocated spaces in the seating / sleeping compartment are already booked on the required service.

(6) 	Operators are not responsible for the assembly / disassembly or operation of disability aids on public transport services, premises or infrastructure.

| Conveyances |  |  |
| --- | --- | --- |
|  Aircraft  Coaches  Ferries  Booked Trains |  |  |

Reason 30.1

To acknowledge operators’ space limitations in transporting luggage, disability aids and mobility aids.  Non-compliant mobility aids such as over sized scooters, should not have to be carried as ‘priority baggage’ when compliant allocated spaces are provided in the seating compartment.  The same principle applies to a person travelling with a mobility aid but wanting to sit in a seat when  the allocated space/s needed to store the mobility aid are already booked by other passengers.   Accommodation of mobility aids is only practical when they are compliant and can be accommodated in the allocated space provided.

Extracts from Clause 1.21 and Part 30 of the Guidelines are brought across to Standards to clarify the maximum size of luggage requirements and provide certainty for operators.

Part 31	Priority

Original Clause

31.1	Priority seating

Operators must designate at least 2 of the seats provided on their unbooked conveyances as priority seating for passengers with disabilities and other groups in need of special assistance (for example, the aging).

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Ferries  Rail cars  Trams  Light rail |  |  |

Revised Clause (Exemption)

31.1	Priority seating

- Operators must designate at least 2 of the seats provided on their unbooked conveyances as priority seating for passengers with disabilities and other groups in need of special assistance (for example, the aging).

- Operators may determine the location and orientation of priority seats.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Ferries  Rail cars  Trams  Light rail |  |  |

Reasons 31.1

Clause (2) added so that train designers and operators can maintain the design flexibility needed to achieve appropriate positioning of all passengers within the space constraints of the train.

Original Clause (No change)

31.2	Information to be provided about vacating priority seating

Operators must inform all relevant passengers (by signage or similar systems) that they should vacate an identified priority seat or allocated space if a passenger with a disability requires it.

| Conveyances |  |  |
| --- | --- | --- |
|  Buses  Ferries  Trains  Trams  Light rail |  |  |

Part 32	Adoption

32.1	Effect and application of these Standards

These Standards apply, on and from the date they come into effect under section 31 of the Disability Discrimination Act 1992, to:

(a)	public transport services provided with:

(i)	newly constructed premises or infrastructure; or

(ii)	conveyances entering service after these Standards come into effect; or

(iii)	premises, infrastructure or conveyances that have undergone substantial refurbishment or alteration; or

(iv)	additional or replacement equipment in premises and infrastructure or on conveyances; and

(b)	new or revised ancillary services that are provided as an adjunct to the public transport operation; and

(c)	new or updated information provided to the public.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

32.2	Manufacture to be completed before target dates

In all cases, manufacture or other work that is required to ensure compliance with these Standards is to be completed before the target dates set out in Schedule 1.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

Part 33	Compliance

33.1	Date for compliance with these Standards — new conveyances, premises and infrastructure

Operators and providers must comply with the specified sections of these Standards for all new premises, infrastructure and conveyances brought into use for public transport service on and from the date these Standards come into effect under section 31 of the Disability Discrimination Act 1992.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

33.2	Date for compliance with these Standards — conveyances, premises and infrastructure in use at target dates

Operators and providers must comply with the specified sections of these Standards for premises, infrastructure and conveyances that are still in use for public transport at the target dates specified in Schedule 1.

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |

33.3	Equivalent access

(1)	Compliance with these Standards may be achieved by:

(a)	applying relevant specifications in these Standards before the target dates; or

(b)	using methods, equipment and facilities that provide alternative means of access to the public transport service concerned (but not using separate or parallel services) with equivalence of amenity, availability, comfort, convenience, dignity, price and safety.

(2)	This may include direct assistance over and above that required simply to overcome discrimination.

33.4	Consultation about proposals for equivalent access

The operator or provider of a public transport service must consult with passengers with disabilities who use the service, or with organisations representing people with disabilities, about any proposal for equivalent access.

33.5	Equivalent access without discrimination

Operators and providers must be able to demonstrate that equivalent access provides public transport without discrimination ‘as far as possible’.

33.6	Direct assistance

(1)	Nothing in these Standards prevents operators or providers from offering assistance directly to passengers.

(2)	If these Standards have not been fully met, direct assistance may be a means of providing equivalent access.

(3)	In addition to compliance with other provisions of these Standards, direct assistance to passengers is required if:

(a)	it is necessary to provide equivalent access to a service; and

(b)	direct access can reasonably be provided without unjustifiable hardship.

33.7	Exceptional cases — unjustifiable hardship

(1)	It is not unlawful to fail to comply with a requirement of these Standards if, and to the extent that, compliance would impose unjustifiable hardship on any person or organisation.

(2)	However, compliance is required to the maximum extent not involving unjustifiable hardship.

(3)	In determining whether compliance with a requirement of these Standards would involve unjustifiable hardship, all relevant circumstances of the particular case are to be taken into account including the following:

(a)	any additional capital, operating or other costs, or loss of revenue, that would be directly incurred by, or reasonably likely to result from, compliance with the relevant requirement of these Standards;

(b)	any reductions in capital, operating or other costs, or increases in revenue, that would be directly achieved by, or reasonably likely to result from, compliance with a relevant requirement of these Standards;

(c)	the extent to which the service concerned operates, or is required to operate, on a commercial or cost-recovery basis;

(d)	the extent to which the service concerned is provided by or on behalf of a public authority for public purposes;

(e)	the financial position of a person or organisation required to comply with these Standards;

(f)	any effect that compliance with the relevant requirement of these Standards is reasonably likely to have on the financial viability of a person or organisation required to comply, or on the provision of the service, or feature of service, concerned;

(g)	any exceptional operational, technical or geographic factors, including at a local or regional level, affecting a person or organisation’s ability to comply with a relevant requirement of these Standards;

(h)	financial, staffing, technical, information and other resources reasonably available to a person or organisation required to comply with these Standards, including any grants, tax concessions, subsidies or other external assistance provided or available;

(i)	benefits reasonably likely to accrue from compliance with relevant requirements of these Standards, including benefits to people with disabilities, to other passengers or to other persons concerned, or detriment likely to result from non-compliance;

(j)	detriment reasonably likely to be suffered by an operator, provider, passenger or other person or organisation concerned, including in relation to equality of amenity, availability, comfort, convenience, dignity, price and safety of services or effectiveness and efficiency of operation if compliance with relevant provisions of these Standards is required;

(k)	if detriment under paragraph (j) involves loss of heritage values — the extent to which relevant heritage value or features of the conveyance, building or other item concerned are essential, and to what extent incidental, to the transport service provided;

(l)	whether compliance with a requirement of these Standards may reasonably be achieved (including by means of equivalent access as provided for in sections 33.3 to 33.5) by less onerous means than those objected to by a person or organisation as imposing unjustifiable hardship;

(m)	any evidence regarding efforts made in good faith by a person or organisation concerned to comply with the relevant requirements of these Standards;

(n)	if a person or organisation concerned has given an action plan to the Commission under section 64 of the Disability Discrimination Act 1992 — the terms of that action plan and any evidence regarding its implementation;

(o)	the nature and results of any processes of consultation, including at local, regional, State, national, international, industry or other level, involving, or on behalf of, an operator concerned, any infrastructure providers as relevant, and people with a disability, regarding means of achieving compliance with a relevant requirement of these Standards and including in relation to the factors listed in this section;

(p)	if a person or organisation seeks a longer period to comply with these Standards, or a requirement of these Standards, than is permitted by the preceding sections on Adoption and Compliance — whether the additional time sought is reasonable, including by reference to the factors set out in paragraphs (a) to (o) above, and what undertakings the person or organisation concerned has made or is prepared to make in this respect.

(4)	If a substantial issue of unjustifiable hardship is raised having regard to the factors listed in paragraphs (3) (a) to (p), the following additional factors are to be considered:

(a)	the extent to which substantially equal access to public transport services (including in relation to equality of independence, amenity, availability, comfort, convenience, dignity, price and safety) is or may be provided otherwise than by compliance with these Standards;

(b)	any measures undertaken, or to be undertaken by, on behalf of, or in association with, a person or organisation concerned to ensure such access.

(5)	For these Standards:

unjustifiable hardship is to be interpreted and applied having due regard to the scope and objects of the Disability Discrimination Act 1992 (in particular the object of removing discrimination as far as possible) and the rights and interests of all relevant parties.

Part 34	Review

34.1	Timetable for review

(1)	The Minister for Transport and Regional Services, in consultation with the Attorney-General, is to:

(a)	review the efficiency and effectiveness of these Standards within 5 years after they take effect; and

(b)	carry out a subsequent review every 5 years after the initial review.

(2)	The review must include:

(a)	whether discrimination has been removed, as far as possible, according to the requirements for compliance set out in Schedule 1; and,

(b)	any necessary amendments to these Standards.

Schedule 1	Target dates for compliance

(section 33.2)

Part 1	Target date — 31 December 2007

1.1	Responsibility

	Operators

	Providers

Requirement

Full compliance with the relevant Standards in relation to:

|  Waiting areas |  Hearing augmentation |
| --- | --- |
|  Symbols |  Information |
|  Signs |  Booked services |
|  Alarms |  Food and drink services |
|  Lighting |  Belongings |
|  Furniture and fittings |  Priority |

Application

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except bus stops |

1.2	Responsibility

	Radio networks

	Co-operatives

Requirement

Response times for accessible vehicles are to be the same as for other taxis.

Application

| Conveyances |  |  |
| --- | --- | --- |
|  Taxis  Dial-a-ride services |  |  |

1.3	Responsibility

	Operators

	Providers

Requirement

Compliance with the relevant Standards by 25% of each type of service in relation to:

|  Access paths |  Doorways and doors |
| --- | --- |
|  Manoeuvring areas |  Lifts |
|  Passing areas |  Stairs |
|  Resting points |  Toilets |
|  Ramps |  Tactile ground surface indicators |
|  Boarding |  Controls |
|  Allocated space |  Street furniture |

Application

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except bus stops |

1.4	Responsibility

	Providers

Requirement

Compliance with the relevant Standards by 25% of bus stops in relation to:

|  Access paths |  Handrails and grabrails |
| --- | --- |
|  Manoeuvring areas |  Stairs |
|  Passing areas |  Symbols |
|  Ramps |  Signs |
|  Waiting areas |  Tactile ground surface indicators |
|  Boarding |  Lighting |
|  Allocated space |  Street furniture |
|  Surfaces |  Information |

Application

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Bus stops |

Part 2	Target date — 31 December 2012

2.1	Responsibility

	Operators

	Providers

Requirement

Full compliance with the relevant Standards in relation to:

|  Surfaces |  Gateways |
| --- | --- |
|  Handrails and grabrails |  Vending machines |

Application

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except bus stops |

2.2	Responsibility

	Operators

Requirement

Full compliance with the relevant Standards in relation to:

	1500 mm minimum head room and vertical door opening

Application

| Conveyances |  |  |
| --- | --- | --- |
|  Accessible taxis |  |  |

2.3	Responsibility

	Operators

	Providers

Requirement

Compliance with the relevant Standards by 55% of each type of service in relation to:

|  Access paths |  Doorways and doors |
| --- | --- |
|  Manoeuvring areas |  Lifts |
|  Passing areas |  Stairs |
|  Resting points |  Toilets |
|  Ramps |  Tactile ground surface indicators |
|  Boarding |  Controls |
|  Allocated space |  Street furniture |

Application

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
|  |  | except bus stops |

2.4	Responsibility

	Providers

Requirement

Compliance with the relevant Standards by 55% of bus stops in relation to:

|  Access paths |  Handrails and grabrails |
| --- | --- |
|  Manoeuvring areas |  Stairs |
|  Passing areas |  Symbols |
|  Ramps |  Signs |
|  Waiting areas |  Tactile ground surface indicators |
|  Boarding |  Lighting |
|  Allocated space |  Street furniture |
|  Surfaces |  Information |

Application

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Bus stops |

Part 3	Target date — 31 December 2017

3.1	Responsibility

	Operators

	Providers

Requirement

Compliance with the relevant Standards by 90% of each type of service in relation to:

|  Access paths |  Doorways and doors |
| --- | --- |
|  Manoeuvring areas |  Lifts |
|  Passing areas |  Stairs |
|  Resting points |  Toilets |
|  Ramps |  Tactile ground surface indicators |
|  Boarding |  Controls |
|  Allocated space |  Street furniture |

Application

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except buses |  | except bus stops |

3.2	Responsibility

	Operators

	Providers

Requirement

Compliance with the relevant Standards by 80% of each type of service in relation to:

|  Access paths |  Doorways and doors |
| --- | --- |
|  Manoeuvring areas |  Lifts |
|  Passing areas |  Stairs |
|  Resting points |  Toilets |
|  Ramps |  Tactile ground surface indicators |
|  Boarding |  Controls |
|  Allocated space |  Street furniture |

Application

| Conveyances |  |  |
| --- | --- | --- |
|  Buses |  |  |

3.3	Responsibility

	Providers

Requirement

Compliance with the relevant Standards by 90% of bus stops in relation to:

|  Access paths |  Handrails and grabrails |
| --- | --- |
|  Manoeuvring areas |  Stairs |
|  Passing areas |  Symbols |
|  Ramps |  Signs |
|  Waiting areas |  Tactile ground surface indicators |
|  Boarding |  Lighting |
|  Allocated space |  Street furniture |
|  Surfaces |  Information |

Application

|  |  | Infrastructure |
| --- | --- | --- |
|  |  |  Bus stops |

Part 4	Target date — 31 December 2022

4.1	Responsibility

	Operators

	Providers

Requirement

All public transport services are to fully comply with the relevant Standards.

Application

| Conveyances | Premises | Infrastructure |
| --- | --- | --- |
| except trains and trams |  |  |

Part 5	Target date — 31 December 2032

5.1	Responsibility

	Operators

	Providers

Requirement

All public transport services are to fully comply with the relevant Standards.

Application

| Conveyances |  |  |
| --- | --- | --- |
|  Trains  Trams |  |  |